3GPP TSG-WG SA2 Meeting #166AH	S2-2500810

# Elbonia, 20th – 24th Jan, 2025		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6008 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification regarding energy collection and exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The approved CR towards TS 23.501 for energy saving topic in SA2#166 meeting established the basis for documenting the related features. This CR is trying to resolve the editor's note and clarification on the following aspects: 1. Add the preamble text in 5.51.2 for referring the 23.502 procedure. And additionally move the “collection time period T” to the next sub clause. 2. Clarify the application-level granularity is supported while it is in fact per-UE-per-application, and the only difference with per-QoS flow is the application ID is provided by the AF, and then the EN can be removed. 3. Clarify the QoS flow level granularity is for the service data flow. In addition, to enable the UPF to estimate the data volume in combination with the current PCC rule (and to not further complicate the SMF logic), it is proposed that the “filter” for the service data flow should be already there for the PCC rule. 4. Update the equation used in Annex. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add the preamble text in 5.51.2 for referring 23.502. And additionally move the “collection time period T” to the next sub clause. 2. Clarify the application-level granularity is supported while it is in fact per-UE-per-application. 3. Clarify the QoS flow level granularity is actually for the service data flow. And add a note for the clarification. 4. Update the equation used in Annex. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description for energy efficiency and energy saving is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2, Annex T |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * The first change * * * *

### 5.51.2	Energy Consumption Information collection, calculation and exposure

#### 5.51.2.1	General

The Energy Information Function (EIF) is defined to collect the UE related Energy Consumption information, calculate the Energy Consumption information at UE, PDU Session, Service data flow granularity and expose the Energy Consumption information to the authorized consumer NF(s) (AF/NEF or 5GC NF) subject to operator's policy.

Procedures for energy consumption information collection and exposure is defined in Clause 4.29 of TS 23.502 [3].

Node level energy consumption is reported PLMN-wide at a configurable period T.

#### 5.51.2.2	Energy Consumption information collection

The Energy Information Function (EIF) collects the UE related Energy Consumption information include Node-level energy consumption information, Node-level data volume from OAM and data volume of the required granularities (i.e. PDU Session and/or QoS flow) from UPF (via SMF).

Node level energy consumption is reported PLMN-wide at a configurable period T.

The serving SMFs are retrieved from the UDM of the UE based on the provided input parameters including the UE ID and (S-NSSAI, DNN).

EIF provides UE ID, DNN/S-NSSAI, IP 5-tuple to retrieve the information from SMF. And the information collected from SMF by EIF, is shown in Table 5.51.2.2-1.

The data volume reporting period from the SMFs is the period T with PLMN-wide configuration.

Table 5.51.2.2-1: Information from SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE IP address | UE IP address. |
| UE ID | SUPI. |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| IP 5-Tuple | IP-5-tuple. |
| List of Data Volume information | The data volume and the associated UPF(s) and gNB(s) serving the UE within the time period. |
| > UL/DL Data Volume of UPF | The UL/DL Data Volume of a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a QoS flow (UE ID, S-NSSAI, DNN, IP 5-Tuple). |
| > (I-)UPF ID(s) | Identifier of any (I-)UPF(s) associated to a reported data volume used by a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a QoS flow (UE ID, S-NSSAI, DNN, IP 5-Tuple). |
| > gNB serving the UE | The gNB which serving the UE. |
| Time period | Time period of the collected information. |

Table 5.51.2.2-2 provides the list of information received from OAM. EIF requests the OAM information with providing the serving gNB ID(s) and (I-)UPF ID(s), which provided by SMFs. The reporting period from the OAM is the period T which is the same as data volume reporting period from SMFs.

Table 5.51.2.2-2: Information from OAM for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| gNB energy consumption | The Energy consumed by a gNB over the configured time period T based on clause 6.7.3.4.2 of TS 28.554 [209]. |
| gNB data volume | The UL/DL data volume handled by a gNB over the configured time period T based on clause 6.7.1.1 of TS 28.554 [209]. |
| UPF energy consumption | The Energy consumed by a UPF over the configure time period T based on clause 6.7.3.1 of TS 28.554 [209]. |
| UPF data volume | Data volume consumed at a UPF. |

When the gNB and/or the (I-)UPF(s) which serving the UE are changed, the serving gNB ID and UPF ID will be updated to the EIF through SMF(s).

NOTE 1:	The Node-level Energy Consumption information received from OAM could be used by EIF for all the UEs serving by the NF Node.

NOTE 2:	In this Release, only the energy related information of user plane communication is supported and that of control plane signalling is not supported.

#### 5.51.2.3	Energy Consumption information calculation

EIF calculates the Energy Consumption information of the required granularities (UE, PDU Session and/or QoS flow), based on input parameters from Tables 5.51.2.2-1 and 5.51.2.2-2 and gets the results. Some example formulas to support the above calculation are described in Annex T.

#### 5.51.2.4	Energy Consumption information exposure

AF/NEF or 5GC NF subscribes the EIF for Energy consumption information of required granularities (UE, PDU session and/or QoS flow).

-	For UE level energy exposure, the consumer NF provides UE ID (SUPI/GPSI).

-	For PDU session level exposure, the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI.

-	For Service data flow level exposure, the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI and Service data flow description(s).

NOTE:	To not cause confliction to current PCC rule, it is assumed that there should be an existing associating PCC rule for the requested Service data flow for exposure.

-	The consumer NF may also subscribe the above information exposure with providing reporting period, reporting frequency.

Editor's note:	Whether and how the renewable energy is introduced is FFS and depending on SA WG5.

# * * * * The second change * * * *

Annex T (informative):
Network Energy consumption information calculation

# T.1	General

This Annex provides examples on how EIF calculate the energy consumption for required granularities.

# T.2	Calculation of Energy Consumption parameters

The energy consumed at a UPF (EUPF)or a gNB (EgNB) is known over a time window T. And the EIF is aware of the data volume (DV) consumed at a UPF by a UE (DVUE,UPF), UE PDU session (DVSession,UPF) and UE QoS flow (DVFlow,UPF) over the time interval T. The EIF is aware of the overall data volume at a gNB (DVgNB), as well as at a UPF  (DVUPF) which serving the UE. Then based on the below example formulas, the required granularities Energy consumption information can be obtained.

To derive the of energy consumed at a gNB over a period of time T by a UE, PDU session, QoS flow, the formulas are:

EUE,gNB=EgNBDVUE,gNBDVgNB

ESession,gNB=EgNBDVSession,gNBDVgNB

EFlow,gNB=EgNBDVFlow,gNBDVgNB

To derive the energy consumed at a UPF over a time period T by a UE, session, QoS flow the formulas are:

EUE,UPF=EUPFDVUE,UPFDVUPF

ESession, UPF=EUPFDVUE,UPFDVUPF

EFlow,UPF=EUPFDVFlow,UPFDVUPF

To derive the energy consumed in the network in the time interval T by a UE (EUE), Session (ESession), QoS flow (EFlow), the formula is:

EUE,gNB=gNBi∈gNBUETEUE, gNBi+ UPFi∈UPFUETEUE, UPFi=ESessionEUE=gNBi∈gNBUETEUE, gNBi+ UPFi∈UPFUETEUE, UPFi=Sessioni∈SessionUETESessioni

where gNBUET are all gNBs used by the UE in an interval T, UPFUET are all UPFs used by the UE in an interval T.

ESessionjSession=gNBi∈gNBSessionjSessionTESessionjSession, gNBi+ UPFi∈UPFSessionjSessionTESessionjSession, UPFi

where gNBSessionT are all gNBs used by the PDU Session in an interval T, UPFSessionT are all UPFs used by the PDU Session in an interval T.

EFlow=gNBi∈gNBFlowTEFlow, gNBi+ UPFi∈UPFFlowTEFlow, UPFi

where gNBFlowT are all gNBs used by the QoS Flow in an interval T, UPFFlowT are all UPFs used by the QoS Flow in an interval T.

# * * * * End of changes * * * *