3GPP TSG-WG SA2 Meeting #166AH	S2-2500814r01

# Elbonia, 20th – 24th Jan, 2025		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5289 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification regarding energy collection and exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In SA2#166 meeting, the basic procedure for energy information collection, calculation and exposure was specified in 23.502, and this is a good starting point for support the energy features in Rel-19. This document tries to further update the procedures with clarifying on the following aspects: 1. Support of per-UE per application granularity; 2. Include the details of the requests in the procedure; 3. Clarify the period T and reporting period; 4. Correct the interaction between SMF and UPF, i.e., only the PSA UPF. 5. Other editorial aspects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the support of per-UE per application granularity; Add the details of the requests in the procedure; Add the period T and reporting period; Modify the interaction between SMF and UPF, i.e., only the PSA UPF. Other editorial aspects. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The energy collection, calculation and exposure procedures are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.29 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.29	Energy consumption information collection and exposure

4.29.0	General

This clause describes how EIF collects the energy consumption information and exposes the energy consumption information to the Consumer NF.

4.29.1	Energy Consumption information exposure

The procedure is used by the AF to subscribe to notifications and to explicitly cancel a previous subscription for Energy Consumption information exposure.

Figure 4.29.1-1: Nnef_EventExposure_Subscribe, Unsubscribe and Notify operations for energy consumption information

1.	The AF subscribes to or cancels subscription for energy consumption information via NEF by invoking the Nnef_EventExposure_Subscribe/ Nnef_EventExposure_Unsubscribe service operation. If the energy consumption information subscription is authorized by the NEF, the NEF proceeds with the steps below.

2.	Based on the request from the AF, the NEF subscribes to or cancels subscription to energy consumption information by invoking the Neif_EventExposure_Subscribe/ Neif_EventExposure_Unsubscribe service operation of EIF.

3.	The EIF acknowledges the subscription or removal of subscription for energy consumption information.

4.	The NEF acknowledges the subscription or removal of subscription for energy consumption information.

5.	The EIF notifies the NEF with the energy consumption information by invoking Neif_EventExposure_Notify service operation.

6.	If the NEF receives the notification from the EIF, the NEF notifies the AF with the energy consumption information by invoking Nnef_EventExposure_Notify service operation.

4.29.2	Energy Consumption information collection

Figure 4.29.2-1 shows the procedure for EIF to collect the Energy consumption information.

Figure 4.29.2-1: Energy Consumption information collection

1.	The Consumer NF (i.e. AF or 5GC NF) subscribes  the energy consumption event  to EIF using Neif_EventExposure_Subscribe service for the required granularities (UE, PDU Session and/or Service data flow) as described in clause 5.51.2.3 of TS 23.501 [2]. The required granularity can also be a certain application of the UE if the Consumer NF is AF. The Consumer NF also provides the reporting period and/or reporting frequency for the subscription.

If the Consumer is AF, the subscription is sent via NEF. If the event subscription is authorized by the NEF, then the NEF sends the event subscription to the EIF via Neif_EventExposure_Subscribe.

2.	The EIF invokes Nudm_UECM_Get service operation to retrieve the SMF ID for the required granularity.

-	The EIF provides UE ID (SUPI/GPSI) if the required granularity is UE, and additionally provides DNN/S-NSSAI if the required granularity is PDU session, QoS flow, or application.

3.	The EIF invokes the Nsmf_EventExposure_Subscribe request (Event ID: User Data Usage Measures) to the SMF to request the data volume of the required granularity, as defined in Clause 5.2.8.3. The reporting period may also be included if received by EIF in step 1.

4.	The SMF informs the UPF with requesting the PSA UPF to report the data volume of the required granularity. The SMF may send the request using either 4a) N4 Session Modification or 4b) Nupf_EventExposure_Subscribe request.

5.	The UPF reports the requested data volume through either 5a) N4 Session Report or 5b) Nupf_EventExposure_Notify, in accordance to step 4.

6. The SMF provides the information for energy consumption calculation (defined 5.51.2.2 of TS23.501[2])  to EIF by invoking Nsmf_EventExposure_Notify service operation for the required granularity.

NOTE 1:	The SMF can derive gNB ID(s)s according to the AMF provided ULI as described in clause 4.3.2.

NOTE 2:	The notification sent by SMF is at PLMN-wide configuration period T within the reporting period.

7. If there is no related Node-level energy consumption information and Node-level data volume, the EIF obtains the Node-level energy consumption information and Node-level data volume from OAM with providing gNB  ID(s) and UPF ID(s).

8.	The EIF performs the energy consumption information calculation as described in clause 5.51.2.3 of TS 23.501 [2] and then reports the energy consumption information of the required granularity to the Consumer NF via Neif_EventExposure_Notify.

# * * * * End of changes * * * *