3GPP TSG-SA2 Meeting #166 Ad Hoc-e	S2-2500821

# Online, 20th Jan 2024 – 24th Jan 2025				                             (revision of S2-25xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1567 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update procedure of Avatar Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on last SA2 #166 meeting discussion, KI#8 avatar communication has some remaining issues to be solved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Update detail procedure of the avatar communication network centric call flow. 2.Remove the ENs that have been solved. 3.Some editorial correction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.11.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * *

### AC.11.3.3	Network centric procedure

Figure AC.11.3.3-1: Procedures of network centric IMS Avatar communication

Figure AC.11.3.3-1 depicts a typical call flow procedure of network centric IMS avatar communication. The main steps in the call flow are as follows:

Editor's note:	Alignment with SA WG4 may be needed.

1.	IMS session and bootstrap data channel have been established. UE1 downloads avatar communication application. The avatar ID list is downloaded as described in clause AC.11.3.1. Then UE1 chooses an avatar ID and runs the avatar communication application.

2.	Application data channel(s) are established between UE1/UE2 and DC AS.

3.	UE1 decides to request network to perform avatar animation based on its status such as power, signal, computing power, internal storage, etc.

4.	The avatar animation negotiation is finished between the DC AS and UE1.

Editor's note:	The avatar animation negotiation procedure and parameters need to be further defined in SA WG4, and cooperation with SA WG4 is needed.

5.	If the negotiation result is successful in step 4, the UE1 initiates new P2A2P application data channels, which are used for avatar data transmission between the UE1/UE2 and the network.

6-7.	During the P2A2P application data channel establishment procedure, the IMS AS notifies the DCSF via Nimsas_SessionEventControl_Notify of the media change request event. The DCSF checks the app id in the ADC establishment request is Avatar communication, then the DCSF get DC AS URL in DCSF service specific data or configured locally in DCSF.

8-9.	The DCSF retrieves the avatar media specification data from DC AS using DC AS URL.

NOTE:	Step 8-9 is out of scope of 3GPP.

10-11.	The DCSF instructs the MF via IMS AS how to establish the data channel and corresponding avatar media processing specification, via Nimsas_MediaControl_MediaInstruction.

12.	The IMS AS sends the MediaControl Instruction response to the DCSF.

13-18.	Steps 11-23 of clause AC.7.2.3 applies.

19.	The application data channel(s) are established between UE1, MF and UE2.

20.	The IMS AS initiates the media re-negotiation between UE1, UE2 and MF, which anchors UE1 and UE2's audio/video to MF.

21.	The MF downloads UE1's avatar representation using the Resource URL in the avatar media specification data sent from DCSF in step 10.

22-23.	The UE1/UE2 may send avatar metadata (e.g. facial expression) through the P2A2P ADC and/or audio/video through RTP channels established in step 17.

24.	MF performs avatar animation based on the avatar metadata received from application data channel and/or audio/video. MF should support different media processing according the avatar type, e.g., animation with no rendering for 2D avatar, animation and rendering for 3D avatar.

25-26.	MF sends the animated video stream to UE2, and sends the animated video stream to UE1 if required.

The above procedures also apply for the transition from audio/video communication between two users to avatar communication. In an ongoing audio/video communication, one of UEs decides to switch the communication to avatar mode, it will interact with the IMS Core and peer UE to establish application data channel for the avatar communication. Then two UEs will exchange avatar media; and the original video media between two UEs may be stopped.

# * * * * End of changes * * * *