3GPP TSG-WG SA2 Meeting #166-AdHoc-e	S2-2500833r03

# E-meeting, January 20 – 24, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0657 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on LMF requesting ML model for LMF-based AI/ML Positioning from the NWDAF containing MTLF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The procedure that LMF requests ML model for LMF-based AI/ML Positioning from the NWDAF containing MTLF as described in clause 6.22.5 needs to be updated to align with clause 5.18. - As specified in TS 23.288 clause 6.2A.1, the LMF may retrieve ML Model information from NWDAF containing MTLF. The ML Model information may include the ML Model file address or ADRF (Set) ID. Therefore the LMF may retrieve ML Model from the ADRF according to the ADRF (Set) ID provided by the NWDAF. - Description of step 6 of Figure 6.22.53-1 needs to be modified as step 6 is not the same as step 5. - Editorial modifications are needed, e.g. changing “AI/ML model” to “ML model”, adding clause 5.18.0 for the hanging paragraphs under clause 5.18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the ML model requesting procedure in clause 6.22.5 to align with clause 5.18, and also modify description of step 6. - Clarify that LMF may retrieve ML Model from ADRF. - Editorial modifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How the LMF requests ML model from NWDAF for LMF-based AI/ML positioning is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

6.22.5	LMF retrieval of a trained AI/ML Model for positioning

The LMF may retrieve a trained ML Model from the NWDAF, and perform UE positioning using the trained ML model after receiving a positioning request.

Figure 6.22.5-1: Retrieval of trained ML model(s) for LMF-based AI/ML positioning from NWDAF

1.	The LMF determines to request trained ML model(s) from the NWDAF for LMF-based AI/ML positioning, e.g. based on local policy.

2.	The LMF discovers an NWDAF that can provide trained ML model(s) for LMF-based AI/ML positioning, by invoking the Nnrf_NFdiscovery_Request service operation including an indication that support of ML Model training for LMF-based AI/ML Positioning is required, ML Model Filter Information, optionally Vendor ID, ML Model Accuracy checking capability and other parameters as described in clause 5.2 of TS 23.288 [37] and clause 5.18.

3.	The LMF receives from the NRF a list of candidates NWDAF instances that match the attributes provided in the Nnrf_NFDiscovery_Request, as specified in clause 5.2.7.3 of TS 23.502 [3].

4.	The LMF selects a NWDAF out of the list of candidates NWDAF instances, and then requests a trained ML Model using Nnwdaf_MLModelInfo_Request or Nnwdaf_MLModelProvision_Subscribe including an indication of requesting for trained ML Model(s) for LMF-based AI/ML positioning and other parameters as described in clause 6.2A of TS 23.288 [37] and clause 5.18.

5.	The NWDAF provides one or more trained ML Models for LMF-based AI/ML positioning to the LMF including the ML Model Identifier, ML Model Information and other parameters as described in clause 6.2A of TS 23.288 [37] and clause 5.18. If the LMF receives ADRF (Set) ID in the ML Model Information, the LMF may retrieve the ML Model from ADRF as described in clause 6.2B.7 of TS 23.288 [37].

6.	If the LMF subscribes to trained ML Model(s) for LMF-based AI/ML positioning from the NWDAF, based on the Nnwdaf_MLModelProvision_Subscribe request received in step 4, the NWDAF provides information of the (re-)trained ML model to the LMF by invoking Nnwdaf_MLModelProvision_Notify service operation as described in clause 6.2A of TS 23.288 [37] and clause 5.18. The LMF updates the trained ML Model(s) accordingly.

The LMF updates the received trained ML Model(s) when the NWDAF provides new ones.

* * * End of Changes * * * *