3GPP TSG-WG SA2 Meeting #166-AdHoc-e	S2-2500834r02

# E-meeting, January 20 – 24, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0658 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on model performance monitoring for LMF-based AIML Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, vivo?, Xiaomi? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Regarding the following EN on data collection by NWDAF from the LMF: Editor's note: Whether the NWDAF needs to be aware of PRU information and indicate to obtain the PRU information from LMF is FFS. Whether the NWDAF needs to indicate Data source type (UE or RAN measurements) and/or data type(s) and/or a target ground truth label precision is also FFS. - Similar to data collection for Analytics ID, where the NWDAF indicates data types to data provider to collect correct input data for model training or deriving analytics, the NWDAF should provide data types for collecting data from the LMF. The NWDAF may also provide data source type (i.e. UE or NG-RAN) to explicitly indicate to the LMF whether measurement data from the UE or NG-RAN are requested. - In addition to data types, it should be possible for the NWDAF to provide input data requirements (e.g. precision, number of data samples/values), including target ground truth label precision, to the LMF for data collection, so as to meet the requested Accuracy level(s) of Interest and/or ML Model Accuracy Threshold (included in ML Model Monitoring Information) requested by the model consumer LMF. - For ML model performance monitoring, the NWDAF shall provide the ML model identifier to the LMF, and the LMF sends the location of PRU(s)/UE(s) estimated using the ML model and measurement data (from the UEs or the NG-RAN) to the NWDAF, so that the NWDAF can monitor the ML model performance by comparing the estimated location with the ground truth data as described in TS 23.288. - The input data includes location measurement data and ground truth data (i.e. locations of PRUs/UEs) for ML model training and performance monitoring. Also there are sample-based measurement data and path-based measurement data for AI/ML positioning as agreed in RAN WGs (see RP-242559). It is proposed to clarify the input data and also modify “data samples” in current descriptions to avoid confusion. - The event subscription usually is not automatically terminated. The NF service consumer may provide parameters for subscription termination (e.g. Expire time) or terminate the subscription by sending an unsubscribe request. It is proposed to remove the description "When the requested number of samples has been provided or the time window for data collection has elapsed, the subscription is automatically terminated", and add the optional input parameter Expire time for Nlmf_DataExposure_Subscribe service operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the input parameters provided by the NWDAF to LMF for data collection. - Clarify that for ML model performance monitoring by NWDAF, the NWDAF shall provide the ML model identifier to the LMF, and the LMF sends the location of PRU(s)/UE(s) estimated using the ML model and the measurement data to the NWDAF. - Modify the descriptions on input data, e.g. clarifying the input data includes location measurement data and ground truth data (i.e. locations of PRUs/UEs) for ML model training and performance monitoring, changing “data samples” to “input data” or “data samples/values”. - Clarify the termination of subscription for data collection from the LMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How the NWDAF collects data from the LMF for ML model training and model performance is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.4, 8.3.4.2, 8.3.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

6.22.4	Input data collection by NWDAF for AI/ML positioning ML model training or ML model performance monitoring

The NWDAF containing MTLF may subscribe to input data (i.e. location measurement data) from LMF for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Figure 6.22.4-1: Procedure of input data collection from LMF

1.	NWDAF containing MTLF determines to train a ML model for LMF-based AI/ML Positioning based on the request from LMF or internal trigger, or the NWDAF containing MTLF determines to perform ML model performance monitoring for LMF-based AI/ML Positioning.

2.	The NWDAF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover an LMF, the service operation includes an AoI and the Nlmf_DataExposure service as discovery parameters. If the NWDAF wants to collect the input data of PRUs, the NWDAF may also include a PRU existence indication for discovering the LMF(s) associated with PRUs (the PRU association procedures are defined in clause 6.17). The NRF selects one or more LMFs based on the AoI, the Nlmf_DataExposure service, and the PRU existence indication (if available), and sends an Nnrf_NFDiscovery_Request Response which includes the profiles of the selected LMFs to the NWDAF.

Editor's note:	The input data used for LMF-based AI/ML Positioning will be decided by RAN WGs.

3.	The NWDAF subscribes to or cancels subscription to input data from LMF by invoking Nlmf_DataExposure_Subscribe / Nlmf_DataExposure_UnSubscribe service operation. The NWDAF includes ML model identifier (for ML model performance monitoring), ,  an AoI and a notification target address to request the input data from LMF. The NWDAF may also include requested number for data samples/values and time window for data collection. The detailed parameters are defined in clause 8.3.4.

Editor's note:	Whether the ML model identifier is needed for FFS,

Editor's note:	Whether the NWDAF needs to be aware of PRU information and indicate to obtain the PRU information from LMF is FFS. Whether the NWDAF needs to indicate Data source type (UE or RAN measurements) and/ordata type(s) and/or a target ground truth label precision is also FFS.

4.	The LMF performs steps 1 to 5 in clause 6.22.2, or steps 1 to 7 in clause 6.22.3 to collect data from PRUs/UEs and/or the NG-RAN. For ML model performance monitoring, the LMF calculates the location of PRU(s)/UE(s) using the collected data and the ML model identified by the ML model identifier.

5.	The LMF sends the collected data (i.e. measurement data from PRUs/UEs and/or the NG-RAN, the corresponding ground truth data (i.e. location of PRUs or UEs)) to the NWDAF by invoking Nlmf_DataExposure_Notify service operation. For ML model performance monitoring, the LMF also sends the location estimation of PRU(s)/UE(s) calculated in step 4. Then the NWDAF trains the ML model or performs ML model performance monitoring based on the input data received from the LMF.

* * * Next Change * * * *

8.3.4.2	Nlmf_DataExposure_Subscribe service operation

Service operation name: Nlmf_DataExposure_Subscribe

Description: Allow the consumer NF to subscribe the input data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Input, Required: Notification Target Address, Notification Correlation ID, AoI.

Input, Optional: ML model identifier,  Time window to collect data, Subscription Correlation ID (in the case of modification of the subscription), Expiry time.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None.

8.3.4.3	Nlmf_DataExposure_Notify service operation

Service operation name: Nlmf_DataExposure_Notify

Description: Allow the consumer NF to get notification about the input data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Input, Required: Notification Correlation ID.

Input, Optional: location measurement  data from PRUs/UEs or the NG-RAN, Ground truth data (location of PRUs or UEs), ML model identifier, location information of PRU(s)/UE(s) calculated using the ML model and measurement data, Subscription Correlation ID (this parameter shall be present if the notification is for informing the assignment of a new Subscription Correlation ID by the LMF).

Editor's note:	The input data used for LMF-based AI/ML Positioning will be decided by RAN WGs.

Output, Required: Operation execution result indication.

Output, Optional: None.

* * * End of Changes * * * *