SA WG2 Meeting #S2-166AH	S2-2500866r01

20 - 24 Jan, 2025, Emeeting

Title:	LS Response on handling of Nsmf events targeting anyUe for I-SMF

Response to:	C3-246487

Release:	Rel-18

Work Item:	UPEAS

Source:	SA2

To:	CT3

Cc:	None

Contact Person:

Name:	Zhu Jinguo

E-mail Address:	zhu.jinguo@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks for CT3 for the LS (C3-246487) on handling of Nsmf events targeting anyUe for I-SMF.

The CT3 LS asked one question regarding the following agreed SA2 23.502 CR#5108 specified (in 23.502 clause 4.15.4.5.3) that:

"

An SMF receiving Nsmf_EventExposure Subscription request to subscribe to UPF data for "Any UE", does not consider PDU sessions for which it is acting as I-SMF when it selects the PDU session(s) and the UPFs it has to send the request to

"

CT3 understands that this avoids a duplication of subscriptions and wants to ask the following question:

Question1: Is this issue specific to UPF events or can it apply to any event type that is supported by the I-SMF?

SA2 Answer: SA2 agreed the following in the attached CR.

An SMF receiving Nsmf_EventExposure Subscription request to subscribe SMF exposure event for "Any UE", does not consider PDU sessions for which it is acting as I-SMF when it selects the PDU session(s).

2. Actions:

To CT3

ACTION:

SA2 kindly asks CT4 to take the information above into account.

3. Date of Next SA2 Meetings:

SA2#167	Feb 17-21, 2025 					Athens, GR

SA2#168	Apr 7-11, 2025 						Goteborg, SE