3GPP SA WG2 Meeting #166AH	S2-2500870r02

20-24 Jan 2025, E-meeting	(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6020 | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolvement of EN in AF Requested modification of the Set of Network Slice(s) for a UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Nokia, Ericsson? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_SliceSel |  |  |  |  |  | Date: |  |  | 2025-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are several EN remaining in clause 5.15.5.2.2a Editor's note: The value of Network Slice Replacement Type is FFS. Proposal: Remove this EN directly. The reason is that the AMF needs to differentiate the Rel-18 network slice replacement feature and Rel-19 feature. The details of value of the Network Slice Replacement Type will be defined in stage 3. Editor's note: Which PCRF to be used to request Notification on outcome of Network Slice Replacement request is FFS. Proposal: Propose to remove the EN directly Editor's note: It is FFS whether the Replaced S-NSSAI should be removed from the Configured NSSAI. Proposal: Remove this EN directly. When the Replaced S-NSSAI is removed from the Configured NSSAI, if the UE is deregistered and during the deregistration period, the AF stops the network slice replacement, there is possibility that the AMF cannot add the Replaced S-NSSAI back to Configured NSSAI. Therefore the Replaced S-NSSAI should not be removed from the Configured NSSAI. It needs to be clarified that the AMF can update the Allowed NSSAI via the UCU procedure to include the Replaced S-NSSAI. This will also remove the Replaced S-NSSAI from the Rejected NSSAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolve the remainging ENs There are other issues - Align the terminology: use the “terminate’ instead of “stop” Add clarification that when the AF terminiates the network slice replacement, the AMF updates the Allowed NSSAI to include the Replaced S-NSSAI and remove the Replaced S-NSSAI from rejected S-NSSAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.5.2.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.15.5.2.2a	AF Requested modification of the Set of Network Slice(s) for a UE

In non-roaming cases, an authorized AF or an AF subject to authorization via the NEF, may request to replace a certain S-NSSAI (Replaced S-NSSAI) for a UE with another S-NSSAI (Alternative S-NSSAI) which is part of the UE subscription. The request in either case is sent to the PCF for the UE handling the impacted subscriber. Similarly, an authorized AF or an AF subject to authorization via the NEF, may request to terminate the replacement then request to use the Replaced S-NSSAI instead of the Alternative S-NSSAI for a UE again. The AF is notified on the outcome of the Network Slice Replacement.

Only a single AF can request the modification of the Set of Network Slice(s) for one replaced S-NSSAI.

NOTE 1:	How the AF obtains the S-NSSAI to be replaced and the Alternative S-NSSAI is out of scope of this specification.

The PCF for the UE based on operator policies considers the AF request. Consequently the PCF for the UE triggers the Network Slice Replacement at the AMF as defined in TS 23.503 [45].The PCF sends Slice replacement management in the access and mobility management policies to the AMF per implicitly subscription, as described in TS 23.503 [45]. The Slice replacement management policy includes the Replaced S-NSSAI and the corresponding Alternative S-NSSAI as provided in the AF request, PCRT to request Notification on outcome of Network Slice Replacement request, together with a Network Slice Replacement Type. The Network Slice Replacement Type indicates that the Network Slice Replacement is requested by an AF. The AMF handles the procedure as following:

-	The AMF shall not handle the Network Slice Replacement procedure as described in clause 5.15.19.

-	If the Alternative S-NSSAI is not within the UE subscription, or is not supported in the UE Registration Area, the AMF shall notify the PCF that the Network Slice Replacement is not triggered with a cause value. The PCF further notify the AF.

NOTE 2:	The AMF may update the UE Registration Area to ensure that the Alternative S-NSSAI is supported in the UE Registration Area.

-	If the Alternative S-NSSAI is within the UE subscription and is supported in the UE Registration Area, the AMF includes the Alternative S-NSSAI in the Allowed NSSAI (if not present). If the Alternative S-NSSAI is subject to Network Slice-Specific Authentication and Authorization, the AMF initiates the NSSAA procedure for the S-NSSAI.

-	The AMF removes the Replaced S-NSSAI from the Allowed NSSAI and add the Replaced S-NSSAI into the Rejected S-NSSAI by a UE Configuration Update procedure.

-	The AMF releases the PDU sessions that were using the Replaced S-NSSAI.

-	The AMF stores in the UE context the access and mobility management policy information as received from PCF and transfers to new AMF during inter AMF mobility.

-	After performing the above steps, the AMF reports to the PCF the outcome of the network slice replacement (e.g. the S-NSSAI has been replaced by an Alternative S-NSSAI) as defined in TS 23.503 [45].

NOTE 3:	Operator policies in the PCF ensures that the UE is configured with a URSP rule that contains the Alternative S-NSSAI to establish a new PDU Session.

The trusted AF or a NEF may send request to PCF for the UE to terminate the Network Slice Replacement. In this case the PCF for the UE sends access and mobility management policies to the AMF, as described in TS 23.503 [45]. The AMF updates the Allowed NSSAI to include the Replaced S-NSSAI and remove the Replaced S-NSSAI from the Rejected S-NSSAI by a UE Configuration Update procedure. The UE may request to register the Replaced S-NSSAIs again next time the UE sends a Requested NSSAI.

End of CHANGES