3GPP SA WG2 Meeting #166AH	S2-2500882r01

20-24 Jan 2025, E-meeting	(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0295 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve EN for policy provisioing for Local offloading management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following EN remains in TS23.548 Editor's note: whether the AF traffic influence information is transferred to I-SMF is FFS. The AF influence on traffic routing information is provided per DNAI by the AF to influence the traffic routing over N6 towards local DN. The SMF uses this information to generate the N4 information and provide it to the L-UPF via I-SMF. For Local Offloading Management, the SMF can still provide the N4 information (e.g. UPF event report, etc.) via I-SMF towards the L-PSA(s). Therefore there is no need to provide the AF influence on traffic routing information to I-SMF. This EN can be removed. Additional clarification is needed to make Offload Identifier as optional feature |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the EN. Clarify that the Offload Identifier is optional, even if the I-SMF is locally configured with the Offload Identifer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not completed and hard to be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | . |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.10.3.2	Local offloading management policy handling

The Local Offloading Management Policy is generated by PCF and retrieved by SMF for Local Offloading Management as described in clause 6.10.2. The Local Offloading Management Policy may also be provided with a Offload Identifier. The Offload Identifier is assigned by PCF, and is unique within the PLMN. The SMF and I-SMF for Local Offloading Management may also be preconfigured with Local Offloading Management Policy and each Local Offloading Management Policy may be tagged with related Offload Identifier(s). The PCF may only provide the Offload Identifier to the SMF.

The detailed information of Local Offloading Management Policy is described in clause 6.4 of TS 23.503 [4].The SMF forwards the received Local Offloading Management Policy or Offload Identifier to the I-SMF transparently.

End of CHANGES