3GPP SA WG2 Meeting #166AH	S2-2500882r02

20-24 Jan 2025, E-meeting	(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0295 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve EN for policy provisioing for Local offloading management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clarification is needed to make Offload Identifier as optional feature |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the Offload Identifier is optional, even if the I-SMF is locally configured with the Offload Identifer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not completed and hard to be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | . |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.10.3.2	Local offloading management policy handling

The Local Offloading Management Policy is generated by PCF and retrieved by SMF for Local Offloading Management as described in clause 6.10.2. The Local Offloading Management Policy may also be provided with a Offload Identifier. The Offload Identifier is assigned by PCF, and is unique within the PLMN. The SMF and I-SMF for Local Offloading Management may also be preconfigured with Local Offloading Management Policy. Each Local Offloading Management Policy may also be tagged with related Offload Identifier(s). In this case, the PCF may only provide the Offload Identifier to the SMF.

The detailed information of Local Offloading Management Policy is described in clause 6.4 of TS 23.503 [4].The SMF forwards the received Local Offloading Management Policy or Offload Identifier to the I-SMF transparently.

End of CHANGES