3GPP TSG-WG 3GPPSA2#166-Ad Hoc-e	S2-2500931

Online, 20th JAN – 24th JAN, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR to TS 23.xyz: Network Functions Services

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: add the services for AMF, NEF, AIOTF, ADM (new data storage NF) in respect to Ambient IoT.

1. Discussion

Follow the conclusion in clause 8.3 of TR 23700-13, this proposal introduces the support of Ambient IoT services for AMF, NEF, AIOTF, ADM (new data storage NF).

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xyz.

# * * * *Option 1 First change (all new texts) * * * *

7.1	General

In the context of Ambient IoT specification, an NF service is offering a capability to authorised upstream and downstream NF consumers, thus to enable the network to offer the end-to-end Ambient IoT services from system perspective.

The following clauses provide for each involved NF the NF services it exposes through its service-based interfaces.

7.2	AIOTF service

The following table shows the Naiotf_AIoT Service and Service Operations.

Table 7.2-1: NF services provided by AIOTF

| NF service name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naiotf_AIoT | Inventory | Request/Response | NEF |
|  | Read | Request/Response | NEF |
|  | Write | Request/Response | NEF |
|  | PermanentDisable | Request/Response | NEF |
|  | Notify | Subscribe/Notify | NEF |

7.2.1	Naiotf_AIoT_Inventory service operation

Service operation name: Naiotf_AIoT_Inventory

Description: the NF consumer requests to perform an inventory device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information or target Reader ID list.

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Correlation ID.

7.2.2	Naiotf_AIoT_Read service operation

Service operation name: Naiotf_AIoT_Read

Description: the NF consumer requests to perform a read device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information or target Reader ID list.

3) Read command specific parameters

-  AIoT data from AF to the target AIoT device(s)

-  Approximate D2R message size.

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Correlation ID.

7.2.3	Naiotf_AIoT_Write service operation

Service operation name: Naiotf_AIoT_Write

Description: the NF consumer requests to perform a write device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information or target Reader ID list.

3) Write command specific parameters:

-  AIoT data from AF to the target AIoT device(s)

-  Approximate D2R message size.

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Correlation ID.

7.2.4	Naiotf_AIoT_PermanentDisable service operation

Service operation name: Naiotf_AIoT_PermanentDisable

Description: the NF consumer requests to perform a permanent disable device operation for an AIoT device or a group of AIoT devices.

Inputs, Required:

1) Information to identify the requested AF:

-	AF ID.

2) Information to be used for AIoT reader selection:

-	At least the target area information.

3) Permanent disable command specific parameters:

-  AIoT data from AF to the target AIoT device(s)

-  Approximate D2R message size.

Inputs, Optional:

1) Information about the target AIoT device(s):

-	At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices.

2) Information to be used for resource allocation:

-	Approximate number of Ambient IoT devices.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure, Correlation ID.

7.2.5	Naiotf_AIoT_Notify service operation

Service operation name: Naiotf_AmbientIoT_Notify

Description: This service operation allows the consumer to receive the status or results of the requested service operation. If the NF consumer invokes the Naiotf_AIoT_Inventory, or Naiotf_AIoT_Read or Naiotf_AIoT_Write or Naiotf_AIoT_PermanentDisable service operation, the NF consumer implicitly subscribes to receive the status or results of the requested service operation

Inputs, Required: Correlation ID, Device Operation Execution Result indication (Success or Failure), List of Ambient IoT Device ID, Failure Cause in case of Failure. AIoT data from the target AIoT device to the AF for each target Ambient IoT Device.

Outputs, Required: Operation execution result indication.

# * * * * Option 2 First change (all new texts) * * * *

7.2	AIOTF service

The following table shows the Naiotf_AIoT Service and Service Operations.

Table 7.2-1: NF services provided by the AIOTF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naiotf_AIoT | Inventory | Request/Response | AF |
|  | Command | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |

7.2.1	Naiotf_ AIoT_Inventory service operation

Service operation name: Naiotf_AIoT_Inventory.

Description: The consumer requests an inventory operation for AIoT Device(s).

Inputs, Required: AF ID, Filtering information for AIoT Devices or AIoT Device ID(s)

Inputs, Optional: target Reader ID list, target area information, the approximate number of target AIoT Devices, approximate D2R message size, report control information.

Outputs, Required: Correlation ID, operation accepted or rejected.

Outputs, Optional: None.

7.2.2	Naiotf_ AIoT_Command service operation

Service operation name: Naiotf_AIoT_Command.

Description: The consumer requests a command operation for AIoT Device(s).

Inputs, Required: AF ID, AIoT data, Filtering information for AIoT Devices or AIoT Device ID(s), service type. The service type includes Read, Write or Disable.

Inputs, Optional: Target Reader ID list, Target area information, the approximate number of target AIoT Devices, report control information.

Outputs, Required: Correlation ID, operation accepted or rejected.

Outputs, Optional: None.

7.2.3	Naiotf_ AIoT_Notify service operation

Service operation name: Naiotf_AIoT_Notify

Description: The AIOTF uses this service operation to notify the results or status of the service operation towards the consumers.

Inputs, Required: Correlation ID.

Inputs, Optional: Results of the service operation. Results of the service operation may include one of the following: Inventory Result (e.g. a list of AIoT Device ID), AIoT data from the target AIoT device to the AF for each AIoT device.

Outputs, Required: None.

Outputs, Optional: None.

7.4	AMF services

AMF supports to expose Ambient IoT services towards the AIOTF as described in Table 7.3-1.

Table 7.4-1: NF services provided by the AMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Namf_Communication | NonUeN2MessageTransfer | Request/Response | AIOTF |
| Namf_Communication | NonUeN2InfoNotify | Subscribe/Notify | AIOTF |

7.4.1	Namf_Communication_NonUeN2MessageTransfer service operation

The Namf_Communication_NonUeN2MessageTransfer service operation is as defined in clause 5.2.2.2.17 in TS 23.502 [x]. This service operation is used by AMF to transfer downlink request message from AIOTF towards the AIoT Device(s).

7.4.2	Namf_Communication_NonUeN2InfoNotify service operation

The Namf_Communication_NonUeN2InfoNotify service operations is as defined in clause 5.2.2.2.20 in TS 23.502 [x]. This service operation is used by AMF to transfer the uplink message of AIoT service operation result from the AIoT Device(s) towards AIOTF.

7.5	ADM services

The following table shows the Nadm_DM Service and Service Operations.

Table 7.5-1: NF services provided by AIOTF

| NF service name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadm_DM | Query | Request/Response | AIOTF, NEF |
|  | Update | Request/Response | AIOTF |

7.5.1	Nadm_DM_Query service operation

Service operation name: Nadm_DM_Query.

Description: NF service consumer requests the AIoT data from the ADM.

Inputs, Required: AIoT Device ID, or AF ID.

Outputs, Required: Requested subscription data.

7.5.1	Nadm_DM_Update service operation

Service operation name: Nadm_DM_Update.

Description: NF service consumer intends to update AIoT data in the ADM.

Inputs, Required: AIoT Device ID, updated AIoT data

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure.

# * * * * End of changes * * * *