3GPP TSG-WG 3GPPSA2#166-Ad Hoc-e	S2-2500932

Online, 20th JAN – 24th JAN, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR to TS 23.xyz: Profile Management for Ambient IoT Device

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: add the support of Profile Management for Ambient IoT Device.

1. Discussion

Following the conclusion in clause 8.2.2 of TR 23.700-13, this proposal introduces the support of Profile Data Management for Ambient IoT Device.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xyz.

# * * * * First change (all new texts) * * * *

5.5	AIoT Device Profile Data Management

The AIoT Device may or may not have operator’s profile data in the network. If the AIoT Device is managed by the network, then the profile data for an AIoT Device is required in the network, otherwise the AIoT device doesn’t have the operator’s profile data in the network, whereas the corresponding profile (e.g., Device ID or credentials) is stored in the AAA server external to the network.

The AIoT Device ID is used by the AIOTF together with local configuration, 3rd party related context to locate the entity stores  the profile data of an AIoT Device.

In case the AIoT Device is managed by the network, the AIOTF checks whether the AIoT Device ID from AIoT Device has a profile in the network and retrieves the profile data. The profile data for AIoT Device is different with UE subscription data as defined in TS 23.502 clause 5.2.3, it is stored in the ADM (AIoT Data Management) network entity that supports management of AIoT Device’s profile data, and this ADM network entity is independent from the UDM managing the UE subscription. The AIoT Device ID is the primary key for AIoT device profile data in the ADM.

Editor’s note: Whether ADM support ID security mechanism pending on SA3 decision.

The table 5.5-1 below describes information storage structures for AIoT device profile data.

Table 5.5-1: AIoT Device Profile Data

| Field | Description |
| --- | --- |
| AIoT Device Permanent ID | Unique identify the AIoT Device. |
| Credential information (Optional) | If credential of the AIoT device is managed by the operator, it is included. |
| Disabled flag(Optional) | Indicate the disabled status if an AIoT device to be permanently disabled |

Editor’s note:	SA WG2 will align security related materials with SA WG3 decision later.

# * * * * End of changes * * * *