3GPP TSG-SA WG2 Meeting #166AH-electronic	S2-2501014r01

# 20-25 January 2025, Online

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1581 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: Update of procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Resolve the editor’s note in TS 23.228 Clause AG.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Parameter definition of the service Nimsas_ImsSessionManagement_Create is corrected. - Editor’s note in AG.1 is removed. - Detailed call flows of adding application data channel(s) to an existing IMS session are provided for A2P, P2P, and P2A2P DC. The detailed IMS AS behaviour is also described. - Editor’s note is added that detailes for standalone IMS DC are FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Solution is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.4.2, AG.1, AG.2.1, AG.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>START OF CHANGES <<<<

#### AA.2.4.4.2	Nimsas_ImsSessionManagement_Create service operation

Service operation name: Nimsas_ImsSessionManagement_Create

Description: This service enables the consumer NF to create an IMS session with standalone data channel media.

Inputs, Required: Calling ID, Called ID, Media Information set.

Calling ID: It is the public identity of calling IMS Subscriber.

Called ID: It is the public identity of called IMS Subscriber.

When the media type indicates BDC or P2A ADC, only one ID indicating the party using the BDC or P2A ADC is required.

Media Information set: Includes a set of media information indicating how the media used in the created session. Each media information contains:

-	Media Correlation ID: It identifies the media component to be created.

-	Media Type: It indicates the media type used in the created session, e.g., Data Channel.

-	Media Parameter: It indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Data Channel, it indicates the Data Channel Type (ADC or BDC):

-	When ADC is selected, it indicates the ADC type (P2A, P2P, P2A2P) and the Application Binding Information.

-	When P2A ADC is selected, the party indicated by the ID is the target of this request using the P2A ADC.

-	When P2A or P2A2P is selected, the MDC2 media endpoint address(es) of the application layer is included.

-	When BDC is selected, it indicates the party  which uses the BDC.

Inputs, Optional: Notification Target Address and Notification Correlation ID, IMS AS instance serving the target subscriber.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1.

Outputs, Required: Result indication.

Outputs, Optional: None.

# >>>>NEXT CHANGE <<<<

Annex AG (normative):
IMS Capability Exposure Framework in IMS

# AG.1	Description

The following scenarios utilizing IMS data channel capability exposure services are depicted in clause AG.2:

-	Adding, removing or updating the data channel to an existing IMS session, for which Nimsas_ImsSessionManagement_Update is used.

-	Creating a standalone data channel session, for which Nimsas_ImsSessionManagement_Create is used.

-	Terminating a standalone data channel session, for which Nimsas_ImsSessionManagement_Delete is used.

The procedures can be triggered via the NEF on the originating or terminating side.

# >>>>NEXT CHANGE <<<<

## AG.2.1	Updating an existing IMS session

This clause describes examples depicting the procedures of adding, removing or updating bootstrap data channel(s) or application data channel(s) to an existing IMS session via the NEF services dedicated to that.

The DC AS sends a request to the NEF via Nnef_ImsSessionManagement_Update to update an existing IMS session with a specific session ID with a data channel. The procedures of adding A2P, P2P, and P2A2P application data channel(s) to an existing IMS session are provided below. For removing or updating application data channel(s) in an existing IMS session, as well as for adding, removing, or updating bootstrap data channel(s) in an existing IMS session, the same principle applies.

### AG.2.1.X	Adding A2P application data channel to an existing IMS session

Figure AG.2.1.X-1 depicts the call flow of adding A2P application data channel(s) to an existing IMS session. In this scenario, the MF-2 is used to anchor the application data channel between the UE B and the DC AS.

Figure AG.2.1.X-1: Procedure of adding A2P application data channel(s) to an existing IMS session

An IMS session between UE A and UE B has been established, and a bootstrap DC has also been established.

0.	DC AS determines to add A2P application data channel(s) to an existing IMS session, e.g. based on the event notification of the related IMS session to which the DC AS subscribed to.

NOTE 1:	It is assumed that the  non 3GPP APIs (e.g. OMA specified) can be used for non-DC session notification in step 0. The IMS Framework in Annex AD can also be used for that purpose. Hence the received notification can include different parameters depending on the deployed method for receiving notifications.

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to add A2P application data channel(s) to the existing IMS session.

The DC AS includes the session ID of the existing IMS session to be updated, the operation type as Adding, media operative set including the parameters for the media type set as A2P ADC, the UE B as the target of the A2P ADC, the application binding information, and the MDC2 media endpoint address in Nnef_ImsSessionManagement_Update request. The DC AS may include a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Update request to be notified of the session update progress. Optionally the request may include the target IMS AS instance.

2.	Unless provided in the incoming request to the NEF, the NEF uses the Nhss_ImsUECM_AsInfoGet service operation to retrieve the IMS AS instance serving UE B and discovers the IMS AS from NRF if needed.

3.	If the NEF authorizes the request,yhe NEF sends a Nimsas_ImsSessionManagement_Update request to the IMS AS-2 requiring to add A2P application data channel(s) to the specific IMS session. The received session ID of the existing IMS session, the operation type as Adding, the media type as A2P ADC, the UE B as the target of the A2P ADC, the application binding information, and the MDC2 media endpoint address shall be included in the request. The NEF may include a Notification Target Address and Correlation ID to the Nimsas_ImsSessionManagement_Update request to be notified of the session update progress. The NEF may also include the target IMS AS instance.

4.	The IMS AS-2 validates the user subscription data and checks the IMS DC capability of UE B and determines whether the data channel request should be notified to DCSF-2. If UE B does not have subscription of IMS data channel or does not have the IMS DC capability, then the request shall be rejected with appropriate cause and subsequent steps are skipped. If the IMS AS-2 allows the request to proceed, it returns a successful Nimsas_ImsSessionManagement_Update response to the NEF.

If the same IMS data channel is already established, and if there are no other operations included in the request, then the request shall be rejected with an appropriate cause and the following steps are skipped.

5.	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

6.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with event set to ExternalSessionUpdateEvent. The notification includes the necessary parameters based on the information extracted from parameters received in step 3 and other stored parameters in the IMS AS-2 if applicable..

NOTE X:	IMS AS-2 extracts from the Media operation set parameter the needed information to construct the Media Info list parameter included in the Nimsas_SessionEventControl_Notify request.

7.	After receiving the session event control notification, the DCSF-2 determines the policy how to process the application data channel establishment request based on the related parameters in the notification and/or DCSF-2 service specific policy.

8.	The DCSF-2 determines that the added application data channel media requires the DC to be anchored in MF-2.

9.		The DCSF-2 instructs the IMS AS-2 to set up the A2P application data channel(s) by sending a Nimsas_MediaControl_MediaI request.

10.	Based on the instruction from DCSF-2, the IMS AS-2 interacts with the MF-2 to allocate data channel media resource towards UE B and MDC2 media resource towards DC AS.

11.	The IMS AS-2 requests MF-2 to update DC AS side MDC2 media resource information using the received MDC2 media endpoint address in step 3.

12.	The IMS AS-2 may respond to Nimsas_MediaControl_MediaInstruction request.

13.	The DCSF-2 may respond to Nimsas_SessionEventControl_Notify.

18.	The IMS AS-2 generates a re-INVITE request(s) in which the SDP offer contains the media information of the data channel required by the DC AS, application binding information, the existing IMS session ID, together with the other existing media descriptions.

The IMS AS-2 sends the re-INVITE request(s) to the target UE B.

19.	DC media negotiation may be performed.

20.	The UE B may need to download the corresponding DC Application signalled in the SDP offer, if not done already and associate it with the requested application DC.

21.	The DC media negotiation is completed, and corresponding data channel(s) are established. The UE B returns a 200 OK response with SDP answer for the application data channel(s).

22.	The IMS AS-2 requests MF-2 to update UE B side media resource information based on the received SDP answer for the application data channel from the UE B.

23.	The IMS AS-2 sends ACK to the UE-B.

When the session update is successfully concluded, the IMS AS-2 notifies the DCSF-2 for the IMS session event with the Nimsas_SessionEventControl_Notify service operation as described in clause AC.7 with the event set to MediaChangeSuccessEvent. This is not depicted in the Figure.

25. If the DC AS included a Notification Target Address in the Nnef_ImsSessionManagement_Update request in Step 1, the IMS AS-2 notifies the NEF on the progress of the session update with the Nimsas_ImsSessionManagement_Notify request. The MF-2 side MDC2 media information is included in the request.

26.	The NEF notifies the DC AS on the progress of the session update with the Nnef_ImsSessionManagement_Notify request, including the MF-2 side MDC2 media information.

27.	The DC AS updates its MF-2 side MDC2 media information and returns with Nnef_ImsSessionManagement_Notify response to the NEF.

28.	The NEF returns a Nimsas_ImsSessionManagement_Notify response to the IMS AS-2.

Editors Note: The figure to be updated and the steps to be numbered correctly, proper names of operations, and parameters where needed.

### AG.2.1.Y	Adding P2P application data channel to an existing IMS session

Figure AG.2.1.Y-1 depicts the call flow of adding P2P application data channel(s) to an existing IMS session. In this scenario, the MF-2 is used to anchor the application data channel between the UE B and UE A.

Figure AG.2.1.Y-1: Procedure of adding P2P data channel(s) to an existing IMS session

An IMS session between UE A and UE B has been established, and the bootstrap DCs have also been established.

0.	DC AS may determine to add P2P application data channel(s) to an existing IMS session based on the event notification of the related IMS Session for which DC AS subscribed to.

NOTE 1:	It is assumed that the  non 3GPP APIs (e.g. OMA specified) can be used for non-DC session notification in step 0. The IMS Framework in Annex AD can also be used for that purpose. Hence the received notification can include different parameters depending on the deployed method for receiving notifications.

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to add P2P application data channel(s) in the existing IMS session.

The DC AS shall include session ID of the existing session, the operation type as Adding, the media type as P2P ADC, and the application binding information in Nnef_ImsSessionManagement_Update request. The DC AS includes the media operative set  parameters and may include  UE B and UE A as the targets of the P2P ADC, as well as a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Update request to be notified for the progress of the session update. Optionally the request may the target IMS AS instance.

2.	Unless provided in the incoming request, the NEF uses the Nhss_ImsUECM_AsInfoGet service operation to retrieve the IMS AS instance serving the UE B and discovers the IMS AS from NRF if needed.

3.	If authorized by the NEF, the NEF sends a Nimsas_ImsSessionManagement_Update request to the IMS AS-2 requiring to add P2P application data channel(s) in the specific session. The received IMS session ID of the existing session, the operation type as Adding, the media type as P2P ADC, and the application binding information are included in the request. The NEF may include UE B and UE A as the targets of the P2P ADC, as well as a Notification Target Address and Correlation ID to the Nimsas_ImsSessionManagement_Update request to be notified on the progress of the session update. Optionally the request may include the target IMS AS instance.

4.	The IMS AS-2 validates the user subscription data and checks the IMS DC capability of UE B and determines whether the data channel request should be notified to DCSF-2. If the UE B does not have subscription for IMS data channel or does not have the IMS DC capability, then the request shall be rejected with an appropriate cause and the subsequent steps are skipped. If the IMS AS-2 allows the request to proceed, it returns a successful Nimsas_ImsSessionManagement_Update response to the NEF.

If the same IMS data channel is already established, then the request shall be rejected with an appropriate cause and the subsequent steps are skipped.

5. 	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

6. 	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with event set to ExternalSessionUpdateEvent. The notification includes the the necessary parameters based on the information extracted from parameters received in step 3 and other stored parameters in the IMS AS-2 if applicable.

NOTE X:	IMS AS-2 extracts from the Media operation set parameter the needed information to construct the Media Info list parameter included in the Nimsas_SessionEventControl_Notify request.

7.	After receiving the session event control notification, the DCSF-2 determines the policy how to process the application data channel establishment request based on the related parameters in the notification and/or DCSF-2 service specific policy.

8.	The DCSF-2 determines that the added application data channel requires the DC to be anchored in the MF-2.

9.	The DCSF-2 instructs the IMS AS-2 to set up the P2P application data channel(s) by sending a Nimsas_MediaControl request.

10.	Based on the instruction from DCSF-2, the IMS AS-2 interacts with the MF-2 to allocate data channel media resources towards UE B and UE A.

11.	The IMS AS-2 may respond to Nimsas_MediaControl_MediaInstruction request.

12.	The DCSF-2 may respond to Nimsas_SessionEventControl_Notify.

13.	The IMS AS-2 generates a re-INVITE request(s) in which the SDP offer contains the media information of the data channel, application binding information, the existing IMS session ID with the existing media descriptions. The IMS AS-2 sends the re-INVITE request(s) to the target UE B.

14.	DC media negotiation may be performed.

15.	The UE B may need to download the corresponding DC Application signalled in the SDP offer, if not done already, and associate it with the requested application DC.

16.	The DC media negotiation is completed. The UE B returns a 200 OK response with SDP answer for the application data channel.

17.	The IMS AS-2 generates a re-INVITE request(s) in which the SDP offer contains the media information of the data channel, application binding information, the existing IMS session ID with the existing media descriptions. The IMS AS-2 sends the re-INVITE request(s) to the target UE A.

18.	DC media negotiation may be performed.

19.	The UE A may need to download the corresponding DC Application signalled in the SDP offer, if not done already, and associate it with the requested application DC.

20.	The DC media negotiation is completed. UE A returns a 200 OK response with SDP answer for the application data channel.

NOTE 2:	Step 13-16 and step 7-20 can be processed in parallel.

21.	The IMS AS-2 requests MF-2 to update UE B side and UE A side media resource information based on the received SDP answer for the application data channel from UE B and UE A.

22.	The IMS AS-2 sends ACK to the UE-B and UE A.

After the session is successfully updated, the IMS AS-2 notifies the DCSF-2 for the IMS session event with the Nimsas_SessionEventControl_Notify service operation as described in clause AC.7 with the event set to MediaChangeSuccessEvent. This is not depicted in the Figure.

23.	If the DC AS included a subscription to the DC session events in the Nnef_ImsSessionManagement_Update request in step 1, the IMS AS-2 notifies the NEF for the progress of the session update with the Nimsas_ImsSessionManagement_Notify request.

24.	The NEF notifies the DC AS for the progress of the session update with the Nnef_ImsSessionManagement_Notify request.

25.	The DC AS may return a Nnef_ImsSessionManagement_Notify response to the NEF.

26.	The NEF may return a Nimsas_ImsSessionManagement_Notify response to the IMS AS-2.

Editors Note: The figure to be updated with proper names of operations, and parameters where needed.

### AG.2.1.Z	Adding P2A2P application data channel to an existing IMS session

Figure AG.2.1.Z-1 depicts the call flow of adding P2A2P application data channel(s) to an existing IMS session. In this scenario, the MF-2 is used to anchor the application data channel between the UE B, the UE A, and the DC AS.

Figure AG.2.1.Z-1: Procedure of adding P2A2P data channel(s) to an existing IMS session

An IMS session between UE A and UE B has been established, and bootstrap DCs have been also established.

0.	DC AS may determine to add P2A2P application data channel(s) to an existing IMS session based on the event notification of the related IMS Session to which DC AS subscribed to.

NOTE 1:	It is assumed that the  non 3GPP APIs (e.g. OMA specified) can be used for non-DC session notification in step 0. The IMS Framework in Annex AD can also be used for that purpose. Hence the received notification can include different parameters depending on the deployed method for receiving notifications.

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring adding P2A2P application data channel(s) in the existing IMS session.

The DC AS shall include session ID of the existing session, the operation type as Adding, the media type as P2A2P ADC, the application binding information, and the MDC2 media endpoint address in Nnef_ImsSessionManagement_Update request. The DC AS includes the media operative set  parameters and may include UE B and UE A as the targets of the P2A2P ADC, as well as a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Update request to be notified on the progress of the session update. Optionally the request may include AS instance for the target subscriber.

2.	Unless included in the incoming request,the NEF uses the Nhss_ImsUECM_AsInfoGet service operation to retrieve the IMS AS instance serving the UE B and discovers the IMS AS from NRF if needed.

3.	If authorized by the NEF, the NEF sends a Nimsas_ImsSessionManagement_Update request to the IMS AS-2 requiring to add P2A2P application data channel(s) in the specific session. The received session ID of the existing session, the operation type as Adding, the media type as P2A2P ADC, the application binding information, and the MDC2 media endpoint address shall be included in the request. The NEF may include UE B and UE A as the targets of the P2A2P ADC, as well as a Notification Target Address and Correlation ID to the Nimsas_ImsSessionManagement_Update request to be notified for the progress of the session update. Optionally the request may include AS instance for the target subscriber.

4.	The IMS AS-2 validates the user subscription data and checks the IMS DC capability of UE B and determines whether the data channel request should be notified to DCSF-2. If the UE B does not have subscription of IMS data channel or does not have the IMS DC capability, then the request shall be rejected with appropriate cause and the subsequent steps are skipped. If the IMS AS-2 allows the request to proceed, it returns a successful Nimsas_ImsSessionManagement_Update response to the NEF.

If the same IMS data channel is already established, then the request shall be rejected with an appropriate cause and the subsequent steps are skipped.

5. 	The NEF returns a successful Nnef_ImsSessionManagement_Update response to the DC AS.

6. 	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request to the DCSF-2 with event set to ExternalSessionUpdateEvent. The notification includes necessary parameters based on the information extracted from parameters received in step 3 and other stored parameters in the IMS AS-2 if applicable..

NOTE X:	IMS AS-2 extracts from the Media operation set parameter the needed information to construct the Media Info list parameter included in the Nimsas_SessionEventControl_Notify request.

Editor's Note: the details of adding an ADC in P2P case to the IMS session are FFS.

7.	After receiving the session event notification, the DCSF-2 determines the policy how to process the application data channel establishment request based on the related parameters in the notification and/or DCSF-2 service specific policy.

8.	The DCSF-2 determines that the added application data requires to anchor the DC in the MF-2.

9.	The DCSF-2 instructs the IMS AS-2 to set up the P2A2P application data channel(s) by sending a Nimsas_MediaControl_MediaInstruction request.

10.	Based on the instruction from DCSF-2, the IMS AS-2 interacts with the MF-2 to allocate data channel media resource towards UE B and UE A, and the originating and terminating MDC2 media resource towards DC AS.

11.	The IMS AS-2 requests MF-2 to update DC AS side MDC2 media resource information using the received MDC2 media endpoint address in step 3.

12.	The IMS AS-2 may respond to Nimsas_MediaControl_MediaInstruction request.

13.	The DCSF-2 may respond to Nimsas_SessionEventControl_Notify.

18.	The IMS AS-2 generates a re-INVITE request(s) in which the SDP offer contains the media information of the data channel, application binding information, the existing session ID with the existing media descriptions. The IMS AS-2 sends the re-INVITE request(s) to the target UE B.

19.	DC media negotiation may be performed.

20	The UE B may need to download the corresponding DC Application signalled in the SDP offer, if not done already and associate it with the requested application DC.

21.	The DC media negotiation is completed. The UE B returns a 200 OK response with SDP answer for the application data channel.

22.	The IMS AS-2 generates a re-INVITE request(s) in which the SDP offer contains the media information of the data channel, application binding information, the existing session ID with the existing media descriptions. The IMS AS-2 sends the re-INVITE request(s) to the target UE A.

23.	DC media negotiation may be performed.

24.	The UE A may need to download the corresponding DC Application signalled in the SDP offer, if not done already and associate it with the requested application DC.

25.	The DC media negotiation is completed. The UE A returns a 200 OK respond with SDP answer for the application data channel.

26.	The IMS AS-2 requests MF-2 to update UE B side and UE A side media resource information based on the received SDP answer for the application data channel from UE B and UE A.

27.	The IMS AS-2 sends ACK to the UE-B and UE A.

After the session is successfully updated, the IMS AS-2 notifies the DCSF-2 for the IMS session event with the Nimsas_SessionEventControl_Notify service operation as described in clause AC.7 with the event set to MediaChangeSuccessEvent. This is not depicted in the Figure.

NOTE Z:	The details on how the DC AS updates its MF-2 side MDC2 media information are out of scope of 3GPP.

28.	If the DC AS included a subscription to the DC session events in the Nnef_ImsSessionManagement_Update request in step, the IMS AS-2 notifies the NEF for the progress of the session update with the Nimsas_ImsSessionManagement_Notify request. The MF-2 side originating and terminating MDC2 media information is included in the request.

29.	The NEF notifies the DC AS for the progress of the session update with the Nnef_ImsSessionManagement_Notify request, including the MF-2 side originating and terminating MDC2 media information.

30.	The DC AS updates its MF-2 side originating and terminating MDC2 media information, returns a Nnef_ImsSessionManagement_Notify response to the NEF.

31.	The NEF returns a Nimsas_ImsSessionManagement_Notify response to the IMS AS-2.

Editors Note: The figure to be updated and the steps to be numbered correctly.

# >>>>NEXT CHANGE <<<<

## AG.2.2	Establishing IMS session with standalone Data Channel

Figure AG.2.2-1 shows the establishment procedure of a standalone data channel session.

Editor's note:	Detailed call flows for the IMS AS behaviour when establishing a standalone IMS session are FFS.

Figure AG.2.2-1: Establishment procedure of a standalone data channel session

0.	The DC AS decides to establish a standalone data channel session. A standalone data channel can be a standalone bootstrap or standalone application data channel.

1.	The DC AS sends a Nnef_ImsSessionManagement_Create request to the NEF  to create a standalone DC session with UE A. The request includes calling ID, called ID, media information set including the instructions for the  BDC, or ADC associated with the standalone IMS DC session. Optionally, the request may include AS instance (s) for the subscribers in the session.

The DC AS may include a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Create request to be notified for the progress of the session update.

2.	If needed, the NEF queries the HSS for the address of the IMS AS serving the specific UE if needed. If the specific UE is unregistered, that is, the HSS fails to return the address of IMS AS, the NEF shall reject the Nnef_ImsSessionManagement_Create request from the DC AS with an error response and the subsequent steps are be skipped.

3.	If the NEF authorizes the request, tThe NEF sends a Nimsas_ImsSessionManagement_Create request to the IMS AS-2 requiring to create a standalone DC session.

4.	The IMS AS-2 validates the user subscription data and checks the IMS DC capability of UE to determine whether the data channel request should be notified to DCSF-2. If the service number does not have subscription for IMS data channel, then the request shall be rejected with appropriate cause. If the IMS AS-2 allows the request to proceed, it returns a successful Nimsas_ImsSessionManagement_Create response to the NEF.

5.	The NEF returns a successful Nnef_ImsSessionManagement_Create response to the DC AS.

6.		The IMS AS-2 sends a Nimsas_ImsSessionEventControl_Notify request to the DCSF-2 with event set to ExternalSessionCreateEvent. The notification includes the necessary parameters based on the information extracted from paratmetersas received in step 3.

The DCSF-2 instructs the IMS AS-2 to set up the data channel by sending a Nimsas_MediaControl_MediaInstruction request. The IMS AS-2 interacts with the MF-2 to allocate data channel resource. The procedures in clause AC.7 are reused to interact with MF.

The Nimsas_MediaControl_MediaInstruction request instructs the IMS AS to originate the indicated data channel(s) from the MF-2 using the MDC1 and/or MDC2 media endpoint address(es) as included in the Nimsas_ImsSessionManagement_Create request.

Editor's Note: the details of establishing an IMS session with ADC in P2P case are FFS.

NOTE:	The type of data channel established is dependent on the instruction given by DCSF.

7.	The IMS AS-2 generates an INVITE request(s) in which the SDP offer contains the media information of the data channel(s) required by the DC AS. The IMS AS uses the calling and called parties as indicated in the Nimsas_SessionManagement_Create request in step 3 as targets when sending the INVITE requests.

The IMS AS-2 sends the INVITE request(s) according to the IMS DC session establishment procedures described in clauses AC.7.1 and AC.7.2 to UE A and UE B with the SIP header "display-name" indicating that IMS AS-2 sends the INVITE request on behalf of the callee.

8.	The DC media negotiation is completed and corresponding data channel are established.

9.	The IMS AS notifies the DCSF for the IMS session events with the Nimsas_SessionEventControl_Notify service operation as described in clause AC.7.

If the DC AS included a Notification Target Address to the Nnef_ImsSessionManagement_Create request in step 1, the IMS AS notifies the NEF for the progress of the session creaton with the Nimsas_ImsSessionManagement_Notify service operation , based on Table AA.2.4.4.x-1.

10.	The NEF notifies the DC AS for the progress of the session creation with the Nnef_ImsSessionManagement_Notify service operation.

# >>>>END OF CHANGES<<<<