3GPP TSG-WG2 Meeting #166-Ad Hoc-e	S2-2501037

# Electronic meeting, 2025-01-20 – 2025-01-24

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0669 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures to collect data from the UE and from NG-RAN, resolution of Editor´s Note. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Progress to the resolution of Editor´s Note Editor's note: The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback. and Editor's note: The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs. Proposal 1: Refer to NRPPa TS 38.455 for the procedure to collect input data from NG-RAN tha also includes the agreed input data for case 3b at RAN1, in additional send the draft LS S2-250xxxx to ask RAN3 to provide any agreed CR or an estimation on when these CRs will be available. Removal of Editor´s Note Editor's note: Whether and how the LMF takes the RAN load into account in this procedure is FFS. Proposal 2: Use the current measurement report procedures defined in NRPPa, TS 38.455 that includes failure cases and possible cancel of report by NG-RAN. Removal of Editor´s Note: Editor's note: Further details on user consent for data collection for model training or for performance monitoring for LMF-based AI/ML positioning will be aligned with SA WG3. Proposal 3: Remove this Editor´s Note and refer to UDM authorization as done in TS 23.288. This is based on the SA3 conclusions in TR 33.784: “Regarding user consent aspect for LMF-based AI/ML model positioning, it is concluded that Annex V of TS 33.501 will be re-used as done in TS 23.288; thus, there is no normative work required.” Removal of Editor´s Note in clause 6.22.4: Editor's note: It is FFS how to adapt this procedure to ML model performance monitoring. Proposal 4: The ground truth data (and the quality) are added to the procedure to collect input data from LMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause 6.22.3 based on Proposal 1 i.e.: Refer to NRPPa TS 38.455 for both the procedure to collect input data from NG-RAN in that also input data for case 3b at RAN1, in additional send the draft LS S2-250xxxx to ask RAN3 to provide any agreed CR or an estimation on when these CRs will be available. Update clause 6.22.3 based on Proposal 2, i.e., use the current measurement report procedures defined in NRPPa, TS 38.455 that includes failure cases and possible cancel of report by NG-RAN. Remove Editor´s Note on how LMF can take into account the RAN load when collecting measurements. Update clause 6.22.2 and 6.22.3 based on Proposal 3, i.e., remove Editor´s Note to wait for SA3 on user consent for training and performance montoring, then refer to UDM checking user consent using data type “user consent”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LMF-based AI/ML positioning is not fully specified in Rel19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22, 6.22.1, 6.22.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************** 1st change **************

*************

************** next change **************

6.22	Procedures of data collection for LMF-based AI/ML Positioning

6.22.1	General

This clause describes the procedures for data collection by the LMF  to train or monitor the performance of the LMF-based AI/ML models for AI/ML-based positioning.

When training and performance monitoring is performed at NWDAF, the NWDAF sends a request for data collection including the positioning case, the area of interest where the AI/ML model is used and may provide a time window for reporting the collected data. When a time window is provided, the LMF stops input data and ground truth data collection, the NWDAF unsubscribes to LMF data collection when the time window expires.

************** next change **************

************** next change **************

************** End of changes **************