3GPP TSG-WG2 Meeting #166-Ad Hoc-e	S2-2501037

# Electronic meeting, 2025-01-20 – 2025-01-24

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0669 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures to collect data from the UE and from NG-RAN, resolution of Editor´s Note. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Progress to the resolution of Editor´s Note Editor's note: The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback. and Editor's note: The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs. Proposal 1: Refer to NRPPa TS 38.455 for the procedure to collect input data from NG-RAN tha also includes the agreed input data for case 3b at RAN1, in additional send the draft LS S2-250xxxx to ask RAN3 to provide any agreed CR or an estimation on when these CRs will be available. Removal of Editor´s Note Editor's note: Whether and how the LMF takes the RAN load into account in this procedure is FFS. Proposal 2: Use the current measurement report procedures defined in NRPPa, TS 38.455 that includes failure cases and possible cancel of report by NG-RAN. Removal of Editor´s Note: Editor's note: Further details on user consent for data collection for model training or for performance monitoring for LMF-based AI/ML positioning will be aligned with SA WG3. Proposal 3: Remove this Editor´s Note and refer to UDM authorization as done in TS 23.288. This is based on the SA3 conclusions in TR 33.784: “Regarding user consent aspect for LMF-based AI/ML model positioning, it is concluded that Annex V of TS 33.501 will be re-used as done in TS 23.288; thus, there is no normative work required.” Removal of Editor´s Note in clause 6.22.4: Editor's note: It is FFS how to adapt this procedure to ML model performance monitoring. Proposal 4: The ground truth data (and the quality) are added to the procedure to collect input data from LMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause 6.22.3 based on Proposal 1 i.e.: Refer to NRPPa TS 38.455 for both the procedure to collect input data from NG-RAN in that also input data for case 3b at RAN1, in additional send the draft LS S2-250xxxx to ask RAN3 to provide any agreed CR or an estimation on when these CRs will be available. Update clause 6.22.3 based on Proposal 2, i.e., use the current measurement report procedures defined in NRPPa, TS 38.455 that includes failure cases and possible cancel of report by NG-RAN. Remove Editor´s Note on how LMF can take into account the RAN load when collecting measurements. Update clause 6.22.2 and 6.22.3 based on Proposal 3, i.e., remove Editor´s Note to wait for SA3 on user consent for training and performance montoring, then refer to UDM checking user consent using data type “user consent”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LMF-based AI/ML positioning is not fully specified in Rel19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************** 1st change **************

### 6.22.3	Data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements

The procedure for data collection from NG-RAN for LMF-based AI/ML positioning is used to e.g. train the AI/ML Model.

Figure 6.22.3-1: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements

1.	The LMF determines that data collection is required e.g. to train an AI/ML Model for UE positioning for a number of UEs or to monitor the AI/ML Model performance. The LMF may also initiate the data collection upon the request of an NWDAF containing MTLF as described in step 3 in figure 6.22.4-1.

2.	The LMF may know the SUPI of these UEs, e.g. when training a AI/ML model using a PRU, which are associated to this LMF that executes data collection, the LMF may optionally invoke an Nnrf_NFDiscovery Request service operation to an NRF in order to identify the LMF(s) which has PRU associated to a list of TAIs. The LMF may send an Nlmf_Location_MeasurementData Request service operation to one or more of the other PRU serving LMFs in order to get the data collection and the known PRU location in this case step 6 follows.

If the LMF does not know the SUPIs of those UEs, the LMF try to get the list of SUPIs from the AMF, before that the LMF finds the AMFs via NRF using Nnrf_NFDiscoveryRequest.

3.	For each AMF serving a specific area. the LMF subscribes to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest".

4.	The AMF send Namf_EventExposure_Subscriber_Response or Namf_EventExposure_Notify ("list of SUPIs in/out the area of interest").

For each SUPI provided by the AMF that is in the area of interest then steps 5 to step 6 are performed.

4.	The LMF checks whether the SUPI provided user consent for data collection for the corresponding purpose with UDM using Nudm_SDM_Get including data type "User consent".

5.	The LMF subscribes to UDM to notifications of changes on subscription data type "User consent" for this SUPI using Nudm_SDM_Subscribe. If user consent is provided, then step 7 follows, otherwise no data is collected for this SUPI, i.e. steps 7 and 8 are not performed.

Editor's note:	Further details on user consent for data collection for a specific purpose, e.g., for model training and/or for performance monitoring for LMF-based AI/ML positioning will be aligned with SA WG3.

6.	The LMF request input data for the target UE from NG-RAN.

7.	The LMF receives ground truth data from the UE.

8.	The LMF may determine that the UE is not longer in the area of interest, based on the AMF notification using Namf_EventExposure service, then the LMF performs step 10 and step 11.

9. The UDM may notify the LMF on changes of user consent at any time after step 5. If user consent is no longer granted for a user for which data has been collected for a specific purpose, the LMF performs step 10 and step 11. The LMF may unsubscribe to be notified of user consent updates from UDM for each SUPI for which data consent has been revoked.

10.	The LMF request NG-RAN to stop reporting input data for the target UE.

11.	The LMF stop any retrieval of ground truth data from the UE.

The measurements and ground truth data are used for training, if step 1 was triggered by Nlmf_DataExposure service, the LMF provides the measurements and ground truth data to the consume. The UE location is derived from the measurements data by using LMF-based AI/ML Positioning. The derived UE location and ground truth data are used for performance monitoring.

The LMF may initiate data collection for multiple UEs simultaneously, as such step 8 and 9 may occur in parallel for a number of SUPIs as determined by the LMF.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

************** End of changes **************