SA WG2 Meeting #166 Adhoc-e	S2-2501041r01

# E-meeting, 20 – 24 January 2025	(was S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3881 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transition between S&F Mode and Normal Mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2025-01-21 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The exception sheet in SP-241918 for 5GSAT_Ph3-ARC includes for S&F operation the following: - (2b) – Functional specification of expected UE and network behaviour when transitioning between operation in S&F mode and operation in normal mode Changes are thus needed to define the transitioning in both a UE and a satellite. In addition, the conclusions of TR 23.700-29 in clause 8.2 state that: The following option is agreed for supporting Store and Forward operation with a full CN onboard the satellite with the following (informative) principles: - The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc. …… - Depending on the deployment, the UE may have a USIM enhanced for IOPS, or a USIM dedicated to the satellite network. NOTE 7: The solution does not support the roaming architectures defined in TS 23.501 [2] or TS 23.401 [5]. With a dedicated USIM for S&F mode, it is not necessary for the UE to detach for one mode when attaching for the other mode. Consequently, UE transitions between S&F and normal modes can be based on whether the UE can support simultaneous attach for normal mode and S&F mode or not. Use of a dual USIM need not be included as it can be considered implementation dependent and out of scope. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of UE and satellite requirements for transition between S&F mode and normal mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE or satellite may not be able to successfully or correctly transition from normal mode to S&F mode or the reverse which could prevent network access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9.x (new), Annex O.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.13.9.x	Transition between S&F mode and normal mode

In order for a UE to switch between S&F mode and normal mode, a UE that cannot support simultaneous attach for both modes shall detach locally, as defined in TS 24.301 [46], for the current mode after successfully completing an attach for the other mode. A UE that can support simultaneous attach for both modes shall not detach locally for the current mode after successfully completing an attach for the other mode.

NOTE 1:	Support of simultaneous attach for S&F mode and normal mode depends on UE implementation and is out of scope in this release of the specification.

When a satellite supporting normal mode loses its last feeder link, support for normal mode shall cease. Depending on operator policy and regulatory requirements, the satellite, if S&F capable, may immediately, or at a later time, switch to S&F mode. If the satellite is not S&F capable, the satellite shall wait until a feeder link becomes available before resuming support for normal mode.

When a satellite in S&F mode obtains a feeder link, and depending on operator policy and regulatory requirement, support for S&F mode may cease and support for normal mode may start. Before support for S&F mode will cease, the onboard MME may complete ongoing NAS procedures and may then perform an S1 release or detach for any UE in ECM Connected state.

NOTE 2:	For a UE in ECM Connected state for S&F mode, an S1 release could be performed when the solution in Annex O.2 is supported or a detach could be performed when the solution in Annex O.3 is supported.

# * * * * End of changes * * * *