3GPP TSG-SA WG2 Meeting #166E	S2-2501091

# Emeeting, 20 - 24 January, 2025												 (revision of S2-2500293)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1520 | rev | 4 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Call setup of UE-Satellite-UE communication with early media in IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Handling of early media for UE-satellite-UE communication has been discussed. S2-2411950 discuss the issue for early media handling, and proposes to sustain using ground-AGW for early media based on operator policy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of procedure for supporting UE-Satellite-UE communication with early media in IMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Early media may be terminated or interrupted when UE-satellite-UE communication is activated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AE.5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change (all new text) ---

AE.5.x	At call setup with early media

During the call setup with both originating and terminating UEs using regenerative NR satellite access, the originating P-CSCF uses the IMS-AGW on ground to transfer early media then, after the IMS session is answered, releases the IMS-AGW on ground and selects the IMS-AGW on satellite to transfer regular media. See Figure AE.5.x-1 that describes the procedure.

In this procedure, the IMS AS for early media uses existing method for providing early media and updating the IMS session when answered.

Figure AE.5.x-1: IMS session establishment procedure with early media

1-11.	Steps 1-11 of clause AE.5.1 apply with the difference that the IMS AS serving early media allocates early media resources with MRF, as well as includes the P-Early-Media header field and a modified SDP answer for early media in the 18X response.

The IMS AS may instruct the MRF to play early media any time before step 12.

12.	Based on the identification of the satellite serving UE B and the P-Early-Media header field in step 11, P-CSCF A determines to perform UE-satellite-UE communication with early media. P-CSCF A selects an IMS-AGW A on satellite but does not release the IMS-AGW A on ground in order to use it for early media.

12a-12b.	P-CSCF A interacts with IMS-AGW A on ground for early media to allocate and configure transport resources for IMS access side as well as configure transport resources for IMS core network side.

13. Step 13 of clause AE.5.1 applies.

The early media is able to be sent to the UE A via the IMS AGW A on ground, the UPF A on ground, and the UPF A on satellite.

14.	Step 14 of clause AE.5.1 applies with the difference that P-CSCF A includes in the SIP 18X response an SDP answer containing the transport address of IMS-AGW A on ground obtained in step 12.

15-16.	Steps 15-16 of clause AE.5.1 apply with difference that P-CSCF A does not generate/modify SDP offer in step 16. Step 17 of clause AE.5.1 is not performed because no SDP offer for regular media is received.

17-18.	Steps 18-19 of clause AE.5.1 apply and UE B is ringing.

19.	The UE B answers the IMS session and sends SIP 200 response for the INVITE.

20.	The IMS AS serving early media instructs the MRF to stop playing early media. UE A may play local ring tone when early media is stopped.

21.	The IMS AS serving early media triggers the SDP re-negotiation procedure between UE A and UE B using existing mechanism, e.g., sending SIP ACK and SIP reINVITE without SDP information towards UE B.

NOTE 1:	In order to reduce the latency, P-CSCF B can delegate UE B, and/or, P-CSCF A can delegate UE A to perform the SDP re-negotiation procedure, i.e., does not interact with UE.

22.	During the SDP re-negotiation procedure, P-CSCF A releases the IMS AGW A on ground and interacts with the IMS-AGW A on satellite selected in step 12 for regular media to allocate and configure transport resources for both IMS access side and IMS core network side.

23.	The SDP re-negotiation procedure continues.

After receiving the SIP 200 response for the INVITE, the UE A is able to send regular media and is ready to receive regular media. The regular media uses UE-satellite-UE communication path.

24.	During the SDP re-negotiation procedure, P-CSCF B updates the allocated IMS-AGW on the satellite for UE B with the transport address in the received SDP information for regular media to configure transport resources for IMS core network side.

25.	P-CSCF B forwards the SIP ACK towards UE B.

After receiving the SIP ACK, the UE B is able to send regular media.

## ---End of the Change---