3GPP TSG SA WG2#166AH-e	            S2-2501115

Electronic Meeting, 20 – 24 January 2025	(revision of S2-2501063r01)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0298 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of Editor’s Note on AF traffic influence |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Samsung, ZTE, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Per the existing specs, SMF provides information that would cover a set of parameters in AF traffic influence listed in clause 5.6.7.1 of TS 23.501. SMF either sends the information as part of N4 rules or encapsulated form in PFCP request. However, the following AF traffic influence parameters that are part of Edge_Ph3 scope, are currently not provided to I-SMF and should be newly sent from SMF to I-SMF: - User Plane Latency Requirement - Indication of considering N6 delay These parameters can be included in TS 23.502 and respective stage 3 specs. Therefore, there is no need to transfer the AF traffic influence from SMF to I-SMF, hence, the existing Editor’s Note can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removes Editor’s Note on AF traffic influence and describes how AF traffic infleunce request is provisioned to I-SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 5332 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 6.10.3.1	General

For Local Offloading Management, traffic routing related policy (e.g., Local Offloading Management Policy) need be provided to the I-SMF. AF provides traffic influence request as defined in clauses 4.3.6 of TS 23.502 [3], and procedure as defined in 4.23.6 of TS 23.502 [3] is performed to provision the related information to I-SMF.

Based on the received traffic routing related policy information and EAS Deployment information, the I-SMF may (re)discover and (re)select ULCL and local PSA UPF for routing the traffic locally.

NOTE:	The AF does not need to be aware of whether Local Offloading Management is applied or not.

End of CHANGES