3GPP TSG SA WG2#166AH-e	            S2-2501121

Electronic Meeting, 20 – 24 January 2025	(revision of S2-2501061r05)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0296 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the procedures defined for local offloading management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Huawei, LGE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The cases defined for UE mobility for Local Offloading Management should cover the case where UE location is not within the anchor SMF service area and either UE location is within/outside Local Offloading Management service area. Also, I-SMF insertion/change/removal/reuse cases require further details to clarify interaction between target and source AMFs, I-SMFs and old/new I-SMF and SMF. Similar to HR-SBO, the retrieval of Local Offloading Management Allowed indication can be part of UE context – see corresponding CR for further details S2-2500abc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Further cases are defined as part of UE mobility in clause 6.10.2.4. - I-SMF insertion/change/removal/reuse cases are further clarified in clause 6.10.2.4 by referring to HR-SBO procedures defined for the same scenarios. - Local Offloading Management Allowed indication retrieval is updated to include UE context transfer between the target and source AMFs as in HR-SBO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear and incomplete specification related to Local Offloading Management. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 6.10.2.4	UE mobility for PDU session supporting Local Offloading Management

For a PDU session supporting Local Offloading Management, during mobility events, e.g. Hand-Over or AMF change, the mobility handling is executed same as the respective mobility handling defined in clause 4.23 of TS 23.502 [3] and clause 6.7.2 with the following difference:

-	The retrieval of the Local Offloading Management Allowed indication either as part of UE context where source AMF provides the target AMF with whether Local Offloading Management is allowed for each PDU session of the UE or the SMF selection subscription data from UDM, the new AMF determines the new I-SMF as follows:

-	If the service area of SMF includes the new UE location and the UE location is out of local offloading management service area, the AMF removes the old I-SMF (if available).

-	If the service area of SMF does not include the new UE location and the UE location is out of local offloading management service area, the AMF follows the procedure defined in clause 4.23 of TS 23.502 [3] without supporting Local Offloading Management with the following differences:

-	If the old I-SMF cannot serve the new UE location, the AMF performs I-SMF change procedure described in Figure 4.23.7.3.2-1 of TS 23.502 [3]. During the handover execution phase per clause 4.23.7.3.3 of TS 23.502 [3], the new I-SMF invokes Nsmf_PDUSession_Update Request to the SMF. Then, the SMF sends PCO to the UE including central DNS server address or EASDF IP address selected by SMF.

-	If the old I-SMF can serve the new UE location, the AMF may keep the old I-SMF and send Nsmf_PDUSession_UpdateSMContext Request without local offloading management allowed indication. In this case, the old I-SMF deletes the DNS Context from the old EASDF and sends Nsmf_PDUSession_Update Request without local offloading management allowed indication to the SMF so that the SMF sends PCO to the UE including central DNS server address or EASDF IP address selected by SMF.

-	If the service area of SMF either includes or does not include the new UE location and the UE location is within local offloading management service area, the AMF changes the existing I-SMF with the new I-SMF that serves the local offloading management service area unless the existing I-SMF can serve the new UE location and local offloading management service area. If there is an old I-SMF which cannot serve the new UE location, the AMF also removes the old I-SMF.

-	To support the local offloading management, the selected I-SMF needs to support Local Offloading Management, serves the new UE location and local offloading management service area; and support the DNN and S-NSSAI associated with the PDU session.

NOTE:	When the UE moves from an AMF which is not upgraded to act based on the Local Offloading Management support indication to an AMF which is upgraded to act based on the Local Offloading Management support indication, the above procedure also applies.

-	For I-SMF insertion/change/removal/reuse, the procedure described for intra (V-)PLMN HO in clauses 6.7.2.6, 6.7.2.7, 6.7.2.8, 6.7.2.9 and 6.7.2.10 is performed with the following differences:

-	removing V-PLMN, H-PLMN, and steps defined for serving PLMN change case,

-	replacing:

-	V-SMF with I-SMF,

-	H-SMF with SMF,

-	V-EASDF with EASDF,

-	V-UPF with I-UPF,

-	HR-SBO with Local Offloading Management (including allowed indication, policy and offload identifiers),

-	enabling the anchor SMF to send PCO to the UE, and

-	not using PLMN ID in Neasdf_DNSContext_Create Request from I-SMF to retrieve EASDF IP address.

End of CHANGES