3GPP TSG-WG SA2 Meeting #166-AdHoc-e	S2-2501138

# E-meeting, January 20 – 24, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0656 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on data collection by LMF for LMF-based AIML Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - There is no “UE positioning capability” Event ID for the AMF, and there may be no UE Positioning Capability and UE User Plane Positioning Capabilities available in the AMF. LMF can also query for UE positioning capabilities (e.g. via LPP messages) if the UE positioning capabilities are not received from AMF. - The user consent check is not needed for data collection from the PRUs, no matter it is for model performance monitoring or model training. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the AMF optionally provides UE positioning capabilities to the LMF. - Further clarify that user consent check is not needed for data collection from the PRUs for LMF-based AI/ML positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect and unclear procedures on data collection by LMF for LMF-based AI/ML positioning. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.1, 6.22.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

6.22.1	General

This clause describes the procedure for data collection by the LMF for input data to train or monitor the performance of the ML model for LMF-based AI/ML positioning.

6.22.2	Data Collection at LMF for the LMF-based AI/ML positioning based on UE measurements

The LMF may obtain input data from the UEs (including PRUs) for ML model training and performance monitoring for LMF-based AI/ML Positioning, and the input data include measurement data and locations of the UEs. The LMF-based AI/ML Positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TAs or cover multiple NG-RAN nodes.

Figure 6.22.2-1: Data collection by LMF for LMF-based AI/ML positioning based on UE measurements

1.	The LMF decides to start data collection from UEs for the purpose to train an ML Model or perform model performance monitoring for LMF-based AI/ML Positioning. This may be based on an internal trigger in the LMF or a request from the NWDAF. The LMF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover AMF(s) that served for the area of interest.

Editor's note:	Whether the LMF will subscribe at NWDAF for Network Performance Analytics to decide when to trigger data collection is FFS.

2.	The LMF subscribes to the AMF(s) to retrieve the list of SUPIs located in an area of interest and optionally the UE Positioning Capability for each UE using Namf_EventExposure_Subscribe request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest", [indication of requesting UE Positioning Capability]).

Editor's note:	Whether and how to limit the number of UEs is FFS.

3.	The AMF sends Namf_EventExposure_Subscribe response (list of SUPIs in the area of interest). If UE Positioning Capability is also requested in step 1, AMF includes UE Positioning Capability and optionally UE User Plane Positioning Capabilities, if available, for each UE in the response message sent to LMF.

4.	The LMF further determines the UEs from the list of SUPIs that are received from AMF in step 3for data collection based on e.g. UE Positioning Capability, UE User Plane Positioning Capabilities, UE consent check result, the PRU information available in the LMF and operator's policy. The User consent check is not needed for data collection from PRU for model performance monitoring. The LMF may query the UE for UE Positioning Capability and optionally UE User Plane Positioning Capabilities if not received in step 3.

Editor's note:	Further details on user consent for data collection for model training or model performance monitoring for LMF-based AI/ML Model positioning will be aligned with SA WG3.

5.	For each UE that provides consent to data collection for model training and can support data collection, the LMF initiates a request for input data. The UE may reject the data collection request from the LMF (e.g. considering UE status, user's input). If the UE accepts data collection request, the UE may cancel the data collection later as defined in clause 6.3.4.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the UE is FFS. The procedure will be coordinated with RAN WGs.

Editor's note:	The input data used for AI/ML based positioning will be decided by RAN WGs.

At the time the LMF decides that the input data for a UE are not needed, e.g. the user consent for data collection for model training is revoked, the LMF stops to collect the input data from the UE.

* * * End of Changes * * * *