3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2501177

# E-meeting, Jan 20 – 24, 2025		(revision of S2-2500570)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0649 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RangingSideLink positioning procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In TS 23.586 (Ranging based services and Sidelink Positioning), the term of UE’s capability is “Ranging/Sidelink Positioning capability”. But in TS 23.273 clause 6.20, the description of “UE’s SL positioning capability” could be misleading as another kind of capability. - Typo in clause 6.20.5: “the types of required location results is absolute solution”, where the “solution” should be “location”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify the UE’s “SL positioning capability” to “Ranging/Sidelink positioning capability”. - Fix typo “solution” to “location”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect terms causing confusion to the readers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.20.2, 6.20.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.20.2	5GC-MO-LR Procedure using Ranging/SL positioning

This procedure is used to estimate the location of a UE based on the location of one or more Located UEs and the distance and/or direction between the UE and the Located UE(s). 5GC-MO-LR Procedure as defined in clause 6.2 applies with the following differences:

-	In step 3, the AMF may take the UE's Ranging/Sidelink Positioning capability into account for LMF selection.

-	In step 4, the AMF provides the UE's Ranging/Sidelink Positioning capability to LMF.

-	Step 5 is replaced by step 10-16 of clause 6.20.3 with the following adaptations:

-	UE1 is the target UE, and UE2/.../UEn is the Located UE.

-	In step 10, the type of required location results is absolute location, and the other UEs 2 to n are the candidate Located UE(s) if included. After LMF determines that the assistance of Located UE is needed for Target UE Positioning, LMF decides that Target UE or LMF selects Located UE, and SL-MT-LR request also includes the indication of Target UE/LMF selecting Located UE. LMF includes capabilities of each candidate Located UE in the request, if available, when Target UE selecting Located UE is indicated.

-	In step 14, if UE1 receives the indication of LMF selecting Located UE in step 11, SL-MT-LR response includes the obtained information of all the discovered Located UEs. LMF performs the Located UE selection based on the obtained information of all the discovered Located UEs, and sends Application Layer ID of the selected Located UEs to the UE1 in step 16. If UE1 receives the indication of Target UE selecting Located UE in SL-MT-LR request, UE1 performs the Located UE selection, and SL-MT-LR response includes Application Layer ID of the selected Located UEs.

# * * * * Second change * * * *

6.20.5	5GC-MT-LR Procedure using Ranging/SL positioning

The procedure is used to estimate the location of a UE by using the location of one or more Located UEs and the distance and/or direction between the UE and the Located UE(s).

NOTE:	The procedure can be triggered by GMLC, e.g. if the location result of Target UE determined by previous MT-LR procedure for the same request cannot fulfil the required QoS.

Procedure of Figure 6.1.2-1 can be reused, with the following adaptations:

-	In step 10 of Figure 6.1.2-1, the AMF may take the UE's Ranging/Sidelink Positioning capability into account for LMF selection.

-	In step 11 of Figure 6.1.2-1, the AMF provides the UE's Ranging/Sidelink Positioning capability to LMF.

-	Step 12 of Figure 6.1.2-1 replaced by the step 10-16 of Figure 6.20.3-1 with the following adaptations:

-	UE1 is the Target UE, and UEs 2 to n are Located UEs.

-	In step 10, the type of required location results is absolute location, and the other UEs 2 to n are the candidate Located UE(s) if included. After LMF determines that the assistance of Located UE is needed for Target UE Positioning, LMF decides that Target UE or LMF selects Located UE, and SL-MT-LR request also includes the indication of Target UE/LMF selecting Located UE. LMF includes capabilities of each candidate Located UE in the request, if available, when Target UE selecting Located UE is indicated.

-	In step 14, if UE1 receives the indication of LMF selecting Located UE in step 11, SL-MT-LR response includes the obtained information of all the discovered Located UEs. LMF performs the Located UE selection based on the obtained information of all the discovered Located UEs, and sends Application Layer ID of the selected Located UEs to the UE1 in step 16. If UE1 receives the indication of Target UE selecting Located UE in SL-MT-LR request, UE1 performs the Located UE selection, and SL-MT-LR response includes Application Layer ID of the selected Located UEs.

# * * * * End of changes * * * *