3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2501289

# E-meeting, Jan 20 – 24, 2025		(revision of S2-2500107)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0525 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Discovery of U2U Multihop Relay for IP PDU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Based on the Conclusions for multihop U2U Relay for IP type PDU in TR 23.700-03, the maximum number of hops is used to limit the hops when performing discovery and building the routing tables. For IP type U2U Relay, the hop limit is enforced by limiting the MANET Discovery Info message propagation depeth based on configurations on the U2U Relay. In some cases, the 5G ProSe End UE may want to has a lower hop limit for communications with other End UEs. It is desirable to have some mechanism to support signalling such limit to the U2U Relay UE. - The multi-hop U2U Relay may connect to different MANET with different RSCs. Therefore, the DNS entries are built per RSC. The similar clarifications for Intermediate U2N Relay is described in clause 6.4.3.8, “The local IP routing table is per RSC and used based on RSC of the link the packet is received.” So, for U2U Relay related clarifications on how the U2U Relay determines the RSC should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the End UE could provide a hop limit for the DNS query-based discovery, which will be used by the UE-to-UE Relay when handling DNS info. - Clarify that the U2U Relay determines the RSC corresponding to the DNS query message based on the local PC5 link information when handling DNS query. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - 5G ProSe End UE is unable to limit discovery hops in multihop U2U Relay for IP type PDU. - Unclear what DNS entry is used for DNS-based U2U discovery. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.10.1, 5.14.3.2, 6.7.5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.4.3.10.1	Layer-2 Link establishment and management for IP PDU type

For the 5G ProSe Communication via 5G ProSe UE-to-UE Relay as described in clause 6.7.5.2:

-	The Direct Communication Request message over the PC5 reference point (between a 5G ProSe End UE and 5G ProSe Multi-hop UE-to-UE Relay, or between 5G ProSe Multi-hop UE-to-UE Relays) includes:

-	User info (i.e. Application Layer ID) of source 5G ProSe End UE or User Info ID of 5G ProSe Multi-hop UE-to-UE Relay: the identity of the source 5G ProSe End UE or 5G ProSe Multi-hop UE-to-UE Relay requesting the connection establishment.

-	User Info ID of 5G ProSe Multi-hop UE-to-UE Relay: the identity of the target 5G ProSe Multi-hop UE-to-UE Relay provided during the 5G ProSe Multi-hop UE-to-UE Relay Discovery procedure.

-	(optional)ProSe Service Info: the information about the ProSe identifier(s) requesting Layer-2 link establishment.

-	RSC: the connectivity service provided by the 5G ProSe Multi-hop UE-to-UE Relay as requested.

-	Security Information: the information for the establishment of security for the PC5 link establishment (see TS 33.503 [29]).

-	(Optional) Hop-Limit: the 5G ProSe End UE may include a hop-limit value smaller than the configured "maximum number of hops per RSC", if it desires to reduce the number of hops the other End UE can communicate with it via the relays.

The Direct Communication Accept message over the PC5 reference point (between a 5G ProSe End UE and 5G ProSe Multi-hop UE-to-UE Relay, or between 5G ProSe Multi-hop UE-to-UE Relays) includes:

-	User Info ID of 5G ProSe UE-to-UE Relay: the identity of the 5G Prose Multi-hop UE-to-UE Relay sending the Accept message.

Other link management procedures as defined in clause 6.4.3 (i.e. Link identifier update in 6.4.3.2, Layer-2 link release in clause 6.4.3.3, Layer-2 link modification in clause 6.4.3.4, Layer-2 link maintenance in 6.4.3.5) can be used for the layer-2 link without enhancements.

When a new layer-2 link is established, or an existing layer-2 link is released, the 5G ProSe Multi-hop UE-to-UE Relay need to inform the status change internally to the MANET router stack.

Each of the established layer-2 link with another 5G ProSe UE-to-UE Relay is treated as a MANET interface as defined in RFC 6130 [36]. The layer-2 links of the same 5G ProSe UE-to-UE Relay may share the same IP address, as they are isolated, i.e. each of the layer-2 link only allows communication with one peer 5G ProSe UE-to-UE Relay.

The 5G ProSe UE-to-UE Relay may based on local configuration use a large value for HELLO_INTERVAL, VALIDITY_TIME and REFRESH_INTERVAL as defined in RFC 6130 [36] to reduce the frequency of periodic HELLO messages. The 5G ProSe UE-to-UE Relay can use the information from 5G ProSe link management procedures defined in clause 6.4.3 for maintaining the information in Neighbor Information Base.

# * * * * Second change * * * *

#### 5.14.3.2	IP based 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication

The RSC configuration as defined in defined in 5.1.5.1 indicates if IP based 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication is used. The 5G ProSe Multi-hop UE-to-UE Relays supporting the same RSC establishe IP based Layer-2 links with each other and form a relay cloud to provide IP connectivity for 5G ProSe End UEs connected to any of the 5G ProSe Multi-hop UE-to-UE Relays.

NOTE:	If a 5G ProSe Multi-hop UE-to-UE Relay support multiple RSCs, different Layer-2 links will be established, with each associated with a specific RSC. The 5G ProSe UE-to-UE Relay operates as separate logical entities in the separate relay clouds of different RSCs, i.e. no routing table sharing and no traffic forwarding across different relays clouds of different RSCs.

The 5G ProSe End UE may establish multiple Layer-2 links with different 5G ProSe UE-to-UE Relays supporting the same RSC, in order to maximize the reachability (e.g. in some cases, different 5G ProSe UE-to-UE Relays may serve different disjointed IP subnets).

Mobile Ad-hoc Network (MANET) based routing protocols are used over the IP connectivies in the relay cloud to provide multi-hop forwarding functionality between 5G ProSe End UEs. Each of the 5G ProSe Multi-hop UE-to-UE Relay acts as a MANET router. The exact MANET routing protocols to be supported, e.g. Neighborhood Discovery Protocol (NHDP) [36], OLSRv2 [35], are associated with the RSCs based on configuration.

Based on configuration associated with the RSC, the 5G ProSe End UE obtains the IP address/prefix in two options:

-	Each of the 5G ProSe End UE may be configured with a routable IP address/prefix associated with the RSC. This IP address/prefix does not change when the 5G ProSe End UE changes the connections with 5G ProSe Multi-hop UE-to-UE Relays.

-	If the 5G ProSe End UE does not have an IP address/prefix configured for the RSC, it obtains an IP address/prefix from the 5G ProSe UE-to-UE Relay it connects to. In this case, the 5G ProSe End UE may need to change the IP address/prefix when it changes its connection to another 5G ProSe UE-to-UE Relay.

The 5G ProSe End UE obtains the IP address/prefix of the target 5G ProSe End UE via DNS queries (per RSC). 5G ProSe Multi-hop UE-to-UE Relay needs to support DNS operations.

Once the 5G ProSe Multi-hop UE-to-UE Relay becomes aware of the 5G ProSe End UE's IP address/prefix, the 5G ProSe Multi-hop UE-to-UE Relay propagates the related DNS information (i.e. User Info ID and associated IP address/prefix) to all other 5G ProSe Multi-hop UE-to-UE Relays connected to the same MANET (i.e. corresponding to the same RSC) using a dedicated MANET Discovery Info message. The MANET Discovery Info message is diffused throughout the MANET using Multipoint Relay flooding (MPR flooding) as defined in IETF RFC 7181 [35], if available. If MPR flooding is not available, then the MANET Discovery Info message is diffused using simple flooding i.e. by transmitting a copy of the MANET Discovery Info message on all interfaces except the one on which the message has been received.

The MANET Discovery Info message is based on the generalized MANET message format defined in IETF RFC 5444 [37]. The 5G ProSe Multi-hop UE-to-UE Relay initiating the sending of the MANET Discovery Info message controls the propagation depth of the MANET Discovery Info message based on the configured "maximum number of hops per RSC" as specified in 5.1.5.1a or based on the Hop Limit if provided by the target End UE, by using the built-in mechanism defined in IETF RFC 5444 [37].

When MANET protocol determines the route for IP forwarding, layer-2 link QoS and hop counts can be refelected, e.g. with the Link Metrics value as defined in OLSRv2 [35]. The 5G ProSe Multi-hop UE-to-UE Relay can also enforce the configured hop limit based on the link metrics when building the routing tables, e.g. ignore the route that exceeds the threshold derived from hop limit.

If 5G 5G ProSe Multi-hop UE-to-UE Relay has traffic for its own, it can use any IP address it owns for the transmission.

# * * * * Third change * * * *

##### 6.7.5.2.1	Layer-2 link establishment via 5G ProSe Layer-3 Multi-hop UE-to-UE-Relays (IP type)

Figure 6.7.5.2.1-1 shows the Layer-2 link establshment procedure via 5G ProSe Layer-3 Multi-hop UE-to-UE-Relays. This procedure is used by an 5G ProSe End UE to connect to the relay cloud, or for a 5G ProSe Layer-3 Multi-hop UE-to-UE-Relay to connect to other 5G ProSe Layer-3 Multi-hop UE-to-UE-Relays.

Figure 6.7.5.2.1-1: 5G ProSe Communication via IP type of 5G ProSe Multi-hop UE-to-UE Relay

0.	The 5G ProSe Layer-3 Multi-hop UE-to-UE-Relays and the other 5G ProSe End UEs have already established IP type of connections. The 5G ProSe Layer-3 Multi-hop UE-to-UE-Relays and 5G ProSe End UEs may have used the discovery procedure as defined in clause 6.3.2.6.2 and established the Layer-2 link as defined in clause 6.4.3.10.1.

1.	The 5G ProSe End UE performs the 5G Multi-hop UE-to-UE Relay discovery procedure as described in clause 6.3.2.6.2 and finds the 5G Multi-hop UE-to-UE Relay-1.

2.	The 5G ProSe End UE establishes an IP based Layer-2 link with the 5G ProSe Multi-hop UE-to-UE Relay-1 as described in clause 6.4.3.10.1.

Depends on the configuration, the 5G ProSe End UE may obtain an IP address/Prefix from the 5G ProSe Multi-hop UE-to-UE Relay-1. Otherwise, the 5G ProSe End UE informs the 5G ProSe Multi-hop UE-to-UE Relay-1 of its pre-configured IP address/prefix.

The 5G ProSe End UE may also configure the corresponding services and PC5 QoS Flows over the Layer-2 like with the 5G ProSe Multi-hop UE-to-UE Relay-1. The 5G ProSe End UE may use the Layer-2 link modification procedure as defined in clause 6.4.3.4 to add/modify/remove PC5 QoS Flows corresponding to the services and link status.

3.	The 5G ProSe Multi-hop UE-to-UE Relay-1 updates its records of the 5G ProSe End UEs and may trigger the MANET signalling, e.g. OLSRv2 signalling [35], to update other connected 5G ProSe Multi-hop UE-to-UE Relays. This will result in a routing table on the relays that allows other UEs to reach the newly connected 5G ProSe End UE.

The 5G ProSe Multi-hop UE-to-UE Relay may also trigger signalling to update the DNS entries in other connected 5G ProSe Multi-hop UE-to-UE Relays for the 5G ProSe End UE using the MANET Discovery Info message, as described in clause 5.14.3.2.

4.	The 5G ProSe End UE may perform a DNS query for a target 5G ProSe End UE, if it does not know the IP address/prefix of the target. The DNS query may stop at the 5G ProSe Multi-hop UE-to-UE Relays that has the entry for the target 5G ProSe End UE.

The UE-to-UE Relay should only use the DNS entry associated with the RSC, corresponding to the PC5 link that the DNS query message is received.

5G ProSe End UE starts to communicate with the target 5G ProSe End UE using IP based communications. The 5G ProSe Multi-hop UE-to-UE Relays forwards the packets based on the routing table established with the MANET protocols.

# * * * * End of changes * * * *