3GPP TSG-WG SA2 Meeting #166 -Ad Hoc-e	S2-2501291

# 20-24 Jan 2025, Emeeting		(revision of S2-2500764r02)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0539 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for ProSe Multihop U2U Relay reselection for Non-IP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In 6.7.4.1 the 5G ProSe End UE trigged multi-hop Layer3-UE-to-UE Relay reselection only considers the case when 5G ProSe End UE detects the link failure or link quality degradation, it dose not include the case when 5G ProSe UE-to-UE Relay detects the link failure or link quality degradation. As a reference, the 5G ProSe Intermediate UE-to-Network Relay already supprts to inform the 5G ProSe Remote UE to trigger the multi-hop Layer3-UE-to-UE Relay reselection, when the 5G ProSe Intermediate UE-to-Network Relay detects the link failure or link quality degradation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding description of 5G ProSe UE-to-UE Relay sending Relay re-selection indication to 5G ProSe End UE to trigger the multi-hop Layer3-UE-to-UE Relay reselection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ProSe Multihop U2U Relay reselection will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new)* * * *

6.7.4.1	General

After being connected to the 5G ProSe UE-to-UE Relay, the 5G ProSe End UEs may trigger the 5G ProSe UE-to-UE Relay reselection based on conditions (e.g. the measured signal strength of PC5 unicast link with the 5G ProSe UE-to-UE Relay) as specified in TS 38.300 [12]. For 5G ProSe multi-hop Layer-3 UE-to-UE Relay for non-IP PDU, when a 5G ProSe UE-to-UE Relay in the path detects link failure or link quality degradation to its adjacent 5G ProSe UE-to-UE Relay, it sends Relay reselection indication and optionally a cause value to 5G ProSe End UE. After receving the reselection indication the 5G ProSe End UE may trigger the multi-hop Layer-3 UE-to-UE Relay reselection.

For 5G ProSe UE-to-UE Relay reselection, a 5G ProSe UE-to-UE Relay may be discovered by either the discovery procedures defined in clause 6.3.2.4 or by the negotiated 5G ProSe UE-to-UE Relay reselection procedure defined in clause 6.7.4.2 or clause 6.7.4.3. For 5G ProSe multi-hop Layer3 UE-to-UE Relay for non-IP, in addition, a 5G ProSe UE-to-UE Relay may be discovered by the discovery procedure defined in clause 6.3.2.6.3, by the negotiated 5G ProSe multi-hop Layer3-UE-to-UE Relay reselection for non-IP type PDU defined in clause 6.7.4.4.

In the negotiated UE-to-UE Relay reselection defined in clause 6.7.4.2 or clause 6.7.4.3, one 5G ProSe End UE initiates the UE-to-UE Relay reselection procedure, the 5G ProSe End UEs can negotiate a new 5G ProSe UE-to-UE Relay using the existing connection and to establish the communication via the reselected 5G ProSe UE-to-UE Relay prior to releasing the communication via the current 5G ProSe UE-to-UE Relay.

# * * * * End of changes * * * *