SA WG2 Meeting ##166-Ad-Hoc-e	S2-2500047

20 - 24 January, 2025, Electronic meeting

3GPP TSG-RAN WG2 Meeting #128	R2-2411197

Orlando, USA, Nov. 18th – 22nd, 2024

Title:	LS on Satellite IDs for store-and-forward operation

Response to:	-

Release:	Rel-19

Work Item:	IoT_NTN_Ph3-Core

Source:	RAN2

To:	SA2

Cc:	-

Contact person:	Xiao XIAO

xiaoxiao@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN2 discussed how an IoT NTN UE capable of store-and-forward operation uses the MME-configured satellite ID list in the access stratum. RAN2 made the assumption that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite.

RAN2 would like to request SA2 to provide feedback on whether the following RAN2 understanding can be confirmed:

- RAN2 understanding: The UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

Also, RAN2 would like to respectfully ask the question below to SA2:

Question: If a satellite is included in the satellite list signalled by the MME to a UE, does it mean one of the below options, or possibly other option(s):

- the satellite has the UE context but does not necessarily support store-and-forward operation; or

- the satellite has the UE context and must support store-and-forward operation (but may be operating in normal IoT NTN mode or in store-and-forward mode when the satellite serves the area where the UE is located)?

# 2	Actions

To SA2

ACTION: 	RAN2 respectfully asks SA2 to provide feedback on whether above RAN2 understanding can be confirmed and provide an answer to the Question above.

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #129	2025-02-17 - 2025-02-21	Athens, GR

TSG RAN WG2 Meeting #129bis	2025-04-07 - 2025-04-11 	TBD, CN