SA WG2 Meeting ##166-Ad-Hoc-e	S2-2500128

20 - 24 January, 2025, Electronic meeting

# Title:	[Draft] Reply LS on Multi-hop U2N Relay Architecture Aspects

# Response to:	LS on Multi-hop U2N Relay Architecture Aspects

# Release:	Rel-19

# Work Item:	5G_ProSe_Ph3

Source:	SA2

To:	SA3

Cc:	CT1

Contact Person:

Name:	Toumi Nassima

E-mail Address:	Nassima.toumi@tno.nl

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

SA2 thanks SA3 for the LS on Multi-hop U2N Relay Architecture Aspects (S2-2412361/ S3-245259).

SA2 has discussed the raised questions and would like to provide the following answers.

Q1: Intermediate Relay’s PKMF reachability via dedicated DNN for UE-to-Network Relay

Some solutions propose that the Intermediate Relay plays the role of the UE-to-Network Relay during link establishment procedure with the Remote UE (or with a child Intermediate Relay). For that, the Intermediate Relay is assumed to be able to reach its HPLMN’s PKMF via the dedicated DNN for UE-to-Network Relay (after successful connection establishment with the UE-to-Network Relay). The same assumption applies for each Intermediate Relay in the path between the Remote UE and the UE-to-Network Relay. Can SA2 verify the assumption, i.e., whether an Intermediate Relay can use the dedicated DNN for UE-to-Network Relay to communicate with its HPLMN’s PKMF?

A1: Yes. SA2 assumes that the Intermediate Relay can use the dedicated DNN for UE-to-Network Relay to communicate with its HPLMN’s PKMF.

Q2: Intermediate Relays and the UE-to-Network Relay belonging to the same PLMN in Model A and Model B discovery.

SA3 discussed a possible simplification of the Model A and Model B discovery with the assumption that the intermediate Relays and the UE-to-Network Relay belong to the same PLMN. This assumption is considered in particular to avoid the need for Remote UE to send or process multiple discovery messages trying different candidates PLMN security material. Can SA2 verify the assumption, i.e., whether it can be assumed for Model A and Model B discovery that the Intermediate Relays and the UE-to-Network Relay belong to the same PLMN?

A2: No. SA2 assumes that the Intermediate Relays and the UE-to-Network Relay can belong to different PLMNs.

2. Actions:

To SA3 group:

ACTION:  SA2 kindly asks SA3 to take the above answers into account in their future work.

3. Date of Next SA2 Meetings:

SA2#167                                    17th – 21st February 2025	 	Athens, GR

SA2#168                 		7th – 11th April 2025 			Gothenburg, Sweden