3GPP SA WG2 Meeting #166-AdHoc-e	S2-2500134

Electronic Meeting, 20-24 January 2025

# Title:	[Draft] LS on emergency call back and paging

# Response to:	-

# Release:	Release 19

# Work Item:	NR_newRAT-Core, NR_redcap-Core5G_CIoT, TEI19

Source:	SA2

To:	CT1

Cc:	RAN2, RAN3

Contact Person:

Name:	Haris Zisimopoulos

Tel. Number:

E-mail Address:	XXX AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA2 in recent discussions to draft alignment CRs for the “LP-WUS” feature in SA2#166 captured the following Editor’s Note in TS 23.501:

Editor's note:	The use of LP-WUSPS for emergency sessions and emergency call back is FFS.

In SA2#166AHE SA2 received LS R2-2411041 from RAN2 “LS on emergency services and eDRX” that is related also to how to handle emergency call back, in this case for UE in RRC_INACTIVE state with eDRX.

# Emergency call back though as defined in TS 23.167 cl. 4.1 and based on service requirements defined in TS 22.101 cl. 10.1.3 is treated as “normal call” based on “local regulation”.

Therefore SA2 assumes that:

- there is no support for emergency call back when the UE is in limited service;

- therefore the UE has to have active IMS PDU session and normal IMS registration to deliver the call;

- in case of roaming such IMS PDU session in most cases will terminate in HPLMN of the UE;

- there is no priority treatment e.g. special ARP, 5QI, RRC establishment cause that can allow the 5GS (CN and RAN) to distinguish between emergency call back and any other mobile terminated call.

Since rel.12 though when 3GPP introduced for first time extended DRX for idle mode in EPS, text was added in TS 23.401 for the UE to disable extended DRX when it has active bearers for emergency services. Similar text was added to TS 23.501 when extended DRX was introduced in 5GS. When “Paging Early Indication with Paging Subgrouping Assistance” (PEIPS) was introduced similar text was added and since the design of LP-WUS follows PEIPS the aforementioned Editor’s Note was added.

SA2 determined that whether it makes sense (adds any benefit) to disable features that may impact paging (eDRX, PEIPS, LP-WUS) will depend on whether there is possibility of sending Mobile Terminated (MT) signalling within the IMS PDU session for reasons other than emergency call back. For example it may be possible in certain cases where SIP timers expire and the UE to receive SIP signalling as part of the Mobile Originated (MO) emergency session after the RRC inactivity timer expires and the UE turns into CM-IDLE.

Therefore SA2 would like to checks with CT1 whether they foresee any scenarios for the UE to receive SIP signalling as part of the Mobile Originated (MO) emergency session after the RRC inactivity timer expires and the UE turns into CM-IDLE.

Depending on the answer to this question, SA2 will update accordingly its specifications and answer also the “LS on emergency services and eDRX” from RAN2.

2. Actions:

To CT1:

ACTION: 	SA2 asks CT1 whether they foresee any scenarios for the UE to receive Mobile Terminated (MT) SIP signalling as part of the Mobile Originated (MO) emergency session after the RRC inactivity timer expires and the UE turns into CM-IDLE.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #167		17-21 February 2025	Athens, Greece

TSG-SA2 Meeting #168		7-11 April 2025	Goteborg, Sweden