SA WG2 Meeting #166-AdHoc-e	S2-2500178

E-meeting, 20 – 24 January 2025	(revision of S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5206 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Energy Consumption information exposure procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some corrections are related to energy comsumption information procedure. 1. In section 4.29.1, the procedure of trusted AF subscribes the energy consumption information is missing. 2. According to section 5.2.26.2.1, the Volume Measurement is included in the UPF event User Data Usage Measures. And the Target of Event Reporting of User Data Usage Measures can be either a specific PDU Session or "any UE", not always for a specific UE. So, in order to support the Energy Consumption information collection for single UE, UE IP address may be used as assistance for measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add trusted AF procedure. Make clarification of how to collect data volume information for UE granularity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to collect the per UE level information is FFS. How to trusted AF request energy related information is FFS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.29.1, 4.29.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change* * *

4.29.1	Energy Consumption information exposure

The procedure is used by the AF to subscribe to notifications and to explicitly cancel a previous subscription for Energy Consumption information exposure.

Figure 4.29.1-1: Nnef_EventExposure_Subscribe, Unsubscribe and Notify operations for energy consumption information

1a.	The untrusted AF subscribes to or cancels subscription for energy consumption information via NEF by invoking the Nnef_EventExposure_Subscribe/ Nnef_EventExposure_Unsubscribe service operation. If the energy consumption information subscription is authorized by the NEF, the NEF proceeds with the steps below.

1b.	Based on the request from the untrusted AF, the NEF subscribes to or cancels subscription to energy consumption information by invoking the Neif_EventExposure_Subscribe/ Neif_EventExposure_Unsubscribe service operation of EIF.

1c.	The trusted AF subscribes to or cancels subscription for energy consumption information directly to EIF by invoking Neif_EventExposure_Subscribe/ Neif_EventExposure_Unsubscribe service operation.

2a.	The EIF acknowledges the subscription or removal of subscription for energy consumption information.

2b.	The NEF acknowledges the subscription or removal of subscription for energy consumption information from untrusted AF.

2c.	The EIF acknowledges the subscription or removal of subscription for energy consumption information from trusted AF.

3a.	The EIF notifies the NEF with the energy consumption information by invoking Neif_EventExposure_Notify service operation.

3b.	If the NEF receives the notification from the EIF, the NEF notifies the untrusted AF with the energy consumption information by invoking Nnef_EventExposure_Notify service operation.

6c.	The EIF notifies the trusted AF with the energy consumption information by invoking Neif_EventExposure_Notify service operation.

* * * 2nd of Change* * *

4.29.2	Energy Consumption information collection

Figure 4.29.2-1 shows the procedure for EIF to collect the Energy consumption information.

Figure 4.29.2-1: Energy Consumption information collection

1.	The Consumer NF (i.e. trusted AF or 5GC NF) subscribes to EIF of the energy consumption information using Neif_EventExposure_Subscribe service for the required granularities (UE, PDU Session and/or QoS flow) as described in clause 5.51.2.3 of TS 23.501 [2].

If the Consumer is untrusted AF, the subscription is sent via NEF. If the event subscription is authorized by the NEF, then the NEF sends the event subscription to the EIF via Neif_EventExposure_Subscribe.

2.	The EIF invokes Nudm_UECM_Get service operation to retrieve the SMF ID for the required granularity.

3.	The EIF invokes the Nsmf_EventExposure Subscription request to the SMF to request the data volume of the required granularity.

4.	The SMF selects the all of the UPFs that serving target UE and requests the UPFs to report the data volume of the required granularity.

For UE level energy exposure, the SMF provides UE ID (SUPI/GPSI) to UPF.

For PDU session level exposure, the SMF provides UE ID (SUPI/GPSI), DNN/S-NSSAI to UPF.

For QoS flow level exposure, the SMF provides UE ID (SUPI/GPSI), DNN/S-NSSAI and Flow description(s) to UPF.

NOTE X: For UE level energy exposure, the UE IP address may also be used by UPF for measurement of data volumn.

5.   The SMF provides UPF ID(s), RAN IDs and the data volume from the UPF to EIF by invoking Nsmf_EventExposure_Notify service operation for the required granularity.

NOTE:	The SMF can derive RAN IDs according to the AMF provided ULI as described in clause 4.3.2.

6. If there is no related Node-level energy consumption information and Node-level data volume, the EIF obtains the Node-level energy consumption information and Node-level data volume from OAM with providing RAN IDs and UPF ID(s).

7.	The EIF performs the energy consumption information calculation as described in clause 5.51.2.3 of TS 23.501 [2] and then reports the energy consumption information of the required granularity to the Consumer NF via Neif_EventExposure_Notify.

* * * End of Change * * *