3GPP TSG-WG2 Meeting #166 Adhoc	S2-2500193

# 20th – 24th January 2025, Online

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0639 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 – Addressing the leftovers in the WI Exception on whether and how LMF can take NG-RAN load into account when performing data collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During SA2#166, part of the context proposals in S2-2412350 aimed to address the evaluation of whether and how RAN load should be considered when the LMF performs data collection. However, these proposals were not successful. Currently, there still remains an EN regarding whether and how the LMF should account for the RAN load in the data collection procedure. "Network Performance Analytics" has been identified as one of the most suitable analytics for the LMF to subscribe to, enabling it to obtain gNB resource usage information within the AoI. It is essential to revisit and advance this proposal to resolve the EN. Although NG-RAN load primarily reflects the network-side resource usage ratio, it is difficult to fully eliminate the influence of UE(s) involved in the data collection process, as both the measurements and ground truth data originate from the UE(s). To this end, UE-related analytics subscriptions (NOT discussed yet), play a critical role in helping the LMF make more informed decisions regarding the timing and volume of data collection from the NG-RAN. Incorporating this aspect could provide a more detailed and comprehensive understanding of the traffic status and/or mobility patterns of UEs, including target UEs and/or PRUs, for tasks such as collecting ground truth data and AI model input measurement results. For example, including UE traffic status and mobility data could improve the timing of NG-RAN data collection and enhance the accuracy of ground truth datasets. Therefore, we propose two recommendations regarding the aforementioned EN: - The LMF should subscribe to "Network Performance Analytics" to obtain comprehensive cell resource usage data. - The LMF can enhance its analytics subscriptions by including "UE Communication" and, optionally, "UE Mobility". These additional subscriptions would provide richer contextual data, thereby improving the training and performance of AI/ML-based positioning models. |  |  |  |  |  |  |  |  |
| Summary of changes: |  | Update step 6&7&8 in accordance with Figure 6.22.3-1 by making LMF subscribe to “UE Communication” and optinally “UE Mobility” Analytics. Remove the EN on whether and how the LMF takes the RAN load into account in this procedure is FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The suitable timing and accuracy of NG-RAN data collection for AI/ML-based LMF positioning model training can not be assured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.13, 6.22.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.273 CR 0584, CR074 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 4.4.13	NL8 Reference Point

The NL8 reference point supports LMF to receive location related analytics or network performance analytics or UE communication analytics or UE mobility analytics from NWDAF as defined in TS 23.288 [37].

* * * Next Change * *

### 6.22.3	Data collection from NG-RAN for LMF-based AI/ML positioning

The procedure for data collection from NG-RAN for LMF-based AI/ML positioning is used to e.g. train the AI/ML Model.

Figure 6.22.3-1: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements

- The LMF determines that data collection is required e.g. to train an AI/ML Model for UE positioning for a number of UEs or to monitor the AI/ML Model performance. The LMF may also initiate the data collection upon the request of an NWDAF containing MTLF as described in step 3 in figure 6.22.4-1.

- The LMF may know the SUPI of these UEs, e.g. when training a AI/ML model using a PRU, which are associated to this LMF that executes data collection, the LMF may optionally invoke an Nnrf_NFDiscovery Request service operation to an NRF in order to identify the LMF(s) which has PRU associated to a list of TAIs. The LMF may send an Nlmf_Location_MeasurementData Request service operation to one or more of the other PRU serving LMFs in order to get the data collection and the known PRU location in this case step 9 follows.

If the LMF does not know the SUPIs of those UEs, the LMF try to get the list of SUPIs from the AMF, before that the LMF finds the AMFs via NRF using Nnrf_NFDiscoveryRequest.

- For each AMF serving a specific area. the LMF subscribes to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest".

- The AMF send Namf_EventExposure_Subscriber_Response or Namf_EventExposure_Notify ("list of SUPIs in/out the area of interest").

For each SUPI provided by the AMF that is in the area of interest then steps 5 to step 9 are performed.

- The LMF checks whether the SUPI provided user consent for data collection for the purpose of training with UDM. If user consent is provided, then step 6 follows. if user consent is not provided, the LMF does not perform step 9 for this UE, that will not be part of the data collection, and its data is not used for the LMF-based AI/ML Model positioning.

Editor's note:	Further details on user consent for data collection for model training for LMF-based AI/ML Model positioning will be aligned with SA WG3.

- The LMF may use Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request (Analytics ID="Network Performance", Target of Analytics Reporting “any UE”, Analytics Filter Information = "Area of Interest" and optionally Analytics Reporting Information = ("Reporting Thresholds" and Analytics target period(s))) to the NWDAF to request the predictions of the gNB status and gNB resource usage in the Area of Interest for data collection from NG-RAN. Additionally, the LMF may also subscribe to the NWDAF to request the predictions of UE communication status and optionally mobility patterns via Analytics ID = “UE Communication” and “UE Mobility”, Target of Analytics Reporting: a single UE (SUPI) or a group of UEs (an Internal Group ID), Analytics Filter Information = “Area of Interest” and optionally Analytics Reporting Information = (“Analytics target period(s)” and “Preferred level of accuracy of analytics”) for the data collection from the UE(s).

- The NWDAF responds to the LMF with predicted outputs that incorporate NG-RAN resource usage, UE communication, and optionlly mobility status. For "Network Performance" predictions, the response includes gNB status and resource usage information in the notification signaling. For "UE Communication" predictions, details on periodic timing, traffic volume, and applications are provided. For "UE Mobility" predictions, the response includes information about UE locations.

- The LMF checks the gNB status and gNB resource usage and UE communication and mobility status in the area of interest, then the LMF may decide to delay the request for data collection from NG-RAN during the current Analytics Target period if the gNB status indicates that gNBs in the area of interest are not available or the resource average usage indicates a high resource usage or the UE indicates in a high-traffic period or the UE is in low-confidence predictions, in these cases step 9 and 10 are skipped and no data is collected from NG-RAN or from the UE happens. At the time the Analytics target period elapses, step 6 may be executed by the LMF, if data collection from NG-RAN is needed.

- The LMF request input data for the target UE from NG-RAN.

- The LMF receives ground truth data from the UE.

The measurements and ground truth data are used for training. The UE location is derived from the measurements data by using LMF-based AI/ML Positioning. The derived UE location and ground truth data are used for performance monitoring.

The LMF may initiate data collection for multiple UEs simultaneously, as such step 8 and 9 may occur in parallel for a number of SUPIs as determined by the LMF.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

* * * End of Changes * * *