3GPP TSG-SA2 Meeting #166-AdHoc-e	S2-2500197

# E-meeting, 20 – 24 January 2025		(revision of S2-25xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0641 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of editor’s notes related to data collection and model retrieval |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following editor’s note related to data collection needs to be resolved: whether and how to limit the number of UEs in the UE list provided by AMF for data collection. TS 23.273, subclause 6.22.2: “Editor's note: Whether and how to limit the number of UEs is FFS.” The following editor’s notes in TS 23.273 related to how LMF retrieves ML model from NWDAF to perform AIML based positioning needs to be addressed: TS 23.273, subclause 5.18: “Editor’s Note: Whether and which additional parameters are considered when LMF requesting model positioning is FFS.” “Editor’s Note: whether multiple model identifiers can be provided is FFS.” TS 23.273, subclause 6.22.5: “Editor´s Note: Whether the LMF provides Inference Input Data information to NWDAF in the Nnwdaf_MLModelProvision service is FFS.” 1st EN: For cases where too many UEs are located in the AOI, the AMF delivers information of all UEs (e.g., UE ID, UE positioning capabilities, UE UP positioning capabilities) to the LMF, which may consume overloaded signalings (especially for invalid samples, e.g., UE not support LPP/LCS). Moreover, the number of UEs provided by the AMF may far exceed the needs of the LMF to train the model, which puts pressure on the LMF to store/buffer all received data as well. Thus, here proposes to limit the maximum number of UEs to be provided by AMF. 2nd EN: no additional parameters need to be considered when LMF is requesting models for positioning. Thus, here proposes to remove the EN. 3rd EN: Multiple ML models can be provided by MTLF to LMF when LMF supports multiple ML models for AIML positioning, just like MTLF provides multiple ML models to other consumers. Thus, here proposes to remove the EN and clarify it. 4th EN: It should be supported that LMF sends information about various settings that are expected to be used by LMF during inference, e.g., data source indicating LMF A, since LMF A intends to use a ML positioning model trained by data collected from LMF A itself. Thus, here proposes to remove the EN and clarify it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Resolve the editor’s note related to limitation of the number of UEs during the data collection; - Resolve several editor’s notes related to how LMF retrieves ML Models from NWDAF; - Some editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Overloaded signalings including UE information for data collection are transferred from AMF to LMF, which also puts pressure on LMF to store/buffer all received data. Several ENs related to how LMF retrieves ML Models from NWDAF are not addressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18, 6.22.2, 6.22.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The changes to subclause 5.18 in this paper is proposed based on the rapporteur’s paper. The missing parts in TS 23.273 version j10 subclause 5.18 are captured by the rapporteur paper in S2-2500194. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.18	Support for UE positioning based on a ML Model at the LMF

The LMF may calculate the UE location and estimate the achieved accuracy by using LMF-based AI/ML Positioning. When receiving the request for a UE location, the LMF selects an appropriate positioning method as described in clause 5.2 to determine the result of the positioning. The result of the positioning may be calculated by using LMF-based AI/ML Positioning ML model supported by LMF. The LMF collects input data from UE or NG-RAN for the LMF-based AI/ML Positioning to perform location calculation and provide the location to the consumer.

NOTE:	Whether to select LMF-based AI/ML Positioning for location result calculation is determined by LMF.

Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

The ML model that is used for LMF-based AI/ML Positioning in LMF may be trained by LMF. The trigger for data collection and model training in LMF is up to implementation. LMF collects input data from UE for ML model training is described in clause 6.22.2. The LMF collects input data from NG-RAN for ML model training is described in clause 6.22.3.

The LMF may also request a trained ML model for LMF-based AI/ML Positioning from NWDAF containing MTLF as described in clause 6.22.5. The LMF discovers a suitable NWDAF containing MTLF via NRF as described in clause 5.2 of TS 23.288 [37] with the following considerations:

- If the ML Model is for a specific positioning case, then the LMF may provide a request to retrieve an AI/ML Model for UE Positioning including the positioning case information, in addition:

- the LMF provides an Area of Interest, and may provide the ML Model Interoperability indicator to discover  an NWDAF that can provide an AI/ML Model that the LMF can run.

- The LMF may requests a NWDAF with ML Model accuracy checking capability to be able to report to LMF that the model is degraded (i.e. for ML model performance monitoring).

NOTE x: Other NWDAF discovery parameters listed in clause 5.2 of TS 23.288 [37], such as Analytics ID, FL capability and related time period, S-NSSAI or any roaming capabilities, are not required by LMF.

Editor´s Note:	The types of LMF-side AI/ML positioning model (e.g., per positioning case) is FFS, which needs to be coordinated with RAN WGs.

The LMF requests ML model for LMF-based AI/ML Positioning with ML Model Filter Information, which may indicate the area of interest. The LMF requests the NWDAF to provide an AI/ML Model as described in clause 6.2A.2 of TS 23.288 [37] with the following considerations:

-	The Notification Target Address to receive the ML Model, the positioning case information, and the Target of ML Model Reporting set to any UE.

-	If vendor specific information is needed to allow running the ML Model, then the ML Model Interoperability Information,

-	If the LMF supports multiple AI/ML Models, the support for multiple AI/ML Models, optionally with the maximum number of ML Models.

NOTE y:	When LMF supports multiple AI/ML Models, the NWDAF can provide LMF with multiple tuples of unique ML Model identifiers and the information as defined in clause 6.2A.2 of TS 23.288 [37].

The NWDAF containing MTLF collects input data to perform the ML model training as described in clause 6.22.4, and the NWDAF containing MTLF performs ML model provision to LMF as described in clause 6.2A of TS 23.288 [37] with the following considerations.

-	The NWDAF provides the ML Model identifier and ML Model Information for the ML Model for UE Positioning and indication of whether the ML Model identifier is updated if model is retrained and optionally the ML Model accuracy Information.

-	Some parameters related to the AI/ML Model to be trained, such as the requested representative ratio of the provided list of SUPIs or the ML Model Target Period and desired model metrics.

Editor's note:	The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs.

Once the ML model for LMF-based AI/ML Positioning is trained and located in the LMF, the LMF may use it to perform UE Positioning after receiving a location determination request from AMF.

Editor´s note:	Whether user consent for UE positioning calculation is needed apart from the existing GMLC check of the LCS privacy profile is FFS.

For training, performance monitoring and UE positioning calculation using LMF-based AI/ML Positioning, the LMF checks with UDM the user consent status before collecting UE related data, see clause 6.22.3 and clause 6.22.4.

Either LMF or NWDAF containing MTLF may perform performance monitoring for LMF-based AI/ML Positioning. When the ML model that is used for LMF-based AI/ML Positioning is trained by LMF, then the LMF monitors the performance of the ML model. When the ML model that is used for LMF-based AI/ML Positioning in LMF is trained by NWDAF containing MTLF, then the NWDAF containing MTLF monitors the performance of the ML model. LMF may determine whether to use the LMF-based AI/ML Positioning to perform location calculation based on the model performance monitoring result. LMF may also trigger the ML model retraining in the training entity based on the result of model performance result.

# * * * * Next change * * * *

6.22.2	Data Collection at LMF for the LMF-based AI/ML positioning based on UE measurements

The LMF needs to obtain input data for LMF-based AI/ML Positioning model training, performance monitoring and may also request the UE to provide the UE location. The LMF-based AI/ML Positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

Figure 6.22.2-1: Data collection at LMF for the LMF-based AI/ML positioning based on UE measurements

1.	The LMF starts data collection for the purpose to train an AI/ML Model for UE Positioning (this may be based on an internal trigger in the LMF or a request from the NWDAF). The LMF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover AMF(s) that served for the area of interest.

Editor's note:	Whether the LMF will subscribe at NWDAF for Network Performance Analytics to decide when to trigger data collection is FFS.

2.	The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest and the UE Positioning Capability for each UE using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest", "UE Positioning Capability", and, optionally, the maximum number of UEs to indicate the maximum number of SUPIs provided by AMF).

3.	The AMF send Namf_EventExposure_Subscriber_Response ("list of SUPIs in the area of interest"). If "UE Positioning Capability" is also requested in step 2, AMF includes UE Positing Capability, UE User Plane Positioning Capabilities if available for each SUPI in the response message sent to LMF. The AMF may consider the maximum number of UEs included in step 2 to determine the list of SUPIs in the response message sent to LMF.

4.	The LMF further determines the UEs from the list of SUPIs that received from AMF in step 3 for data collection based on UE Positioning Capability, UE User Plane Positioning Capabilities, UE consent check result, the PRU information available in the LMF and operator's policy. The User consent check is not needed for data collection from PRU for performance monitoring.

Editor's note:	Further details on user consent for data collection for model training or model performance monitoring for LMF-based AI/ML Model positioning will be aligned with SA WG3.

5.	For each UE that provides consent to data collection for model training and can support data collection the LMF initiates a request for input data. The UE may reject the data collection request from the LMF (e.g. considering UE status, user's input). If the UE accepts data collection request, the UE may cancel the data collection later as defined in clause 6.3.4.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the UE is FFS. The procedure will be coordinated with RAN WGs.

Editor's note:	The input data used for AI/ML based positioning will be decided by RAN WGs.

At the time the LMF decides that the input data for a UE are not needed, e.g. the user consent for data collection for model training is revoked, the LMF stops to collect the input data from the UE.

# * * * * Next change * * * *

6.22.5	LMF retrieval of a trained AI/ML Model for positioning

The LMF may retrieve a trained AI/ML Model from NWDAF. The LMF obtains a trained AI/ML model prior to a UE positioning request that requires a trained ML Model.

Figure 6.22.3-1: The LMF retrieves a trained AI/ML based positioning model from NWDAF

1.	The LMF determines using local policies, that a positioning method requires a trained ML Model, the trained ML Model is not trained locally but is retrieved from NWDAF.

2.	The LMF discovers the NWDAF that can provide a trained model, using Nnrf_NFdiscovery including a request for AI/ML Model for UE positioning, ML Model Filter Information that includes the positioning case information and an Area(s) of Interest, the ML Model Interoperability indicator applicable to the trained ML Model for UE Positioning, optionally the ML Model Accuracy checking capability.

NOTE:	NWDAF discovery and selection is described in clause 5.2 of TS 23.288 [37], the applicable NWDAF parameters used for discovery of an NWDAF supporting an AI/ML Model for UE Positioning are described in clause 5.18.

3.	The LMF receives a list of candidates NWDAF instances, validity period for the discovery, per NWDAF instance the NWDAF instance ID, FQDN or IP address, and the ML Model Interoperability Indication.

4.	The LMF selects a NWDAF out of the list of candidates NWDAF instances, that can provide a ML Model that supports the same ML Model Interoperability Indication then request a trained AI/ML Model using Nnwdaf_MLModelInfo_Request or Nnwdaf_MLModelProvision_Subscribe including the request for a trained AI/ML Model for UE Positioning according to the information listed in clause 5.18.

NOTE x:	The LMF can provide Inference Input Data information (as defined in clause 5.2 of TS 23.288 [37]) to NWDAF in the Nnwdaf_MLModelProvision service.

5.	The NWDAF provides one or more AI/ML Models to LMF including a ML Model Identifier and the ML Model Information as described in clause 5.18.

6.	Same as in step 5.

The LMF updates the received trained ML Model(s) when the NWDAF provides new ones.

# * * * * End of changes * * * *