3GPP TSG-SA WG2 Meeting #166-AH-e	S2-2500222

Online, 20-24 January 2025

Agenda item:	19.49

Source:	Nokia

Title:	Discussion on Incoming LS S2-2500053/R3-247910 on FS_VMR_Ph2 solution impacts to RAN (MWAB mobility)

WI/Release:	VMR_Ph2/Rel-19

Document for:	Discussion and Decision

# 1	Discussion

RAN3 has provided a reply LS on FS_VMR_Ph2 solution impacts to RAN (MWAB mobility) in the following document:

| R3-247910 | Reply LS on FS_VMR_Ph2 solution impacts to RAN (MWAB mobility) |
| --- | --- |

In this document RAN3 confirmed SA2’s assumptions in the following agreement:

“The ‘two logical gNB solution’ can support UE’s AMF change during WAB-gNB mobility.”

This solution has no CN impact and so should be preferable from SA2 standpoint.

Proposal 1: the two logical gNBs solution is supported as baseline behaviour for MWABs when AMF change is required.

Additionally, the LS provides indication that RAN3 has studied two more alternatives to support a one gNB approach and asked SA2 to provide feedback on whether either of these single-gNB solutions can be achieved without CN impact.

Nokia has analyzed Option 1 and for sure this has CN impact as it is not supported today to lookup the ULI in a UL NAS transport message and based on that trigger an AMF relocation.

When analyzing Option2, this is a variant of the two gNB solutions with a N2 Handover that has in the Target ID the same Global RAN Node ID as the source node, albeit a different Selected TAC. See clause 9.3.1.25 of TS 38.413:

Figure 1 – the structure of the Target ID.

This seems in principle compatible with an AMF behavior if the AMF uses the TARGET ID IE just for routing, but the only possible obstacle to adoption seems whether the AMFs will compare the source gNB ID and target gNB IDs in the TARGET ID IE and upon detecting them being equal would trigger some error condition. This behavior would not be a standard one but cannot be rule out.

Proposal 2: The one logical gNBs Solution 1 is not supported by CN today and since it has CN impact it should not be further considered.

Proposal 3: The one logical gNBs Solution 2 is similar from CN standpoint to the two gNB solution. There can be a non quantifiable risk that some AMF could trigger an error condition if the target gNB's Global RAN Node ID is the same as the source gNB's. SA2 should not recommend this solution but it could be used in the field if conditions allow (i.e. testing confirms it causes no issue in the PLMN/SNPN).

# 2	Conclusions

It is proposed that the Proposal above are agreed and form the basis of a reply LS.