SA WG2 Meeting #166-AdHoc-E	S2-2500245

January 20 – 24, 2025, Electronic

Source:	Ericsson, Apple, Google

Title:	Support for CONNECT-UDP with MPQUIC-only option

Document for:	Approval

Agenda Item:	19.13.2

Work Item / Release:	MASSS / Rel-19

Abstract of the contribution: This contribution discusses how to enable MPQUIC-UDP for a UE supporting “MPQUIC-IP without ATSSS-LL”.

# Background

At the last meeting, it was agreed that the UE can indicate one or more of the following capabilities:

A.	ATSSS-LL functionality with any steering mode.

B.	MPTCP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode.

C.	MPQUIC-UDP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode.

D.	MPQUIC-IP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode.

E.	MPQUIC-E functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode.

F.	MPQUIC-IP functionality with any steering mode without any ATSSS-LL functionality.

In summary, MPTCP, MPQUIC-UDP, MPQUIC-IP and MPQUIC-E are all bundled with minimal ATSSS-LL active-standby and minimal PMF support (access availability/unavailability indications). In addition, there is an option for a UE to indicate support for MPQUIC-IP without any ATSSS-LL support.

# Discussion

MPQUIC-UDP is quite similar to MPQUIC-IP in the sense that both use the MP version of QUIC. A few aspects to consider when comparing MPQUIC-UDP and MPQUIC-IP are:

- 	Both MPQUIC-IP and MPQUIC-UDP use HTTP CONNECT methods to establish the MPQUIC proxying for an SDF (HTTP CONNECT-UDP and HTTP CONNECT-IP)

- 	CONNECT-UDP can be considered an optimization of CONNECT-IP for UDP traffic. When using CONNECT-UDP, only the payload of the UDP frames is carried in the QUIC connection while when using CONNECT-IP the full IP packet is carried in the QUIC connection. MPQUIC-UDP thus has less overhead than MPQUIC-IP. MPQUIC-IP can always serve as a fallback since MPQUIC-IP can carry UDP traffic.

- 	CONNECT-UDP is overall simpler than CONNECT-IP and was specified by IETF before CONNECT-IP. I.e., the effort to add CONNECT-UDP support to an ATSSS implementation supporting CONNECT-IP is estimated to be small. It is therefore not unlikely that a UE implementing CONNECT-IP will also support CONNECT-UDP.

- 	CONNECT-UDP and CONNECT-IP (and multiple of each) can be used simultaneously in the same QUIC connection, i.e. no need for separate QUIC connections for MPQUIC-IP and MPQUIC-UDP.

Observation 1: MPQUIC-UDP can be considered an optimization of MPQUIC-IP (for UDP traffic) and it is not unlikely that an ATSSS implementation supporting CONNECT-IP will also support CONNECT-UDP.

A UE indicating support for capabilities C + D is able to support MPQUIC-UDP and MPQUIC-IP for a MA PDU Session, but only bundled with ATSSS-LL active-standby. However, a UE indicating only MPQUIC-IP (capability F) will not be able to use MPQUIC-UDP, even if supported.

Observation 2: A UE supporting "MPQUIC-IP without ATSSS-LL" is not able to use MPQUIC-UDP even if supported, which seems to be an oversight from the last meeting’s agreement and is also an unnecessary and artificial limitation.

At last meeting it was discussed to allow an option for “MPQUIC-UDP without ATSSS-LL active-standby”, but this was not agreed as it was preferred by several companies to re-use the existing rel-18 approach where MPQUIC-UDP is bundled with ATSSS active-standby. It was also commented that a standalone MPQUIC-UDP option would be tricky to handle since it could not serve all traffic, and this would require it to be used together with some other capability. However, something that was not discussed was whether to bundle MPQUIC-UDP with MPQUIC-IP. With the support of capability “F”, such bundling would make sense to allow also such UEs to be able to use MPQUIC-UDP, if supported. That would allow a self-contained capability where any traffic can be handled.

Observation 3: For a UE that supports "MPQUIC-IP without ATSSS-LL", it would make sense to allow also MPQUIC-UDP to be used, if supported.

Building on the agreement from last meeting, there are different ways MPQUIC-IP and MPQUIC-UDP can be bundled:

- 	Alt X: Modify existing capability “F” to support both MPQUIC-IP and MPQUIC-UDP.

o 	This option thus assumes that a UE indicating “F” supports both MPQUIC-IP and MPQUIC-UDP.

- 	Alt Y: Modify existing capability “F” to support MPQUIC-IP and optionally MPQUIC-UDP. There are different variants for how this can be done:

o 	Alt Y1: Fallback to MPQUIC-IP if the UE does not support MPQUIC-UDP:

In this case, if the DNN configuration allows MPQUIC-UDP, the network creates PCC rules and ATSSS rules as if both MPQUIC-UDP and MPQUIC-IP are supported by the UE. If the ATSSS rule indicates MPQUIC-UDP, then MPQUIC-UDP is used by the UE if supported, otherwise the UE uses MPQUIC-IP for that SDF. If the ATSSS rule indicates MPQUIC-IP, then MPQUIC-IP shall be used. No new capability indications are needed between UE and network. If the N4 rules indicate MPQUIC-UDP, the UPF accepts either CONNECT-UDP or CONNECT-IP for that SDF.

o 	Alt Y2: Network indicates per PDU Session whether MPQUIC-UDP is allowed:

In this case the network creates PCC rules and ATSSS rules with only MPQUIC-IP. However, if the DNN configuration allows MPQUIC-UDP, the SMF indicates in the ATSSS information in the PDU Session Establishment Accept that the UE can use MPQUIC-UDP for UDP flows of this MA PDU Session, even if the ATSSS rule says MPQUIC-IP. It is then up to the UE to use either CONNECT-UDP or CONNECT-IP for a UDP flow, if supported. The SMF requests the UPF to provide proxy address supporting MPQUIC-UDP and MPQUIC-IP. In this case the UPF accepts CONNECT-UDP for an SDF even if the N4 rule says MPQUIC-IP.

o 	Alt Y3: UE can try with MPQUIC-UDP, if supported:

In this case the network creates PCC rules and ATSSS rules with only MPQUIC-IP. If supported and allowed by the DNN configuration, the SMF request UPF to provide proxy address supporting MPQUIC-UDP and MPQUIC-IP. The UE can try to use MPQUIC-UDP for a UDP flow, without any specific indication from the network. If the UE sends a CONNECT-UDP and the network supports/allows it, it is accepted by the UPF. If the network does not support/allow it, the UPF sends a HTTP reject based on existing RFCs (e.g. a 501 Not Implemented) and the UE should then use MPQUIC-IP instead. No new capability indications needed between UE and network. In that case the UPF accepts CONNECT-UDP for an SDF even if the N4 rule says MPQUIC-IP.

o 	NOTE: In all cases Y1-Y3, the same proxy information (UPF proxy IP address) and proxy connection is used for both CONNECT-UDP and CONNECT-IP.

- 	Alt Z: Clarify the meaning of providing C + F (i.e. the case where UE provides both “MPQUIC-UDP with ATSSS-LL active-standby” and “MPQUIC-IP without ATSSS-LL”):

o	Another option is to describe in the standards how to interpret a case where the UE provides capabilities C and F. The current standard does not prevent a UE from providing capabilities C + F, but in this case, it is ambiguous whether the UE supports “ATSSS-LL with active-standby” or not. It can be noted that there is an existing non-ambiguous option for a UE supporting “MPQUIC-IP, MPQUIC-UDP with ATSSS-LL active-standby” to indicate capability C + D. To enable a combination with “MPQUIC-IP, MPQUIC-UDP without ATSSS-LL”, it could therefore be possible to utilize the combination C + F and make it clear in the standards that such combination is representing “MPQUIC-IP, MPQUIC-UDP without ATSSS-LL”.

- 	Alt W: Add a new capability “G” for “MPQUIC-IP and MPQUIC-UDP functionality with any steering mode without any ATSSS-LL functionality”.

o 	With this option, the UE can indicate explicitly that it supports “MPQUIC-IP functionality with any steering mode and MPQUIC-UDP functionality with any steering mode without any ATSSS-LL functionality”

The different options are compared in Table 1 below.

From the author’s point of view, all options described above are acceptable. However, Alt Y1 seems to be the preferred option due to the following reasons (see also Table 1):

- 	MPQUIC-UDP is optional in the UE if the UE indicates “MPQUIC-IP without ATSSS-LL”

- 	It does not require new capability indications between the UE and the network

- 	It enables the operator to control per SDF whether MPQUIC-UDP is allowed

Observation 4. From the author’s point of view, all options described above are acceptable. However, Alt Y1 seems to be the preferred option due to simplicity and capability.

A comparison of alternatives follows where “F” and “C” are the capabilities described earlier in this DP.

Table 1. Comparison of alternatives

|  | Pros | Cons | Specification impacts |
| --- | --- | --- | --- |
| X | - It does not require new capability indications between the UE and the network - It enables the operator to control per SDF whether MPQUIC-UDP is allowed | - Capability “F” is redefined as (or assumes) both MPQUIC-IP and MPQUIC-UDP support, and there is no option with MPQUIC-IP only. | Update existing capability (“F”) |
| Y1 | - MPQUIC-UDP is optional in the UE if the UE indicates “MPQUIC-IP without ATSSS-LL”. - It does not require new capability indications between the UE and the network - It enables the operator to control per SDF whether MPQUIC-UDP is allowed | - Not possible to know at MA PDU Session Establishment whether MPQUIC-UDP is supported by the UE | Clarify UE and network behaviour for use of MPQUIC-UDP in case capability “F” is provided |
| Y2 | - MPQUIC-UDP is optional in the UE if the UE indicates “MPQUIC-IP without ATSSS-LL”. | - New indication to UE whether MPQUIC-UDP is allowed - Does not allow per-SDF control on whether MPQUIC-UDP is used - Not possible to know at MA PDU Session Establishment whether MPQUIC-UDP is supported by the UE | Clarify UE and network behaviour for use of MPQUIC-UDP in case capability “F” is provided. New indication from SMF to UPF in PDU Session Establishment Accept. |
| Y3 | - MPQUIC-UDP is optional in the UE if the UE indicates “MPQUIC-IP without ATSSS-LL” - It does not require new capability indications between the UE and the network | - Trial-and-error approach may increase the traffic between UE and UPF - Does not allow per-SDF control on whether MPQUIC-UDP is used - Not possible to know at MA PDU Session Establishment whether MPQUIC-UDP is supported by the UE | Clarify UE and network behaviour for use of MPQUIC-UDP in case capability “F” is provided. |
| Z | - MPQUIC-UDP is optional in the UE if the UE indicates “MPQUIC-IP without ATSSS-LL”. - It does not require new capability indications between the UE and the network - It enables the operator to control per SDF whether MPQUIC-UDP is allowed | - May be a bit non-intuitive due to combination of seemingly conflicting capabilities | Clarify the meaning of a UE providing capability C+F |
| W | - MPQUIC-UDP is optional in the UE if the UE indicates “MPQUIC-IP without ATSSS-LL”. | - New capability needed | Define new capability provided by the UE |

# Proposal

It is proposed to discuss the alternatives described above and agree on a way forward. Alt Y1 is captured in CRs in S2-2500246.