3GPP TSG-SA2 Meeting #166-Ad Hoc-e	S2-2500313

# Online, , 20th Jan 2025 - 24th Jan 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0528 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on multihop ProSe U2N Relay with model B discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For multihop ProSe U2N Relay with model B discovery, IP address assignment and relay reselection procedure were updated at the last meeting. In TS23.304, for IP address assignment, it is stated that IM Relay builds their local IP routing table based on Remote UE’s info but multihop path information between Remote UE and U2N relay also need to be considered together. It shall be clarified. In TS23.304, for relay reselection procedure, it is stated that Remote UE may perform U2N relay discovery to discover available multihop path to U2N relay but there is no description about the Remote UE’s action after discovery procedure. It shall be clarified that Remote UE sends DCR or LMR with newly discovered multihop path information as relay reselection procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification on IP address assignment and relay reselection procedure are added for multihop U2N relay with model B discovery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multihop U2N relay with model B will not work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.9.2, 6.4.3.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of 1st Changes * * *

##### 6.4.3.9.2	IP address/prefix management for 5G ProSe Layer-3 Multi-hop UE-to-Network Relay

When IP based PDU session types are used by the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay acts as an IP Router.

The IP address allocation is performed by the 5G ProSe UE-to-Network Relay, as defined in clause 6.5.1.

DHCP based IP address configuration and IPv6 routing advertisement is performed as described in clause 6.4.3.8.2.

After IP address assignment, the Remote UE may initiate a Link Modification Procedure which includes the Remote UE ID and the Remote UE information. The 5G ProSe Intermediate UE-to-Network Relays builds their local IP routing table based on the Remote UE info and the received multihop path info for UE-to-Network Relay during link establishment procedure for unicast PC5 connection (step 4 in clause 6.5.1.4 or step 3 in clause 6.5.1.5).

* * * Start of 2nd Changes * * *

##### 6.4.3.9.3	Relay reselection

After being connected to the 5G ProSe UE-to-Network Relay, the 5G ProSe Remote UE and 5G ProSe Intermediate UE-to-Network Relay keep performing the measurement of the signal strength of PC5 unicast link with its parent Relay (Intermediate Relay or U2N relay) for relay reselection.

When a 5G ProSe Intermediate UE-to-Network Relay detects the link failure or link quality degradation, the 5G ProSe Intermediate UE-to-Network Relay informs the 5G ProSe Remote UE of the link failure or link quality degradation of the PC5 link so that the 5G ProSe Remote UE may initiate multihop UE-to-Network Relay reselection procedure.

The 5G ProSe Remote UE may perform the 5G ProSe UE-to-Network Relay discovery procedures defined in clause 6.3.2.3 to discover available 5G ProSe UE-to-Network Relays or 5G ProSe multi-hop UE-to-Network Relay Discovery procedures defined in clause 6.3.2.5.3 to discover available multihop paths to the 5G ProSe UE-to-Network Relays for 5G ProSe (multihop) UE-to-Network Relay reselection and select a new path for 5G ProSe (multi-hop) UE-to-Network relay. And the 5G ProSe Remote UE may initiate link establishment procedure for unicast PC5 connection by sending a DCR or a LMR with the newly selected path info to the next hop 5G ProSe Intermediate UE-to-Network Relay at the newly selected path.

After the 5G ProSe multihop UE-to-Network relay reselection procedure, if the same 5G ProSe UE-to-Network Relay is used for connection with a 5G ProSe Remote UE, the existing IP address of the 5G ProSe Remote UE is reused. The IP address of the Remote UE may be included by the 5G ProSe UE-to-Network Relay and Intermediate UE-to-Network Relays in the DCA/LMA message.

After a preconfigured duration, the Relay that detected the need for reselection releases the link with the previous and next relays if applicable, unless it is reselected by the Remote UE. This will trigger them to perform the same procedure until the complete path has been released. Then, the 5G ProSe UE-to-Network Relay removes its association with the 5G ProSe Remote UE including the allocated IP address.

* * * End of Changes * * *