3GPP TSG-SA WG2 Meeting #166-AdHoc-e	S2-2500327

# E-meeting, 20 – 24 January 2025						                           (revision of S2-25xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0534 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on multi-hop U2N Relay architecture aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the incoming LS (S2-2500022/S3-245259), SA3 asks SA2 to verify some architecture assumptions to fulfill the SA3 requirements. For the assumption that an Intermediate Relay can use the dedicated DNN for UE-to-Network Relay to communicate with its HPLMN’s PKMF, it can be verified as long as the same DNN (included in the PDU Session parameters) can be configured in the Intermediate UE-to-Network Relay and the UE-to-Network Relay. For the assumption that for Model A and Model B discovery that the Intermediate Relays and the UE-to-Network Relay belong to the same PLMN, there is no restriction on Intermediate UE-to-Network Relay and UE-to-Network Relay belonging to the different PLMNs from SA2 architecture point of view, however, to fulfil the SA3 requirements SA2 can make the assumption that the Intermediate UE-to-Network Relay and the UE-to-Network Relay belong to the same PLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the same provisioning PDU Session parameters (including DNN) as UE-to-Network Relay to Intermediate UE-to-Network Relay. Clarify the assumption that the Intermediate UE-to-Network Relay and the UE-to-Network Relay belong to the same PLMN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SA3 requirements can not be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

4.2.7.1A	5G ProSe Multi-hop Layer-3 UE-to-Network Relay reference architecture

Figure 4.2.7.1A-1 shows the high level reference architecture for 5G ProSe multi-hop Layer-3 UE-to-Network Relay. There may be one or multiple 5G ProSe Intermediate UE-to-Network Relays between the 5G Prose Remote UE and the 5G ProSe UE-to-Network Relay. In this figure, the 5G ProSe Layer-3 UE-to-Network Relay may be in the HPLMN or a VPLMN. The 5G ProSe Intermediate UE-to-Network Relay and the 5G ProSe UE-to-Network Relay have subscriptions from the same PLMN. The 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay may have subscriptions from the same PLMN or different PLMNs.

Figure 4.2.7.1A-1: Reference architecture for 5G ProSe multi-hop Layer-3 UE-to-Network Relay

* * * * 2nd Change * * * *

5.1.4.1a	Policy/Parameter provisioning for 5G ProSe multi-hop UE-to-Network Relay

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay when "served by NG-RAN":

-	PLMNs in which the UE is authorized to relay traffic for 5G ProSe Layer-3 Remote UEs supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

The authorisation for a UE to act as a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay also authorizes the use of 5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery with Model A and Model B.

NOTE 1:	It is up to UE and application implementation to select a discovery model or whether to perform both models simultaneously.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-Network Relay Layer Indicator per RSC, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-Network Relay Layer Indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. RSC dedicated for emergency service may also be provisioned. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe multi-hop UE-to-Network Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

NOTE 2:	The RSC used for multi-hop service is different from the RSC used for single-hop service.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement and Relay Discovery Additional Information messages and receiving Relay Discovery Solicitation messages;

-	For 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay, the PDU Session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) to be used for the relayed traffic for each ProSe Relay Service Code;

NOTE 3:	5G ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the 5G ProSe UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

3)	For 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay to relay Ethernet or Unstructured traffic from 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay by using IP type PDU Session:

-	Mapping of ProSe Service(s) to ProSe Application Server address information (consisting of IP address/FQDN and transport layer port number).

4)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 multi-hop UE-to-Network Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay and thereby enabling the use of a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

1)	Authorisation policy for using a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

-	For 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay, indicates whether the UE is authorised to use a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

The authorisation for a UE to act as a 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay also authorizes the use of 5G ProSe Layer-3 multi-hop UE-to-Network Relay discovery with Model A and Model B.

NOTE 4:	It is up to UE and application implementation to select a discovery model or whether to perform both models simultaneously.

2)	Policy/parameters for 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

-	Includes the parameters for 5G ProSe Relay Discovery and for enabling the UE to connect to the 5G ProSe UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay after discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-Network Relay Layer indicator per RSC, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-Network Relay Layer Indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. RSC(s) dedicated for emergency service may be provisioned to enable the support of emergency services for multi-hop UE-to-Network Relaying. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe multi-hop UE-to-Network Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

NOTE 5:	The RSC used for multi-hop service is different from the RSC used for single-hop service.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Solicitation messages and receiving Relay Discovery Announcement and Relay Discovery Additional Information messages;

-	For 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay, the PDU Session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) to be used for the relayed traffic without using N3IWF access, or an indication of N3IWF access, for each ProSe Relay Service Code;

-	For 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay, optionally the ProSe application Traffic Descriptor(s) (as defined in TS 23.503 [9]) to be used for the relayed traffic for each ProSe Relay Service Code;

3)	Policy/parameters for N3IWF selection for 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

-	N3IWF identifier configuration for 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay (either FQDN or IP address) in the HPLMN.

-	5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay access node selection information - a prioritized list of PLMNs for N3IWF selection. It also indicates if selection of an N3IWF in a PLMN should be based on Tracking Area Identity FQDN or on Operator Identifier FQDN.

NOTE 6:	5G ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

4)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 multi-hop UE-to-Network Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 Intermediate UE-to-Network Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 Intermediate UE-to-Network Relay:

-	For 5G ProSe Layer-3 Intermediate UE-to-Network Relay, indicates whether the UE is authorised to act as a 5G ProSe Layer-3 Intermediate UE-to-Network Relay.

The authorisation for a UE to act as a 5G ProSe Layer-3 Intermediate UE-to-Network Relay also authorizes the use of 5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery with Model A and Model B.

NOTE 7:	It is up to UE and application implementation to select a discovery model or whether to perform both models simultaneously.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe Layer-3 Intermediate UE-to-Network Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-Network Relay Layer Indicator per RSC, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-Network Relay Layer Indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. RSC dedicated for emergency service may also be provisioned. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe multi-hop UE-to-Network Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

NOTE 8:	The RSC used for multi-hop service is different from the RSC used for single-hop service.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement and Relay Discovery Additional Information messages and receiving Relay Discovery Solicitation messages;

-	For 5G ProSe Layer-3 Intermediate UE-to-Network Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

-	For 5G ProSe Layer-3 Intermediate UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay, the PDU Session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) to be used for the relayed traffic for each ProSe Relay Service Code;

NOTE 9:	5G ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the 5G ProSe Layer-3 Intermediate UE-to-Network Relay.

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 Intermediate UE-to-Network Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay, in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay and thereby enabling the use of a 5G ProSe Layer-3 UE-to-Network Relay and in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 Intermediate UE-to-Network Relay:

1)	Radio parameters for 5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

-	Default PC5 DRX configuration (see TS 38.331 [16]).

2)	Radio parameters for 5G ProSe Layer-3 multi-hop UE-to-Network Relay communication when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Communication over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 10:	The validity time of these radio parameters is the same as the validity time of the Policy/Parameter listed above for 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay or 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay or 5G ProSe Layer-3 Intermediate UE-to-Network Relay.

3)	Policy/parameters related to privacy:

-	A privacy timer value indicating the duration after which the UE shall change each source Layer-2 ID self-assigned by the UE when privacy is required.

* * * * 3rd Change * * * *

6.3.2.5.2	Procedure for 5G ProSe multi-hop UE-to-Network Relay Discovery with Model A

Figure 6.3.2.5.2-1 presents the procedures for 5G ProSe multi-hop UE-to-Network Relay Discovery using Model A discovery.

Figure 6.3.2.5.2-1: 5G ProSe multi-hop UE-to-Network Relay discovery procedure with Model A

0.	The 5G ProSe Remote UE, the 5G ProSe Intermediate UE-to-Network Relay and the 5G ProSe UE-to-Network Relay are (pre-)configured or provisioned with the related information to support multi-hop UE-to-Network Relay discovery, as specified in clause 5.1.4.

1.	If a 5G ProSe UE-to-Network Relay is successfully registered and authorized by the 5GS and has the configurations to operate multi-hop UE-to-Network Relay in the current region, it can start the discovery procedure. Otherwise, it shall not perform the 5G ProSe multi-hop UE-to-Network Relay discovery, i.e. not sending any announcement message.

2.	The 5G ProSe UE-to-Network Relay sends a 5G ProSe UE-to-Network Relay Discovery Announcement message. To support multi-hop UE-to-Network Relay discovery, the 5G ProSe UE-to-Network Relay sets the Hop-Count to 1, sets the Announcer Info to its own User Info ID and sets the RSC according to the configuration as in clause 5.1.4.

3.	The 5G ProSe Intermediate UE-to-Network Relay creates a corresponding discovery entry based on the received 5G ProSe UE-to-Network Relay Discovery Announcement message, if the discovery message meets the AS layer criteria and the Hop-Count is less than the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message.

The discovery entry includes the following information:	RSC, (optional) Root Relay Info, Announcer Info, Hop-Count, (optional) Accumulated QoS for PC5 link, (optional) NCGI, (optional) RRC Container.

The Announcer Info is set to the Announcer Info of the received message. The Root Relay Info is set to the Root Relay Info of the reived message. The Hop-count is set to the value of the received message. The Accumulated QoS for PC5 link, if present in the received discovery message, will be updated based on the QoS of the PC5 link between the 5G ProSe Intermediate UE-to-Network Relay and the sender of the message.

If an discovery entry already exist, i.e. with the same RSC and (optional) Root Relay Info and if the Hop-Count of the received message is less than the Hop-Count stored in the discovery entry, the 5G ProSe Intermediate UE-to-Network Relay updates the discovery entry accordingly. The Announcer Info and Hop-Count are set to the Announcer Info and Hop-Count of the newly received discovery message.

NOTE:	Based on implementation, a 5G ProSe Intermediate UE-to-Network Relay may decide to update the discovery entry based on the Accumulated QoS for PC5 link instead of the Hop-Count, or a combination of the two information elements.

4a.	5G ProSe Intermediate UE-to-Network Relay sends a 5G ProSe UE-to-Network Relay Discovery Announcement message, using the information from the stored discovery entry. The Hop-Count value will be incremented by 1 and the Announcer Info is set to the User Info of the 5G ProSe Intermediate UE-to-Network Relay.

When received by the 5G ProSe Remote UE, the information in the 5G ProSe UE-to-Network Relay Discovery Announcement message will be used for relay (re)selection, if the Hop-Count is less or equal to the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message. The 5G ProSe Remote UE can also take the Accumulated QoS for PC5 link into account. For relay reselection, the Root Relay Info may be used to prioritize a relay offering the connection to the same 5G ProSe UE-to-Network Relay.

4b.	5G ProSe Intermediate UE-to-Network Relay sends a 5G ProSe UE-to-Network Relay Discovery Announcement message, using the information from the stored discovery entry. This message may reach another 5G ProSe Intermediate UE-to-Network Relay.

5.	The 5G ProSe Intermediate UE-to-Network Relay drops the received message if the Hop-Count is not less than the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message.

6.	The 5G ProSe Intermediate UE-to-Network Relay sets a locally configured timer for each of the discovery entry based on implementation. If the 5G ProSe Intermediate UE-to-Network Relay does not have an active PC5 link with the relay identified by the Announcer Info and it does not receive any new announcement message from that relay, the 5G ProSe Intermediate UE-to-Network Relay removes the corresponding discovery entry and stops sending related 5G ProSe UE-to-Network Relay Discovery Announcement message.

Based on configuration, the 5G ProSe Intermediate UE-to-Network Relay may need to establish a Layer-2 link with the relay identified by the Announcer Info in the discovery entry before sending the 5G ProSe UE-to-Network Relay Discovery Announcement message.

* * * * End of Changes * * * *