3GPP TSG-WG2 Meeting #166-Ad-Hoc-e	S2-2500337

# 20 - 24 January, 2025, Electronic meeting				                             (revision of )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1336 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the trusted AF as the server case in the VFL preparation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Resolve the following EN by adding the trusted AF as the server case together with the NWDAF as the server case in the VFL preparation procedure. Editor's note: Whether the case of trusted AF as VFL Server is included in the clause for NWDAF as VFL server or in the clause for untrusted AF as VFL server is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the trusted as the VFL server case together with the NWDAF as the server case in the VFL preparation procedure. Remove the following ENs: Editor's note: Whether the case of trusted AF as VFL Server is included in the clause for NWDAF as VFL server or in the clause for untrusted AF as VFL server is FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The VFL preparation procedure is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes * * *

6.2H.2.2.1	Preparation procedure for Vertical Federated Learning when NWDAF is the VFL Server

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when NWDAF or Trusted AF is the VFL Server

Editor´s note:	For UEs as samples, additional discussion is needed on whether UE needs to be registered or not and whether the NWDAF as VFL server can check whether UEs are registered before interacting with VFL clients.

1.	The NWDAF or Trusted AF as VFL Server may send a Vertical Federated Learning preparation request including the Analytics ID to each of the NWDAF VFL Client(s), using Nnwdaf_VFLTraining_Request and to each of the AF VFL Clients(s), using Naf_VFLTraining_Request possibly via NEF when the VFL Client is an untrusted AF. The NWDAF as a VFL Server also provides, the suggested VFL Interoperability Information to negotiate the intermediate resuls that will be used in training, the suggested list of sample IDs that will be used in training.

NOTE 1:	For the trusted AF is acting as VFL server, the VFL client can only be the NWDAF.

2.	Each VFL Client checks if it can meet the ML Model training requirement. Each VFL Client ID checks the list of sample IDs suggested by the VFL Server and then provides to the VFL Server the list of sample IDs that it can accept out of the sample IDs suggested by the VFL Server. Each VFL Client checks the VFL Interoperability Information and determines which VFL Interoperability information that the VFL Client accepts. The VFL Clients provides the list of supported Feature IDs to the VFL Server, if available, alternatively the VFL clients may not want to reveal Feature IDs or VFL Server may know the supported FeatureIDs for a VFL Client based on configuration.

3.	Each NWDAF VFL Client invokes Nnwdaf_VFLTraining_Response or and each AF VFL Client invokes Naf_VFLTrainingRequest_Response, possibly via NEF when the AF is untrusted, to indicate to the VFL Server whether it accepts the ML Model training requirements, the VFL Client can also indicate that it cannot join the FL process.

4.	The VFL server determines the final list of samples considering the samples that all selected VFL clients support, if used the Feature ID per VFL client and VFL Interoperability Information to be used for training and provide them to the selected VFL Clients at the start of the training phase, as described in clause 6.2H.2.3.1.

* * *End of Changes * * *