# 3GPP TSG-SA WG2 Meeting #166-Ad Hoc-e	S2-2500366

eMeeting, 20 - 24 January 2025	(Revision of S2-xxxxxxx)

Title:	[Draft] Reply LS on FS_VMR_Ph2 solution impacts to RAN (MWAB mobility)

Response to:	S2-2500053/R3-247910

Release:	Rel-19

# Work Item:	     FS_VMR_Ph2, VMR_Ph2

Source:	SA2

To:	RAN3

Cc:	RAN2

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.501 CR 2934

1. Overall Description:

SA2 thanks RAN3 for the LS (S2-2500053/R3-247910) on FS_VMR_Ph2 solution impacts to RAN with focus on MWAB mobility.

The LS highlights the issue as stated below:

1. Overall Description:

RAN3 would like to thank SA2 for their LS on FS_VMR_Ph2 solution impacts to RAN (R3-243021/S2-2405822). RAN3 has further discussed question 3:

- Question 3: To support mobility of the MWAB, some solutions assume that the MWAB-gNB can instantiate two cells (with same gNB ID or different gNB ID), and handover connected UEs between the two cells. The different gNB IDs use case is driven by the need to change AMF if the MWAB moves into a geographic area where a different AMF must be chosen to serve UEs. SA2 would like to ask RAN3 to confirm if this can be supported or not.

RAN3 confirmed SA2’s assumptions in the following agreement:

“The ‘two logical gNB solution’ can support UE’s AMF change during WAB-gNB mobility.”

RAN3 has discussed the following solutions that allow change of the UE’s AMF with a single logical WAB-gNB.

Solution 1: Using single WAB-gNB with single cell and changing TAC

- The WAB-gNB establishes a separate NG-C connection with the new AMF.

- It reports the initial TAC, TAC1, only to the initial AMF and the new TAC, TAC2, only to the new AMF.

- The WAB-gNB cell updates the SI from TAC1 to TAC2.

- For UE in RRC CONNECTED state:

  - When receiving the SI update, the UE performs an MRU, which is forwarded by the WAB-gNB to the initial AMF. The WAB-gNB includes TAC2 as part of the User Location Information to the initial AMF. Based on TAC2 of the received ULI, the initial AMF initiates an AMF reallocation to the new AMF. There is no handover procedure to be performed for the purpose of UE’s AMF change.

- For UE in RRC IDLE/INACTIVE state:

  - When receiving the SI update, the UE performs an MRU, which is forwarded by the WAB-gNB to the new AMF. The new AMF retrieves the UE’s context from the initial AMF.

Solution 2: Using single WAB-gNB with two cells and different TACs

- The WAB-gNB establishes a separate NG-C connection with the new AMF.

- It instantiates a second cell whose SI includes only TAC2, while the first cell’s SI only includes TAC1.

- It reports TAC1 only to the initial AMF and TAC2 only to the new AMF.

- For UE in RRC CONNECTED state:

  - The WAB-gNB initiates an NG handover from the first cell to the second cell for the UE. When the initial AMF receives the HO Required message, it decides to conduct an inter-AMF handover to the new AMF based on the TAC2 included in the Target ID IE of the HO Required message, and forwards the HO signalling to the new AMF. After all UEs have been handed over, the WAB-gNB discontinues operation of the first cell.

- For UE in RRC IDLE/INACTIVE state:

  - When operation of the first cell is discontinued, the UE reselects the second cell and performs an MRU, which is forwarded by the WAB-gNB to the new AMF. The new AMF retrieves the UE’s context from the initial AMF.

2. Actions:

To SA2 group.

ACTION: 	RAN3 would like to ask SA2 to provide feedback on whether either of these single-gNB solutions can be achieved without CN impact.

SA2 discussed the solution#1 with single WAB-gNB and consider it has CN impact and new procedures/handling needs to be introduced to support such. Thus solution#1 is not recommended.

SA2 considers that the solution#2 can address the UE mobility handling as long as there are two cells/TACs during the MWAB node mobility with AMF change. The existing UE mobility procedures (e.g. NGAP procedure or NAS procedure) can be re-used to handle the UEs connected via MWAB node. However, SA2 considers that single gNB connects to multiple AMF sets and reporting different TACs to different AMF sets are rather abnormal setup with additional logics in gNB. There is probably also potential issues for signalling target gNB and extra handling in CN may be needed.

Based on above, SA2 considers no reason to pursue the single logic gNB solutions and has agreed the attached CR indicating the re-use of existing mobility procedures for UE mobility when it is connected via MWAB node.

2. Actions:

To RAN3

ACTION: 	SA2 kindly asks RAN3 to take above information into consideration and update the corresponding specifications.

3. Date of Next SA2 Meeting:

SA2#167   				17 – 21 February, 2025			        Athens, GR

SA2#168   				 07 – 11 April, 2025			        	     Goteborg, SE