SA WG2 Meeting #166-Ad Hoc-e	S2-2500367

eMeeting, 20 – 24 January 2025

Source:	Ericsson

Title:	MWAB mobility analysis

Document for:	Agreement

Agenda Item:	19.49

Work Item / Release:	VMR_Ph2 / Rel-19

Abstract of the contribution: This contribution discusses the MWAB mobility handling based on LS response from RAN3.

# Background

RAN3 has provided LS reply in S2-2500053/R3-247910 which addressed the MWAB mobility aspects. The LS reply highlights two things:

1) It confirms the SA2 assumption that “two logic gNB solution” can be used to support UE mobility during MWAB node mobility.

2) It also indicates that RAN3 has discussed “single logic gNB solutions” and asks if the 2 solutions based on “single logic gNB” have CN impact.

Observation 1: The “two logic gNB solution” can be supported.

# Discussion on “single logic gNB solutions”

## Sol#1: Using single WAB-gNB with single cell and changing TAC

RAN3 LS reply highlighted the following for solution#1.

- The WAB-gNB establishes a separate NG-C connection with the new AMF.

- It reports the initial TAC, TAC1, only to the initial AMF and the new TAC, TAC2, only to the new AMF.

- The WAB-gNB cell updates the SI from TAC1 to TAC2.

- For UE in RRC CONNECTED state:

  - When receiving the SI update, the UE performs an MRU, which is forwarded by the WAB-gNB to the initial AMF. The WAB-gNB includes TAC2 as part of the User Location Information to the initial AMF. Based on TAC2 of the received ULI, the initial AMF initiates an AMF reallocation to the new AMF. There is no handover procedure to be performed for the purpose of UE’s AMF change.

- For UE in RRC IDLE/INACTIVE state:

  - When receiving the SI update, the UE performs an MRU, which is forwarded by the WAB-gNB to the new AMF. The new AMF retrieves the UE’s context from the initial AMF.

For this sol#1, when initial AMF receives the NAS Mobility Registration Update via either NGAP Uplink NAS Transport message (i.e. for UE in RRC_CONNECTED), the initial AMF will recognize the UE based on GUTI and find the corresponding UE context and handle the NAS message accordingly. Performing MRU redirection based on TAC2 which is not a valid TAC from initial AMF point of view is a totally new behavior.

This sol#1 also states that for UE in RRC_INACTIVE state, the NAS MRU message will be “forwarded” to the new AMF. It’s assumed that NGAP Initial UE message will be used in this case towards the new AMF, the N2 connection release between initial AMF and WAB-gNB becomes a rather frequent case while. currently the release of an old N2 connection is considered rather an abnormal/rare case.

Considering both RRC_CONNECTED state and RRC_INACTIVE state are treated as CM-CONNECTED mode by CN, it’s abnormal that UEs are handled differently.

Observation 2: Solution#1 has clear CN impact, especially for the UE in RRC_CONNECTED state.

In addition, in this sol#1, the same global gNB-ID of the MWAB-gNB is used when it connects to both initial AMF and the new AMF which is not due to the backup purpose. This seems a rather abnormal setup. It may create some potential problems if certain signaling is targeting a gNB. Extra logic may be needed in CN to deal with the problem.

Observation 3: Single gNB connects to multiple AMFs (or AMF sets), not for the backup purposes, is an abnormal case and may create potential problem.

## Sol#2: Using single WAB-gNB with two cells and different TACs

RAN3 LS reply highlighted the following for solution#2.

- The WAB-gNB establishes a separate NG-C connection with the new AMF.

- It instantiates a second cell whose SI includes only TAC2, while the first cell’s SI only includes TAC1.

- It reports TAC1 only to the initial AMF and TAC2 only to the new AMF.

- For UE in RRC CONNECTED state:

  - The WAB-gNB initiates an NG handover from the first cell to the second cell for the UE. When the initial AMF receives the HO Required message, it decides to conduct an inter-AMF handover to the new AMF based on the TAC2 included in the Target ID IE of the HO Required message, and forwards the HO signalling to the new AMF. After all UEs have been handed over, the WAB-gNB discontinues operation of the first cell.

- For UE in RRC IDLE/INACTIVE state:

  - When operation of the first cell is discontinued, the UE reselects the second cell and performs an MRU, which is forwarded by the WAB-gNB to the new AMF. The new AMF retrieves the UE’s context from the initial AMF.

For this sol#2, it supports two cells/TACs. This solution can address the UE mobility handling during the MWAB node mobility with AMF change. However, one gNB (e.g. with same slice configuration) in a certain area normally connects only to one AMF set. Different TACs of the same gNB served by different AMF is a rather abnormal setup and requires extra logic in the gNB (e.g. when a gNB connects to one AMF set, it normally reports all the TACs during NGAP setup).

Observation 4: From UE mobility procedure support point of view, Sol#2 provides the needed function with extra supporting logics in the gNB logic.

Observation 5: observation 3 above is also applicable to sol#2.

Considering that the Reply LS from RAN3 already indicated the possibility of support two logical gNB solution. It’s reasonable to not pursue the single logic gNB solution as discussed above and supports the two logical gNB solution only.

# Proposal

Based on the above observations and discussion, it is proposed to do the following:

1) SA2 specifies that existing procedures with TAC changes are used for UE mobility and removes the corresponding EN. Please see CR S2-2500368.

2) Reply LS to RAN WGs providing SA2 view on the recommendation of two logical gNB solution and agreed CR based on above. Please see LS Reply S2-2500366