SA WG2 Meeting #166-AdHoc-e	S2-2500388

E-meeting, 20 – 24 January 2025

Source:	LG Electronics

Title:	Discussion on behavior of Intermediate U2N Relay related to Model A discovery

Document for:	Discussion

Agenda Item:	19.7.2

Work Item / Release:	5G_ProSe_Ph3 / Rel-19

Abstract of the contribution: This paper discusses the LS from RAN2 on relay discovery announcement and how to clarify behavior of Intermediate U2N Relay related to Model A discovery.

Discussion

RAN2 sent the LS on relay discovery announcement (S2-2500045/R2-2411184) as below.

RAN2 would like to ask SA2 to provide feedback on following question: - Whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message.

[1] Layer-2 multi-hop UE-to-Network relaying

R2-2410981(Report of [Post127][401][Relay] Multi-hop relay discovery and (re)selection (LG)) proposes scenarios to be supported and scenarios not to be supported in 'Conclusion and recommendation' section based on email discussion as below:

[EASY] Proposal 13: following cases are suppored: - One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE) FFS whether the L2 ID(D) and L2 ID(D’) can be the same value in the below figure. (Figure 2.5A-2) - Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE. FFS whether the L2 ID(B) and L2 ID(B’) can be the same value in the below figure. (Figure 2.5B-1) [EASY] Proposal 14: following cases are not suppored: - Physically one intermediate Relay UE has two PC5 unicast link with the physically one last Relay UE. (Figure 2.5A-1) - Physically different two Remote UEs have each indirect connection via physically one intermediate Relay UE. The intermediate Relay UE has two PC5 unicast link with the physically one last Relay UE (Figure 2.5B-2) - One intermediate Relay UE, which also acts as Remote UE, can have two different indirect connection via physically different last Relay UE (Figure 2.5A-3) - Two physically different Remote UE can have each indirect path via the physically same intermediate Relay UE and physically different last Relay UE(s). i.e., physically one intermediate Relay UE has connections with physically two different last Relay UE(s) to support physically different Remote UE(s). (Figure 2.5B-3) - Two different indirect links at the physically same Remote UE can have connections with the gNB via the physically same intermediate Relay UE and physically same last Relay UE. (Figure 2.5C-1) - Two different indirect links at the physically same Remote UE can have connections with the gNB via the physically same intermediate Relay UE and physically different last Relay UE. (Figure 2.5C-2) - (2.5D-1) One intermediate Relay UE, which also acts as Remote UE, can have two different indirect connection via physically different last Relay UE. Each last Relay UE has a connection with different cells in the same gNB. (Figure 2.5D-1) - (2.5D-2) Two physically different Remote UE can have each indirect path via the physically same intermediate Relay UE and physically different last Relay UE(s) i.e., physically one intermediate Relay UE has connections with physically two different last Relay UE(s) to support physically different Remote UE(s). Each last Relay UE has a connection with different cells in the same gNB. (Figure 2.5D-2) - (2.5D-3) Two different indirect links at the physically same Remote UE can have connections with the gNB via the physically same intermediate Relay UE and physically different last Relay UE. Each last Relay UE has a connection with different cells in the same gNB. (Figure 2.5D-3)

Observation-1: It is observed that Layer-2 Intermediate UE-to-Network Relay (i.e. Intermediate Relay UE in the figures above) can connect with only one Layer-2 UE-to-Network Relay (i.e. Last Relay UE in the figures above) over PC5. That is, Layer-2 Intermediate UE-to-Network Relay is not allowed to have more than one Layer-2 UE-to-Network Relay connected over PC5 to support Remote UE(s).

Based on the above observation, we would like to propose the following to clarify behavior of Layer-2 Intermediate U2N Relay related to Model A discovery and the question from RAN2:

Proposal-1: If the Layer-2 Intermediate UE-to-Network Relay has the PC5 unicast link established with the Layer-2 UE-to-Network Relay, it forwards UE-to-Network Relay Discovery Announcement messages sent only from this Layer-2 UE-to-Network Relay.

Proposal-2: When the Layer-2 Intermediate UE-to-Network Relay that does not have PC5 unicast link yet with any Layer-2 UE-to-Network Relay receives a UE-to-Network Relay Discovery Announcement message from Layer-2 UE-to-Network Relay, it establishes a PC5 unicast link with the Layer-2 UE-to-Network Relay sent the discovery message before forwarding the discovery message.

Proposal-1 is because it is considered that for Intermediate U2N Relay that has PC5 unicast link with any Layer-2 U2N Relay, forwarding the discovery message from other Layer-2 U2N Relay is not useful.

Proposal-2 is to avoid ambiguity regarding what if Intermediate U2N Relay that does not have PC5 unicast link with any Layer-2 U2N Relay receives discovery announcement messages from multiple U2N Relays simultaneously. In this case, this proposal is to make the Intermediate U2N Relay establish PC5 unicast link with one U2N Relay sent the discovery message and forward the discovery message only sent by the U2N Relay just connected via PC5 (per Proposal-1).

[2] Layer-3 multi-hop UE-to-Network relaying

We believe that it would be good to also clarify behavior of Layer-3 Intermediate U2N Relay related to Model A discovery.

For layer-3 multi-hop U2N relaying, restriction described in Observation-1 is not needed.

It is considered that decision on whether to forward the received UE-to-Network Relay Discovery Announcement message by Layer-3 Intermediate UE-to-Network Relay can be based on implementation.

The Layer-3 Intermediate UE-to-Network Relay can decide to forward the received UE-to-Network Relay Discovery Announcement message, for example:

-	because it has the PC5 unicast link established with the relay that sent the discovery message.

-	because it does not have PC5 unicast link yet with the relay that sent the discovery message, but the PC5 signal strength/quality of the received message is good.

Based on the above discussion, we would like to propose the following to clarify behavior of Layer-3 Intermediate U2N Relay related to Model A discovery:

Proposal-3: It is proposed to clarify that the following text described in clause 6.3.2.5.2 of TS 23.304 is for Layer-3 U2N relaying.

Based on configuration, the 5G ProSe Intermedidate UE-to-Network Relay may need to establish a Layer-2 link with the relay identified by the Announcer Info in the discovery entry before sending the 5G ProSe UE-to-Network Relay Discovery Announcement message.

Proposal-4: It is proposed to add one NOTE to clause 6.3.2.5.2 of TS 23.304 for clarifying that decision on whether to forward the received UE-to-Network Relay Discovery Announcement message by Layer-3 Intermediate UE-to-Network Relay can be based on implementation.

Proposal

It is proposed to discuss and approve the following Tdocs reflecting the proposals in Discussion section:

-	S2-2500389: TS 23.304 CR "Clarification on behavior of Intermediate U2N Relay related to Model A discovery"

-	S2-2500390: [Draft] Reply LS on relay discovery announcement