3GPP TSG SA WG2 Meeting #166 AH-E	S2-2500409

20 – 24 January, 2025, (e-meeting)   	 (was S2-2411710)

Source:	Ericsson

Title:	Solving KI#4 with N6 metadata using KI#2 mechanisms

Document for:	Approval

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2/ Rel-19

Abstract of the contribution: This contribution discusses whether SA2 should specify a solution for KI#4 using KI#2 mechanisms.

# Background

The purpose of this paper is to address the Editor’s Note in TS 23.501 clause 5.37.9.1:

“Editor’s Note: Other parameters related to KI#4 are FFS.

TS 23.501 Clause 5.37.9.1 describes the general aspects for how to support transferring media related information over N6. The EN leaves FFS whether KI#4 related parameters should be sent as media related information.

KI#4 has studied the Traffic detection and QoS flow mapping for multiplexed data flows. An exception request has been agreed for XRM Phase 2. Addressing this EN is covered by the exception request as follows:

“Support differentiated QoS handling of ciphered multiplexed XRM traffic other than SRTP.  Scope is DL only. It is contentious whether this feature must be done in Rel-19”.

# Discussion

Discussion follows in relation to the EN in TS 23.501 clause 5.37.9.1:

“Editor’s Note: Other parameters related to KI#4 are FFS.

This EN leaves FFS whether any parameters related to KI#4 should be added to the media related information that 5GC can receive using the mechanisms defined for KI#2.

KI#2 conclusions included that other KIs than KI#2 could contribute to the media related information (MRI), e.g. KI#5.

Then, KI#5 conclusions state explicitly that “The filter information between AF and UPF (via PCF and SMF) is enhanced to be able to detect the "expedited transfer indication" in N6 metadata for QoS flow mapping”. The “Expedited Transfer Indication” is already listed in the media related information in TS 23.501 clause 5.37.9.1.

But, whereas for KI#4, the TR includes many solutions and some of these solutions propose that 5GC receives media related data on N6 e.g. reusing the mechanisms discussed for KI#2, none has been included in the agreed conclusions (TR has no evaluation). That is, Using N6 metadata is not part of the agreed conclusions.

Observation 1: KI#4 related parameters in XRM metadata over N6 was not part of the conclusions of the study phase. There was no agreement to promote this to normative phase.

There was no time for evaluations and the TR does not include the reasons why KI#4 related parameters in XRM metadata over N6 using KI#2 agreed methods were not part of the conclusions. However, S2-2406230 provided reasons why KI#4 solutions based on XRM metadata over N6 do not solve the UL.  As exception request indicates now, for this topic “Scope is DL only”.

Observation 2: a solution based on KI#4 related parameters in XRM metadata over N6 is a partial solution which can only be used for the DL.

For KI#4, several solutions in the TR say to address the scenario where (S)RTP headers are not sent in clear (e.g. if media is sent with IETF draft for RoQ) and propose to use metadata over N6. But what information that should be sent, have not been discussed and agreed. Just with the solutions in the TR, alternatives for KI#4 data in MRI include Track ID (as in Solution#9), stream info (as in Solution #14) Connection or Stream ID (as in Solution #15), QSC-ID (as in Solution #27), sub-flow ID (as in Solution #29) or Obfuscated codes (as in Solution #12). As proposed, they are provided by AF to match the data provided by AS over N6. A variant (not in the TR) would be a label provided by AF which is then shared in metadata by all the media streams that share QoS needs. Another solution (not in the TR) would be replicating the agreed (S)RTP Headers added to the Packet Filters in the metadata. These solutions have commonalties and differences that deserve proper analysis The selected solution is likely to impact more than N6.

Observation 3: it is unclear which parameters related to KI#4 should be added to the N6 metadata. The many alternatives need to be evaluated and compared, so as any additional impacts on 5GC.

And as in exemption request, intention is to “Support differentiated QoS handling of ciphered multiplexed XRM traffic other than SRTP”.

The adoption of QUIC as transport protocol is a very clear internet trend. When QUIC is used (e.g. when RTP-over-QUIC (RoQ) is used), also the RTP headers are ciphered (i.e. encrypted) and not visible.

As in KI#4 TR definition, it is this KI concern that “Several media streams could be multiplexed on the same end-to-end transport layer connection”.  Luckly, IETF has specified extensions for multipath QUIC (MP-QUIC) that can be used to send the different media streams multiplexed on the same end-to-end transport layer connection (i.e. the one QUIC connection and single security context) on different paths (i.e. using different 5-tuples for the different media). The information flow is shown in the Figure below. Any Load balancers in the path can be configured to guarantee all the packet flows belonging to the same QUIC connection are steered to same end point.

Figure 1: MP-QUIC for stream multiplexing in the same QUIC transport connection

Being this an emerging technology, application design can be influenced; applications can be recommended to multiplex their media streams on multiple 5-tuples with MP-QUIC. If so, these streams can receive differentiated QoS both in UL and DL and 5GS capabilities suffice to provide this differentiated QoS treatment.

Observation 4: the value added by a (DL only) N6 metadata-based solution for ciphered multiplexed media is unclear. QUIC based XRM applications can use multiple paths (different 5-tuples) for the different media streams multiplexed in same transport connection.

Based on the observations above, the following Proposal is included.

# Proposal

Adding parameters related to KI#4 in the N6 metadata was not agreed in the study phase. The value it adds to the 5GS is unclear considering such solution would only work for DL and taking into account that, if transport is QUIC (a major internet trend), different 5-tuples can be used for the media streams multiplexed in a single QUIC transport connection. It is also unclear how the N6 metadata solution should look like (which parameters); the alternatives were not evaluated due to lack of time (evaluation work would be required now).

Proposal:

Remove the EN. No further work on KI#4 related parameters for the Media related Information in Release’19. CR provided in S2-2500410.

KI#4 for E2E encrypted traffic (e.g. RoQ) can be a candidate for a future release.

Discuss potentially adding Rel-19 recommendation that media applications designed to use QUIC transport use MP-QUIC extensions to multiplex the media streams so different streams (same transport connection) use different 5-tuples. If so, a CR can be provided to next meeting.