3GPP TSG-WG2 Meeting #166AH-e	S2-2500440

# E-meeting, January 20 - 24, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0164 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Indicating optional steps of the pre-flight procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The discription of the procedure can be percieved misleading. If the NEF detemines in Step 5 to reject the pre-flight planning request, the further steps 8-15 are not required and shall be skipped. However, this aspect is not specified properly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Specifying in Step 6 that some of the remaining steps of the procedure are skipped in cases when the NEF determines to reject the request, but, if the NEF determines to proceed, it is specified with which steps the procedure needs to continue. -Adding a note clarifying which steps are optional and that these optional steps can be invoked indepedently. -Updating the figure accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading description that could lead to wrong stage 3 implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

### 5.12.2	Procedure for NEF Assisted Pre-flight Planning

USS / UTM

USS / UTM

GMLC

GMLC

NFs, OAM

NFs, OAM

NWDAF

NWDAF

UAV UE

UAV UE

UAS NF / NEF

UAS NF / NEF

1. PDU session establishment as described in clause 5.2.3 of TS 23.256 (for communication between UAV/UAV-C and USS/UTM)

1. PDU session establishment as described in clause 5.2.3 of TS 23.256 (for communication between UAV/UAV-C and USS/UTM)

2. Pre-flight planning request (via the application layer)

3. Determining information for the pre-flight planning request towards the UAS NF/NEF

3. Determining information for the pre-flight planning request towards the UAS NF/NEF

4. Nnef_RetrieveInfoUAVFlight_Get request

5. Determining NFs and services to invoke for the request

5. Determining NFs and services to invoke for the request

6. Nnef_RetrieveInfoUAVFlight_Get response

7. Response to the Pre-flight planning request (via the application layer)

7. Response to the Pre-flight planning request (via the application layer)

8. Steps 2–5 from clause 5.3.4 of TS 23.256 to obtain a list of aerial UEs in a geographic area

8. Steps 2–5 from clause 5.3.4 of TS 23.256 to obtain a list of aerial UEs in a geographic area

9. Obtaining the Movement Behaviour analytics as described in clause 6.21.4 of TS 23.288 [6]

9. Obtaining the Movement Behaviour analytics as described in clause 6.21.4 of TS 23.288 [6]

10. Obtaining the QoS Sustainability analytics as described in clause 6.9.1 of TS 23.288 [6]

10. Obtaining the QoS Sustainability analytics as described in clause 6.9.1 of TS 23.288 [6]

11. Invoking GMLC service(s)

11. Invoking GMLC service(s)

12. Generating the pre-flight assistance information

12. Generating the pre-flight assistance information

13. Nnef_RetrieveInfoUAVFlight_Get response

13. Nnef_RetrieveInfoUAVFlight_Get response

14. Planning a flight path(s)

14. Planning a flight path(s)

15. Response to the Pre-flight planning request (via the application layer) containing the planned flight path(s)

15. Response to the Pre-flight planning request (via the application layer) containing the planned flight path(s)

Figure 5.12.2-1: Procedure for NEF Assisted Pre-flight Planning

1.	The UAV establishes a PDU Session for communication with the USS/UTM as described in clause 5.2.3.

2.	(via application layer) The UAV requests the pre-flight planning service from the USS/UTM.

3.	The USS/UTM determines to request a pre-flight assistance information from the 5GC and derives the information to provide to the NEF/UAS NF.

NOTE:	The content of pre-flight planning service information derived at USS is out of scope of 3GPP.

4.	The USS/UTM sends an Nnef_RetrieveInfoUAVFlight_Get request with the indication to provide a pre-flight planning assistance for a target UAV (i.e. aerial UE). The request message includes identifier of the UAV (e.g. GPSI), information about the starting and ending points (and any intermediate point, if any) for the flight. The request may also include requirements on the flight path (e.g. on time, fastest, shortest), candidate flight path(s) (either received from the UAV or locally derived at the USS/UTM) and accuracy level of predictions relevant to the flight planning.

5.	The NEF maps parameters included in the received Nnef_RetrieveInfoUAVFlight_Get request from the USS/UTM to information used by the 3GPP system (e.g. map the geographical area into an area of interest that is represented by a list of Cell IDs, gNB IDs or TAIs). The NEF determines services needed for the request and relevant NFs, e.g. NEF service on UAV tracking and mode (UAV location reporting mode, UAV presence monitoring mode, List of Aerial UEs in a geographic area), NWDAF analytics service (Movement Behaviour analytics and/or QoS Sustainability Analytics), GMLC service (e.g. Ranging/Sidelink Positioning location).

6.	If the NEF determines that the 5GS cannot satisfy the requirements provided by the USS/UTM on flight planning (e.g. cannot get predictions relevant to the flight planning from NWDAF with the required accuracy level), the NEF rejects the pre-flight planning request from the USS/UTM and the NEF may include the details about the reasons of the rejection inside the response; steps 8 – 15 are skipped.

7.	(via application layer) If the USS/UTM receives the response to the Nnef_RetrieveInfoUAVFlight_Get with the rejection of the pre-flight planning in step 6, the USS/UTM may respond to the UAV's pre-flight planning request and include the received details rejection reason(s) if available.

8.	If the NEF determines to proceed with the pre-flight planning, the NEF performs steps 2-5 of clause 5.3.4 to determine a list of UAVs in the areas of interest.

NOTE X:	Steps 9-11 are optional and can be invoked independently of each other and based on the NEF decision in Step 5.

9.	The NEF subscribes/requests for notification on Movement Behaviour analytics from NWDAF as defined in clause 6.21.4 of TS 23.288 [20]. The request includes an identifier of the UAV (e.g. GPSI) obtained in step 4 and height information of the UAV; other parameters included in the request are described in clause 6.21.4 of TS 23.288 [20]. Additionally, if the list of UAVs in the areas of interest is obtained in step 8, it may be used by the NEF to request Movement Behaviour analytics from NWDAF.

10.	The NEF subscribes/requests for notification on QoS Sustainability analytics from NWDAF as defined in clause 6.9.1 of TS 23.288 [20]. The request includes an identifier of the UAV (e.g. GPSI) obtained in step 4 and the UAV's flight path information; other parameters included in the request are described in clause 6.9.1 of TS 23.288 [20]. Additionally, if the list of UAVs in the areas of interest is obtained in step 8, it may be used by the NEF to request QoS Sustainability analytics from NWDAF.

11.	The NEF may request GMLC service(s), e.g. Ranging/Sidelink positioning location as defined in clause 6.20.4 of TS 23.273 [8]. The list of UAVs in the areas of interest output from step 8 may be used by the NEF to request the GLMC service on Ranging/Sidelink Positioning location.

12.	The NEF generates the pre-flight assistance information based on the outputs from steps 8-10. The pre-flight assistance information may contain the best matching path(s) among the ones provided by the USS/UTM in step 4, or potential flight path information if candidate flight path(s) are not provided in step 4.

13.	The NEF responds to the USS/UTM with the pre-flight assistance information.

14.	The USS/UTM derives the flight path(s) using the retrieved pre-flight assistance information and flight planning mechanisms); these mechanisms are outside the scope of 3GPP.

15.	(via application layer) The USS/UTM responds to the UAV with the planned flight path(s).

>>>> End of Changes <<<<