SA WG2 Meeting SA2#166 adhoc E-meeting                                                S2-2500487

Elbonia, January 20 – 24, 2025

Source:	Samsung

Title:	Discussion and proposals on multi-modal service awareness

Document for:	Information

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2, NR_XR_Ph3-Core / Rel-19

Abstract of the contribution: This contribution discusses the current progress related with multi-modality awareness at RAN, and provides the proposals to support the multi-modal service awareness in R-19 work.

1. Discussion

RAN WG2 sent an LS [S2-2407407] to SA2 to ask whether/what information can be provided from SA WG2 (e.g. MMSID and/or synchronization thresholds in order to allow e.g. coordinated handling of the flows at RAN).

During SA2#164 meeting, SA2 discussed the LS from RAN2 and sent back an LS[S2-2409444] to RAN2. In this LS, SA2 replied that the MMSID provided by the CN to RAN is technically feasible and questioned its usage.

Per LS response (S2-2500014/R2-2409272) RAN2 provided the usage of MMSID in the NG-RAN side according to the question from SA2, which is for joint admission control and QoS flow to DRB mapping.

And per LS response (S2-2500015/ R3-245682) RAN3 replied that it is technically feasible to enhance NGAP to provide the MMSID to NG-RAN node.

Moreover, per LS response (S2-2500028/ S4-242223), SA4 replied that the joint admission control information is determined by application layer. So such requirements for joint admission control should be provided to 5GS from AF.

And, per LS (S2-2500074/ SP-241921), SA plenary recommends that CN may provide MMSID to NG-RAN.

Observation 1: It is beneficial for CN to provide NG-RAN with the MMSID to help joint admission control and DRB mapping.

Observation 2: In order to support joint admission control in NG-RAN for the multi-modal QoS flows, joint admission control requirements should come from application server to 5GC and, if joint admission control is required, CN should send joint admission control indication (i.e. all the multi-modal QoS flows are handled together as either admitted or rejected.)

In the LS (S2-2500014/R2-2409272) from RAN2, it also agreed that traffic synchronization may be achieved by packet level synchronization information, if it can be provided from CN to RAN. But, per LS response(S2-2500028/ S4-242223), SA4 replied that such packet level synchronization is not required.

Observation 3: packet level synchronization for multi-modality service support is not required.

Based on the above observations, it is proposed that SA2 should support the following enhancements in R19 to support multi-modality awareness in NG-RAN.

Proposal 1: AF may provide joint admission control requirement to 5GC together with MMSID. If joint admission control is required, CN should send joint admission control indication (i.e. all the multi-modal QoS flows are handled together as either admitted or rejected.) as well as MMSID to NG-RAN.

2. Proposal

Based on the above discussions, the following proposals are required:

Proposal 1: AF may provide joint admission control requirement to 5GC together with MMSID. If joint admission control is required, CN should send joint admission control indication (i.e. all the multi-modal QoS flows are handled together as either admitted or rejected.) as well as MMSID to NG-RAN.