3GPP TSG-SA2 Meeting #166-Ad Hoc-e	S2-2500488

# Online, 20th Jan 2025 - 24th Jan 2025	                                                  revision of S2-24xxxx

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0280 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 Remove EN on AF traffic influence information for I-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TR 23.700-49, the conclusion describes that SMF may forward AF provided traffic influence information to I-SMF. The related EN should be addressed: “Editor's note: whether the AF traffic influence information is transferred to I-SMF is FFS.” The AF traffic influence information can be delivered to the I-SMF to enable I-SMF to perform reporting UP path change directly (i.e., not via SMF) to AF. Otherwise, although Local Offloading Management is applied, the SMF still suffers from managing and processing the UP path change notification in the middled of I-SMF and AF for the traffic subject to Local Offloading Management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove EN SMF to forward AF provided traffic influence information to I-SMF when Local Offloading Management is applied Editorial clean up |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s note remains. Mis-operation is expected when Local Offloading Management is applied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.3.1. 6.10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

6.10.3.1	General

For Local Offloading Management, traffic routing related policy (e.g., local offloading management policy) need be provided to the I-SMF.

Based on the received policy information and EAS Deployment information, the I-SMF may (re)discover and (re)select ULCL and local PSA UPF for routing the traffic locally.

If Local Offloading Management is applied, the AF traffic influence information can be transferred to the I-SMF in addition to Local Offloading Management Policy information to enable I-SMF to perform reporting UP path change directly (i.e., not via SMF) to AF.

NOTE:	The AF does not need to be aware of whether Local Offloading Management is applied or not.

6.10.3.2	Local offloading management policy handling for Local Offloading Management

The Local Offloading Management Policy is generated by PCF and retrieved by SMF for Local Offloading Management as described in clause 6.10.2. The Local offloading Management Policy may also be provided with an Offload Identifier. The Offload Identifier is assigned by PCF, and is unique within the PLMN. The SMF and I-SMF for Local Offloading Management may also be preconfigured with Local Offloading Management policy and tagged with related Offload Identifier(s). In this case, the PCF only provides the Offload Identifier to the SMF.

The detailed information of Local Offloading Management Policy is described in clause 6.4 of TS 23.503 [4]. The SMF forwards the received Local Offloading Management Policy or Offload Identifier to the I-SMF transparently.

>>>> End of Changes  <<<<