3GPP TSG-SA2 Meeting #166-Ad Hoc-e	S2-2500490

# Online, 20th Jan 2025 - 24th Jan 2025	                                                  revision of S2-24xxxx

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0281 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 Handling application not matched with LOM policy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In LOM scenario, there might be a case where a DNS query is not matched with the LOM policy information, but its associated information is included in the EDI. Such an application associated with the DNS query needs to be served by EAS even though it will not be treated as Local Offloading Management. For this purpose, this CR proposes that |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification of Local Offloading Management Allowed indication and configuration of EASDF by I-SMF. Terminology alignment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification remains unclarified and misaligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

6.10.2.3	EAS Discovery Procedure with EASDF for Local Offloading Management

The EASDF is selected and configured by the I-SMF and the DNS query of the UE is routed to the EASDF directly or via UL CL as defined in clause 6.7.2.3.

The I-SMF handles traffic for the application not matched with Local Offloading Management information at I-SMF as follows:

-	Based on operator policy, the I-SMF may provide BaselineDNSPattern for the application of which information is not part of FQDN contained in the Local Offloading Management information but contained in the EAS Deployment information available at I-SMF (via preconfiguration or retrieved from NEF) in order to DNS handling as follows:

-	If the target FQDN of the DNS query from UE is not part of FQDN contained in the Local Offloading Management information and if the target FQDN of the DNS query is part of the retrieved EDI at I-SMF (i.e., matched with Baseline DNS message detection template provided by I-SMF), the EASDF sends the DNS query which may include ECS option for EAS discovery. The EASDF also sends DNS message reporting to the I-SMF if the EAS IP address or the FQDN in the DNS Response message based on the I-SMF instruction;

-	If the target FQDN of the DNS query is not part of the EDI at I-SMF (i.e., neither matched DNS handling or Baseline DNS message detection template) rule, based on I-SMF instruction, the EASDF simply forwards the DNS query towards a preconfigured DNS server/resolver.

>>>> End of Changes  <<<<