3GPP TSG-SA2 Meeting #166-AdHoc-e	S2-2500515

# E-meeting, 20 – 24 January, 2025                  			             			(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5967 | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Sove the editor’s note related to mobility support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR is proposed to solve the following editor’s note based on the RAN3 LS (S2-2405822/R3-243021), Editor's note: This clause may need update once RAN WG3 concludes on mobility aspects for connected mode UEs upon MWAB mobility. Based on the RAN3 LS, it is confirmed that the two logical gNBs can support UE’s AMF change during WAB-gNB mobility. So this CR adds the description related to two logical gNBs. Furthermore, two more solutions are also included in the LS, for the solution 2, as follows, Solution 2: Using single WAB-gNB with two cells and different TACs - The WAB-gNB establishes a separate NG-C connection with the new AMF. - It instantiates a second cell whose SI includes only TAC2, while the first cell’s SI only includes TAC1. - It reports TAC1 only to the initial AMF and TAC2 only to the new AMF. - For UE in RRC CONNECTED state: - The WAB-gNB initiates an NG handover from the first cell to the second cell for the UE. When the initial AMF receives the HO Required message, it decides to conduct an inter-AMF handover to the new AMF based on the TAC2 included in the Target ID IE of the HO Required message, and forwards the HO signalling to the new AMF. After all UEs have been handed over, the WAB-gNB discontinues operation of the first cell. - For UE in RRC IDLE/INACTIVE state: - When operation of the first cell is discontinued, the UE reselects the second cell and performs an MRU, which is forwarded by the WAB-gNB to the new AMF. The new AMF retrieves the UE’s context from the initial AMF. Considering it has already been supported by the following description in clause 4.9.1 in TS 23.502: The Inter NG-RAN node N2 based handover procedure specified in clause 4.9.1.3 may also be used for intra-NG-RAN node handover. NOTE: One use case for intra-NG-RAN handover to be performed by the Inter NG-RAN node N2 based handover procedure is when an NG-RAN node serves a satellite access system that covers more than one country. In such a situation, the UE might move from a "cell" in one country into a "cell" in another country and the NG-RAN node may need to cause the AMF to change to an AMF serving the UE's new country. This CR also adds the description related to instantiating a second cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Solve the editor’s note related to mobility support. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unsolved editor’s note. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

5.49.5.2	UE mobility when the UE is moving together with a MWAB cell

For a UE served by an MWAB-gNB cell, the MWAB-gNB may trigger the intra-gNB handover procedures for UEs in CM-CONNECTED mode as specified in TS 38.401 [42] and/or the Mobility Registration Update as defined in TS 23.502[3] when the AMF serving the UE is not changed. In this case, for UEs in CM-IDLE mode, the procedure for cell (re) selection as specified in TS 38.304 [50] and TS 23.122 [17] for RRC_IDLE and RRC_INACTIVE and/or Mobility Registration Update as defined in TS 23.502[3] is used.

When the MWAB moves into a geographica area where a different AMF in Broadcasted PLMN should be chosen to serve UEs, to handle UE mobility, the MWAB-gNB triggers the inter NG-RAN node N2 based handover wih AMF relocation specified in TS 23.502 [3] for UEs in RRC_CONNECTED state. For UEs in RRC_IDLE and RRC_INACTIVE state, the Mobility Registration Update as specified in TS 23.502 [3] is used.

NOTE 1:	The detailed behavior of MWAB-gNB is specifed in TS 38.401 [42].

* * * End of Change * * *