3GPP SA WG2 Meeting #166-AdHoc-e	S2-2500519

Electronic Meeting, 20-24 January 2025

# Title:	[Draft] Reply LS on Clarification request on usage and control of UE Local Configuration

# Response to:	Reply LS on Clarification request on usage and control of UE Local Configuration (C1-247121/ S2-2500034)

# Release:	Release 15

# Work Item:	5GS_Ph1

Source:	SA2

To:	CT1, GSMA URSPWI

Cc:	GSMA TSG

Contact Person:

Name:	Hong Cheng

Tel. Number:

E-mail Address:	hongc AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 would like to thank CT1 for their LS (C1-247121/ S2-2500034) on Clarification request on usage and control of UE Local Configuration.

SA2 has discussed the questions in the LS for SA2 and would like to provide the answers as follows:

For the following question:

Q2. If the operator can populate the UE local configuration for non seamless non-3GPP offload (see question 1.1.), what is the intended use of the RSD parameter "Non Seamless Offload Indication" defined in the URSP rule?

Answer: UE local configuration is optional and may or may not be configured on the MT or TE. If UE local configuration is configured on the MT or TE, it takes precedence over URSP rules (for non-seamless non-3GPP offload). However, if UE local configuration is not configured on the MT or TE, the operator can request non seamless non-3GPP offload by use of the RSD parameter "Non Seamless Offload Indication" defined in the URSP rule. Further, operator specific UE local configuration is not needed for the case when the non-seamless non-3GPP offload does not go through the ePDG or N3IWF entities that are defined by 3GPP.

CT1 asks SA2 to confirm the above response.

SA2’s answer: SA2 can confirm the response from CT1. Furthermore, SA2 would like to point out that the operator specific UE local configuration may be used by operators to direct certain traffic away from the ePDG or N3IWF entities.

For the following question:

Q3. Any particular reason why what seem to be three distinct sets of UE local configurations are captured in the same object?

Answer: CT1 asks SA2 to provide appropriate response.

SA2’s answer: From 3GPP perspective, different methods are provided to manage the UE local configurations. This offers a flexible tool to address all possible deployment cases. However, it does not mean that all three sets of configurations need to be present for a particular UE. The operators can decide the appropriate configurations to use for a specific deployment.

SA2 would like to ask GSMA and CT1 to take the above information in account.

2. Actions:

To GSMA, CT1:

ACTION: 	GSMA and CT1 to take the above answers from SA2 into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #167		17-21 February 2025	Athens, Greece

TSG-SA2 Meeting #168		7-11 April 2025	Goteborg, Sweden