3GPP TSG-WG SA2 Meeting #166 Ad-Hoc-e	S2-2500522

20th – 24th January, 2024, Online

Source:	China Mobile

Title:	Reader selection for Ambient IoT service request

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT

Abstract: This paper proposes reader selection methods.

1. Introduction/Discussion

This contribution is prepared to adds reader selection functionality in 5.3.

2. Text Proposal

It is proposed to approve the following changes.

FIRST CHANGE

- High level functionality and features

5.3	Reader selection

In topology 1, the AIoTF can select one or multiple targeted AIoT RAN Nodes by matching location information in the service request from AF initially and AIoT RAN Node serving location information. The AIoT RAN Node serving location can be obtained per AIoT RAN node granularity or per reader granularity. For the former one, only the information of AIoT RAN node is contained. While for the latter one, information of both AIoT RAN node and reader are contained.

In direct connection mode, an AIoT RAN node should perform NGAP procedure towards AIoTF to register itself into AIoTF successfully. The AIoT RAN node transfers its features e.g. AIoT RAN node serving location information to AIoTF via Nx interface. In addition, the AIoTF can also obtain AIoT RAN node’s serving location by OAM configuration.

Note 1: AIoT RAN Node may obtain serving AIoTF by OAM configuration in direct connection mode.

In indirect connection mode, an AIoT RAN node should perform NGAP procedure towards AMF, once it is registered into AMF successfully, AMF should forward AIoT RAN Node relevant information, e.g. AIoT RAN node serving location information, to AIoTF via Nz interface. To acquire and update the AIoT RAN node relevant information, AIoTF should subscribe the Subscribe/Notify service towards AMF.

The AF need provide the expected location information to NEF inAmbient IoT service request. The location information can be geographic location or external location identifier pre-negotiated between Operator and the AF. NEF can convert geographic location or the external location identifier to internal location identifier (TA list or cell list). NEF can query NRF to obtain serving AIoTF based on internal location identifier. Then NEF will forward the Ambient service request to AIoTF with internal location identifier.

Note 2: The AIoT RAN Node information is not exposed to the AF.

AIoTF can select AIoT RAN nodes with location information or with readers information depending on the AIoT RAN node serving location granularity.

If AIoT RAN node informs AIoTF of serving location information on AIoT RAN node granularity, AIoTF will select target AIoT RAN Nodes by matching location information in the service request from AF via NEF and AIoT RAN Node serving location information. Once the target AIoT RAN node is selected, AIoTF will forward the Ambient IoT service requests including internal location identifier list to it. Then the AIoT RAN node chooses readers based on internal location identifier list info and configuration.

If AIoT RAN node informs AToTF of serving location information on reader granularity, AIoT will select both target AIoT RAN Nodes and readers by matching location information in the service request from AF via NEF and AIoT RAN Node serving location information. Once the target AIoT RAN node is selected, AIoTF will forward the Ambient IoT service requests including the selected reader list to it. Then the AIoT RAN node can choose the readers decided by AIoTF to execute AIoT service operations.

If there is no matching between location information in the service request from AF and AIoT RAN Node serving location information, the AIoTF shall reject the service request with appropriate cause.

If there are multiple AIoT RAN Nodes matching between location information in the service request from AF and AIoT RAN Node serving location information, the AIoTF shall send the AIoT service request to these matched AIOF RAN nodes simultaneously.

Optionally, if the operation is towards to one or multiple certain Ambient IoT Devices, the AIoTF can take the last serving AIoT RAN node or Reader into account. After receiving the service request including target location information and target AIoT Device or multiple AIoT Devices, AIoTF firstly looks up the stored last serving AIoT RAN node or Reader for each AIoT Device and selects the last serving AIoT RAN node or Reader to perform this operation. If the selected AIoT RAN node or Reader cannot find out the target Device(s), the AIoTF performs reader selection again based on the location information depicted as the above-mentioned solutions.

Note3: The information of last serving AIoT RAN node is not applied to operations for a group of AIoT Devices and for the case where the identifiers of Device(s) obtained from AF cannot be recognized.

END OF CHANGES