3GPP TSG-SA2 Meeting #166AH-e	S2-2500533

# Electronic meeting, January 20 – 24, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5252 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Provisioning of Non-3GPP Device Identifier(s) Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. Non-3GPP Device Identifier provisioning procedure in clause 4.15.6.15 describe that Service Description contains an AF-Service-Identifier indicating that the request is for providing non-3GPP device Identifier information. However, such indication is not needed because Service Parameter itself only contains information related Non-3GPP Device Identifier. Therefore, NEF can determine whether to update Non-3GPP Device Identifier Information based on the parameter itself. 2. Service Parameters Contains optional DNN and S-NSSAI information. However such information is not used in anywhere. Therefore, those parameters can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Removed "indicating that the request is for providing non-3GPP device Identifier information." from the AF-Service-Identifier description. 2. Removed optional DNN and S-NSSAI parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessay information exchange without any usage in the network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

#### 4.15.6.15	Provisioning of Non-3GPP Device Identifier(s) Information

This clause describes the procedures to allow an AF to provision QoS information for the non-3GPP device(s) (as defined in clause 5.52 of TS 23.501 [2]) that require differentiated QoS treatment for a UE subscription to the 5G system via NEF. The AF may belong to the operator or to an external party.

To provision QoS information for the non-3GPP device(s) that require differentiated QoS treatment, the procedure defined in clause 4.15.6.7 is performed with the following considerations:

1)	Service Description. Contains an AF-Service-Identifier.

2)	Service Parameters. Contains the non-3GPP Device Identifier Information which has the following parameters:

-	Non-3GPP Device Identifier is a generic string, which uniquely identifies a non-3GPP device behind a specific UE subscription.

-	QoS Reference or Individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20].

-	Optionally, Flow description information as specified in clause 6.1.3.6 of TS 23.503 [20]

3)	The Target UE identifier(s). A specific UE identified by e.g. GPSI or SUPI.

The NEF sets the value of Data Subset Identifier to "Non-3GPP Device Identifier Information" in Nudr_DM_Create/Update/Delete service operations when storing/updating/deleting this data. NEF determines to use this data subset based on AF-Service-Identifier.

* * * * End of Changes * * * *