3GPP TSG-SA2 Meeting #166-Ad Hoc-e	S2-2500538

Online, 20th Jan 2024 – 24th Jan 2024

# Title:	[Draft] Reply LS to RAN2: Reply LS on Satellite IDs for store-and-forward operation

# Response to:	LS on Satellite IDs for store-and-forward operation (S2-2500047/ R2-2411197)

# Release:	Release 19

# Work Item:	5GSAT_Ph3_ARC, IoT_NTN_Ph3-Core

Source:	SA2

To:	RAN2

Cc:

Contact Person:

Name:	Saubhagya Baliarsingh

Tel. Number:

E-mail Address:	saubhagya.baliarsingh@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: CR3852

RAN2 discussed how an IoT NTN UE capable of store-and-forward operation uses the MME-configured satellite ID list in the access stratum. RAN2 made the assumption that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite.

RAN2 would like to request SA2 to provide feedback on whether the following RAN2 understanding can be confirmed:

- RAN2 understanding: The UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

SA2 would like to thank RAN2 for the LS. Please find the answers provided below.

Also, RAN2 would like to respectfully ask the question below to SA2:

Question: If a satellite is included in the satellite list signalled by the MME to a UE, does it mean one of the below options, or possibly other option(s):

- the satellite has the UE context but does not necessarily support store-and-forward operation; or

- the satellite has the UE context and must support store-and-forward operation (but may be operating in normal IoT NTN mode or in store-and-forward mode when the satellite serves the area where the UE is located)?

[SA2 answer] SA2 confirms the above understanding is correct. The satellite ID is only a recommendation for the UE to minimize reattempts and failures, but it doesn’t prevent the UE from camping on different satellites other than the configured list.

[SA2 answer] The satellite ID list is formed on the basis of context synchronization. i.e., only those satellites are listed where the UE context is available. But a satellite listed may operate in S&F or normal mode depending on the availability of feeder link connectivity while serving the UE.

2. Actions:

To RAN2:

ACTION: 	SA2 asks RAN2 to take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #167		17-21 February 2025	Athens, Greece

TSG-SA2 Meeting #168		07-11 April 2025	Goteborg, Sweden