3GPP TSG-SA2 Meeting #166AH-e	S2-2500539

# Electronic meeting, January 20 – 24, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0287 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification and correction on the LOM Service Area |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Local Offloading Management can be applied regardless UE is within or outside of the SMF service area and current description with Local Offloading Management Service Area is unclear how it is configured. Also, when the Target AMF removes the old I-SMF while the SMF cannot support the new UE location, additional steps and signaling is expected to remove old I-SMF and to select another new I-SMF for ETSUN. It is proposed with updates for clarification and correction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | § 6.10.1 - Clarify that the Local Offloading Management Service Area is configured in the AMF. § 6.10.2.4 - Propose to update for the case when the UE is outside local offloading mangement service area while the old I-SMF can serve new UE location. The old I-SMF can be kept for ETSUN and prevent additional signalling and handover duration. § 6.10.3.2. - Editorial to remove 'transparently' from the description on the Local Offloading Management Policy provided via SMF to the I-SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear how the Local Offloading Management Service Area is configured and examined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.1, 6.10.2.4, 6.10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

6.10.1	General

In edge computing deployment, to reduce the impact on a centrally deployed SMF on management of the edge computing related information, 5GS may support the Local Offloading Management.

In Distributed Anchor Point and Multiple PDU Sessions connectivity models (see clause 4.3), the existing means such as URSP and SSC mode are used to support Local Offloading Management.

In Session Breakout connectivity model, the Local Offloading Management only applies to the non-roaming PDU session. For Session Breakout connectivity model, if the AMF determines that the UE is within the Local Offloading Management service area configured in the AMF and Local Offloading Management is allowed in the SMF selection subscription data, AMF selects/re-selects/reuses an I-SMF to support Local Offloading Management. This also includes the case when the UE is within the SMF service area. For the Local Offloading Management, the I-SMF is in charge of edge computing handling and supports following functionalities:

-	EASDF configuration and DNS message handling, as defined in clauses 6.2.1 and 6.2.3, instead of SMF.

-	EASDF discovery and selection is performed by the I-SMF instead of SMF as described in clause 6.3.23 of TS 23.501 [1].

-	Management and retrieval of EDI from NEF as described in 6.2.3.4.

NOTE 1:	The SMF can be also preconfigured with the EDI.

-	Receiving of the Local Offloading Information from PCF via SMF, which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN.

NOTE 2:	When the I-SMF is inserted to support Local Offloading Management, the SM NAS message is still terminated to the SMF.

When the I-SMF is inserted for the Local Offloading Management, i.e. the inserted I-SMF providing Local Offloading allowed indication to SMF, the SMF does not discover, select and configure EASDF. as well as DNS message handling for the locally offloaded traffic towards the local part of DN.

The UE is unaware of whether the Local Offloading Management is applied or not for a PDU Session.

* * * * Start of 2nd Change * * * *

6.10.2.4	UE mobility for PDU session supporting Local Offloading Management

For a PDU session supporting Local Offloading Management, during mobility events, e.g. Hand-Over or AMF change, the mobility handling is executed same as the respective mobility handling defined in clause 4.23 of TS 23.502 [3] with the following difference:

-	Based on the retrieval of the Local Offloading Management Allowed indication as part of the SMF selection subscription data from UDM, the new AMF determines the I-SMF as follows:

-	If the service area of SMF includes the new UE location and the UE location is out of local offloading management service area, the AMF removes the old I-SMF (if available).

-	If the service area of SMF does not include the new UE location and the UE location is out of local offloading management service area, the AMF removes the old I-SMF unless the old I-SMF can serve the new UE location. If the old I-SMF can serve the new UE location and the UE is outside the local offloading management service area, the AMF may keep the old I-SMF and invokes Nsmf_PDUSession_UpdateSMContext Request without local offloading management allowed indication. In this case, the old I-SMF also invokes Nsmf_PDUSession_UpdateSMContext Request without local offloading management allowed indication to the SMF.

When I-SMF and SMF receives Nsmf_PDUSession_UpdateSMContext Request without local offloading management allowed indication, the I-SMF deletes the DNS Context from the old EASDF and the SMF selects new EASDF and creates DNS Context.

-	If the UE location is within local offloading management service area, the AMF selects the new I-SMF or changes the old I-SMF to the new I-SMF unless the existing I-SMF can serve the new UE location and supports Local Offloading Management.

-	To support the local offloading management, the selected I-SMF need to support Local Offloading Management, serves the new UE location; and support the DNN and S-NSSAI associated with the PDU session.

-	To support as defined above, the new AMF needs to be aware of whether the old I-SMF supports Local Offloading Management by interacting with the NRF for the SMF capability.

NOTE:	When the UE moves from an AMF which is not upgraded to act based on the Local Offloading Management support indication to an AMF which is upgraded to act based on the Local Offloading Management support indication, the above procedure also applies.

-	For I-SMF insertion and relocation case, the target I-SMF and SMF interacts as follows:

-	The target I-SMF sends new selected EASDF IP address and Local Offloading Management allowed indication to the SMF.

-	The SMF sends the new selected EASDF IP address to the UE via PCO.

-	For I-SMF removal case, the SMF performs as follows:

-	A new EASDF is selected by the SMF and the SMF sends the new selected EASDF IP address to the UE via PCO.

-	For I-SMF reuse case when the local offloading management does not apply, the SMF and I-SMF performs as follows:

-	An old EASDF is removed by the old I-SMF.

-	A new EASDF is selected by the SMF and the SMF sends the new selected EASDF IP address to the UE via PCO.

* * * * Start of 3rd Change * * * *

6.10.3.2	Local offloading management policy handling for Local Offloading Management

The Local Offloading Management Policy is generated by PCF and retrieved by SMF for Local Offloading Management as described in clause 6.10.2. The Local offloading Management Policy may also be provided with a Offload Identifier. The Offload Identifier is assigned by PCF, and is unique within the PLMN. The SMF and I-SMF for Local Offloading Management may also be preconfigured with Local Offloading policy and tagged with related Offload Identifier(s). In this case, the PCF only provides the Offload Identifier to the SMF.

The detailed information of Local Offloading Management Policy is described in clause 6.4 of TS 23.503 [4].The SMF forwards the received Local Offloading Management Policy or Offload Identifier to the I-SMF.

* * * * End of Changes * * * *