3GPP TSG-SA2 Meeting #166AH-e	S2-2500544

# Electronic meeting, January 20 – 24, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5973 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MMS ID to support Multi-modality awareness at RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the LS from SA(S2-2500074 / SP-241921), SA reached to consensus that the multimodality awareness can be achieved in the RAN by the CN providing multimodality information (i.e., the MMSID) by N2 SM information. It is proposed with updates to provide multi-modality information to the NG-RAN in order to support multimodality awareness at RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §5.7.1.2 - Multi-modal Awareness ID is added to the QoS Profile to be provided to the NG-RAN §5.7.2.X(new) - New clause under the 5G QoS Parameters to add description on the Multi-modal Awareness ID §5.37.1 - Add reference to the new clause for the support for Multi-modality awareness at RAN. §5.37.X(new) - New clause for the description to support for Multi-modality awareness at RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-modal awareness at RAN is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.1.2, 5.7.2.X (new), 5.37.1, 5.37.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.7.1.2	QoS Profile

A QoS Flow may either be 'GBR' or 'Non-GBR' depending on its QoS profile. The QoS profile of a QoS Flow is sent to the (R)AN and it contains QoS parameters as described below (details of QoS parameters are described in clause 5.7.2):

-	For each QoS Flow, the QoS profile shall include the QoS parameters:

-	5G QoS Identifier (5QI); and

-	Allocation and Retention Priority (ARP).

-	For each QoS Flow, the QoS profile may also include the QoS parameters:

-	PDU Set QoS Parameters (described in clause 5.7.7).

-	Multi-modal Awareness ID (described in clause 5.7.2.X).

-	For each Non-GBR QoS Flow only, the QoS profile may also include the QoS parameter:

-	Reflective QoS Attribute (RQA).

-	For each GBR QoS Flow only, the QoS profile shall also include the QoS parameters:

-	Guaranteed Flow Bit Rate (GFBR) - UL and DL; and

-	Maximum Flow Bit Rate (MFBR) - UL and DL; and

-	In the case of a GBR QoS Flow only, the QoS profile may also include one or more of the QoS parameters:

-	Notification control;

-	Maximum Packet Loss Rate - UL and DL.

NOTE:	In this Release of the specification, the Maximum Packet Loss Rate (UL, DL) is only provided for a GBR QoS Flow belonging to voice media.

Each QoS profile has one corresponding QoS Flow identifier (QFI) which is not included in the QoS profile itself.

The usage of a dynamically assigned 5QI for a QoS Flow requires in addition the signalling of the complete 5G QoS characteristics (described in clause 5.7.3) as part of the QoS profile.

When a standardized or pre-configured 5QI is used for a QoS Flow, some of the 5G QoS characteristics may be signalled as part of the QoS profile (as described in clause 5.7.3).

* * * * Start of 2nd Change * * * *

5.7.2.X	Multi-modal Awareness ID

The Multi-modal Awareness ID is an identifier generated by the PCF based on the combination of AF identifier and the Multi-modal Service ID from the AF that refers to the multi-modal communication service.

Multi-modal Awareness ID(s) associated with each QoS Flow are provided to the NG-RAN in the QoS profile.

* * * * Start of 3rd Change * * * *

5.37.1	General

This clause provides an overview of 5GS functionalities for support of XR services (AR/VR applications) and interactive media services that require high data rate and low latency communication, e.g. cloud gaming and tactile/multi-modal communication services according to service requirements documented in TS 22.261 [2]. The standardized 5QI characteristics for such interactive services are provided in Table 5.7.4-1 and TSCAI is used to describe the related traffic characteristics as defined in clause 5.27.2. Further enhancements for these interactive media services are as follows:

-	The 5GS may support QoS policy control for multi-modal traffic, see clause 5.37.2.

-	The 5GS may support network information exposure which can be based on ECN markings for L4S, see clause 5.37.3 or 5GS exposure API, see clause 5.37.4.

-	The 5GS may support PDU Set based QoS handling including PDU Set identification and marking, see clause 5.37.5.

-	The 5GS may ensure that the UL and DL packets together meet the requested round trip delay and also update the delay for UL and DL considering QoS monitoring results, see clause 5.37.6.

-	The 5GS may perform per-flow Packet Delay Variation (PDV) monitoring and policy control according to AF provided requirements, see clause 5.37.7.

-	The 5GC may provide traffic assistance information to the NG-RAN to enable Connected mode DRX power saving, see clause 5.37.8.

-	The 5GS may consider dynamically changed traffic characteristics for better resource management, see clause 5.37.10.

-	The 5GS may support traffic identification for multiplexed media flows in the same transport layer connection, see clause 5.37.11.

-	The 5GC may perform PDU Set Importance based transport level packet marking, see clause 5.8.2.7.

-	The 5GC may provide Multi-modality awareness information to the NG-RAN to support multi-modality awareness at RAN, see clause 5.37.X.

* * * * Start of 4th Change * * * *

5.37.X	Support for Multi-modality awareness at RAN

The following Multi-modality information can be provided by the CN to the NG-RAN in order to support multi-modality awareness at RAN:

-	Multi-modal Awareness ID.

The PCF includes the Multi-modal Awareness ID in the PCC rule to the SMF. Based on the PCC rule, for a QoS Flow used for the multi-modal service, the SMF includes Multi-modal Awareness ID to the QoS Profile to the NG-RAN.

NOTE 1:	The Multi-modal Awareness ID in the PCC rule is ensured to be unique within the operator's network by the PCF as described in clause 5.7.2.X.

NOTE 2:	Multiple Multi-modal Awareness IDs can be associated with a QoS Flow because the SMF can bind multiple SDFs into a single QoS Flow.

* * * * End of Changes * * * *