3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500553

# 20-24 January, 2025, Online		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5975 | rev |  | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | User plane information related to N3GPP device within PDU session modification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current specification refers to a generic IP address and port, but in case of IPV4 NAT the IP addresses are 2, one is the IP address in the LAN and a second one which is the IP address of the PDU session. The IP address in the loca LAN is generally a private IP address which has no meaning for the N3GPP system ,therefore even not strinclty speaking needed to send the IP address of the PDU session in order to us eteh same approach for all cased is proposed to send it. In case of IPv6, it is also not clear whether the IPaddress refers to the IPv6 assigned to the N3GPP device behind the UE, for example the IP address from the Prefix or the IPv6 Prefix assigned to the PDU session. In case of Ethernet , the specification refers to the MAC address, but it is also not clear of which MAC address and of which VLAN tag is referring to Furthermore the specification states that the MAC address or the IP address may be send in addition to the N3GPP device identifier, so it may be not sent. However if not sent the SMF and PCF are not able to identify which are the QoS rules to be sent therefore this information is mandatory in order to have a proper working feature. In case of the VLAN tag ID it shall be oted that from R16 the VLAN Tag ID received from the UE may be changed when the packet is send into the PDU session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified which IP address is sent to the SMF and that in case of VLAN Tag the VLAN Tag ID may be changed by the UE or added when not present in the LAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Erroneous specification since the QoS rules can not be correctly generated to identify the traffic of a specific N3GPP device |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.52.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.52.3	Session management enhancement

For the traffic of non-3GPP devices requiring differentiated QoS:

-	For Ethernet PDU Session Type, the UE sends the Non-3GPP Device Identifier and shall also send MAC address of the Non-3GPP device and/or may send the VLAN tag ID used in PDU session to carry the traffic that is associated with the Non-3GPP Device Identifier to the SMF in PDU Session Modification procedure.

NOTE 1: The VLAN ID used to carry the traffic between the Non-3GPP device and the UE/5G-RG may be changed when the traffic is carried over  the PDU session  Furthermore the traffic associated to Non-3GPP Device Identifier  between the Non-3GPP device and the UE/5G-RG may be not VLAN tagged, while it is carried on a VLAN in the PDU session.

-	For IPv4 PDU Session Type, the UE sends the Non-3GPP Device Identifier and may also send the PDU session IP Address and/port ranges used by the in the PDU session  associated with the Non-3GPP Device Identifier to the SMF in PDU Session Modification procedure.

NOTE 2: In case of IPv4 NAT the port(s) used for the traffic between the Non-3GPP device and the UE/5G-RG are mapped to different port(s) when the traffuic is carried in PDU session.

-	For IPv6 PDU Session Type, the UE sends the Non-3GPP Device Identifier and shall also send the IPv6 Address/prefix(sub) assigned to the N3GPP device  that is associated with the Non-3GPP Device Identifier to the SMF in PDU Session Modification procedure.

Editor's note:	Whether the UE sends the Non-3GPP Device Identifier and user plane information (e.g. IP Address) in PDU session establishment is FFS.

# * * * * End of changes * * * *