3GPP TSG-WG SA2 Meeting #166 Ad Hoc	S2-2500554

# Online, 20th Nov – 24nd Jan, 2025		(revision of S2-241xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1480 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PCC enhancements for support of identified non-3GPP devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following scenarios are not currently specified: - The UE changes the Non-3GPP device associated to a local information., e.g. the local IP address is assigned to a different Non-3GPP device - The Non-3GPP devices change the IP address (e.g. due an IP address renewable) or change the ports (e.g. due to the change in NAT when new application starts) - The Non-3GPP device disconnects from the UE (e.g the Non-3GPP device left the LAN or is turned off) - The UE disconnects a Non-3GPP device (e.g. the user decide to disconnect a specific device) or all Non-3GPP devices (e.g. the user turn off the Wifi interface) - The Non-3GPP device identifier is removed from the UE subscription information - The UE sends a Non-3GPP identifier which is not included in UE subscripton (e.g. the user has not added it in the subscription via AF interaction or a misuses in UE) All the above needs to be addressed properly as follow: - In case 1 and 2 the UE shall start a PDU session modification in order to update the PCC rules associated to the binding between the Non-3GPP Device Identifier and the User plane information - In case 3 and case 4 the UE may start a PDU session modification to remove the associated PCC rule. This is optional since the UE may wait some time in case that the disconnection is temporary. For example in case 4 based on implementation the UE may wait for a period of time before starting the PDU session modification to take into account possible temporary loss of connection or the UE (e.g. the application) allows to consider that some Non-3GPP device can always connect? with the same configuration (e.g. due to some static local IP address assignment to a specific device for enabling remote connection or for specific application such as survelliance camera, TV, etc). In case 4 probably the UE will perform immediately the PDU session modification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following specifications are added: - PDU session modification in order to update the PCC rules associated to the binding between the Non-3GPP Device Identifier and the User plane information when the user plane information changes or the Non-3GPP device is disconnected. - a PDU session modification shall be triggered by PCF or SMF for removing the related QoS rules when the Non-3GPP Device Identifier is removed from UE subscription. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong implementation of feature where QoS is not correctly managed for the correct Non-3GPP device due to misuses by UE and impredictable results. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.31 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

#### 6.1.3.31	Policy control for non-3GPP devices connecting behind a UE

The support of QoS differentiation for non-3GPP devices connecting behind a UE using a Non-3GPP Device Identifier is specified in clause 5.52 of TS 23.501 [2].

The SMF may receive from the UE the Non-3GPP Device Identifier and, optionally, the corresponding user plane address as specified in clause 4.3.3.2 of TS 23.502 [3]. When a Policy Control Request Trigger is met, the SMF sends the Non-3GPP Device Identifier and the user plane address of the non-3GPP device to the PCF. At reception of the Non-3GPP Device Identifier and user plane address, the PCF creates PCC rule(s) considering the Non-3GPP Device Identifier. The PCF determines QoS parameters in the PCC rule(s) based on the Non-3GPP Device Identifier Information which is retrieved from the UDR and/or based on operator policy. The PCF installs the PCC Rules at the SMF to enable QoS handling of the traffic to/from the non-3GPP device behind the UE.

If the UE decides to change the association between the Non-3GPP Device Identifier and the information it holds about this Non-3GPP Device (e.g. IPv4 address, IPv4 address and port range, IPv6 Address/prefix, MAC Address, MAC address and VLAN Tag ID), the UE shall start a PDU session modification and send the Non-3GPP Device Identifier with the new associated information.

NOTE X: For example if the UE decides to associate the Non-3GPP Device Identifier to a different Non-3GPP device or the IP address assigned to the Non-3GPP device is changed for any reason the UE needs to notify the SMF in order to update the information enabling the traffic differentiation.

It is assumed that there is only 1 PCC rule per each Non-3GPP Device Identifier, but the SMF may associate different PCC rules for the same QoS.

If the Non-3GPP Device Identifier is not included within the Non-3GPP Device Identifier Information as specified in clause 5.2.12.2.1 of TS 23.502 [3] for the UE, then the PCF indicates to the SMF that the corresponding Non-3GPP Device Identifier is not available for the UE. In case of PDU Session Modification, the PCF shall reject PDU Session Modification. When PDU Session Modification is rejected, a cause code is used to notify the UE that the Non-3GPP Device Identifier is not available for the UE.

If the UE detects, based on implementation specific mechanism, that the associated Non-3GPP device is no more connected behind the UE, the UE may start a PDU session modification and sends the Non-3GPP Device Identifier with an indication that it has been released. The SMF provides this information to PCF which removes the PCC Rules associated to that Non-3GPP Device Identifier.

NOTE y: Based on implementation the UE may wait for a period of time before starting the PDU session modification to take into account possible temporary loss of connection or the UE (e.g. the application) may not release a Non-3GPP devices with a static configuration (e.g. for enabling remote connection or for specific application such as surveliance camera, TV, etc).

If the Non-3GPP Device Identifier is removed from the UE subscription, the UDR notifies to the PCF via Nudr_DM_Notify service of removal of N3GPP device (as defined in TS 23.502 clauses 4.15.6.7.2 and 4.15.6.15). If there is PCC rules associated to the traffic associated to the removed Non-3GPP Device Identifier in a PDU session ( i.e. the UE has previously send a PDU session modification with the  Non-3GPP Device Identifier), the PCF shall remove the PCC rules,  start a PDU session modification and send an indication to the UE that the traffic for the Non-3GPP Device Identifier is no more allowed.

* * * End of Changes * * * *